| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0064 |
| --- | --- |

| Title: | TE 3: Cross-check results of test 4.6.2.2 on combined mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Hahyun Lee Jung Won Kang Jinho Lee 218 Gajeongno, Yuseong-gu Daejeon, 305-700, Korea | Tel: Email: | +82-42-860-6138 hanilee@etri.re.kr jungwon@etri.re.kr jinosoul@etri.re.kr |
| Source: | ETRI |  |  |

_____________________________

# Abstract

This document reports the cross-check results of 4.6.2.2 on combined mode proposed by Qualcomm. All the BD-rate results matched those provided by the proponent.

# Introduction

Combined mode is only applied to inter mode CUs and skip mode CUs. This mode is only applied to the luma component. The combind_mode_flag is signaled at the CU level.

# Cross-check results

No YUV files were written when the decoding times were measured. Encoding was done on a heterogeneous Linux cluster. Decoding was done sequentially on a single Linux PC.

All the BD-rate results matched exactly the results provided by proponent. Table 1 show the cross-check results. More detailed results can be found in the Excel attachment

Table 1 cross-check results of test 4.6.2.2

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.3% | -2.1% | -2.2% |  |  |  | -0.4% | -2.1% | -2.5% |
| Class B | -0.4% | -1.8% | -2.1% | -0.6% | -2.5% | -3.0% | -1.0% | -4.0% | -4.7% |
| Overall (EL+BL) | -0.4% | -1.9% | -2.1% | -0.6% | -2.5% | -3.0% | -0.8% | -3.5% | -4.1% |
| Overall (EL) | -0.6% | -3.2% | -3.5% | -1.2% | -5.2% | -6.3% | -1.3% | -5.6% | -6.4% |
| Enc Time[%] | 116.9% |  |  | 114.7% |  |  | 112.1% |  |  |
| Dec Time[%] | 99.8% |  |  | 99.6% |  |  | 99.5% |  |  |
| Enc Mem[%] | #NUM! |  |  | #NUM! |  |  | #NUM! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.0% | -3.5% | -3.6% |  |  |  | -1.4% | -4.2% | -4.6% |
| Class B | -1.3% | -2.7% | -3.1% | -1.5% | -4.6% | -5.3% | -3.0% | -7.3% | -8.7% |
| Overall (EL+BL) | -1.2% | -2.9% | -3.3% | -1.5% | -4.6% | -5.3% | -2.5% | -6.4% | -7.5% |
| Overall (EL) | -1.8% | -4.8% | -5.3% | -2.6% | -8.8% | -9.9% | -3.6% | -9.4% | -10.9% |
| Enc Time[%] | 119.6% |  |  | 114.6% |  |  | 115.2% |  |  |
| Dec Time[%] | 99.0% |  |  | 98.9% |  |  | 98.0% |  |  |
| Enc Mem[%] | #NUM! |  |  | #NUM! |  |  | #NUM! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

# Conclusion

The results of TE 3 test 4.6.2.2 on combined mode presented by Qualcomm (JCTVC-L0206) are verified by ETRI