| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0065 |
| --- | --- |

| Title: | TE5.2.3: Results of test 5.2.3 on inter-layer motion vector prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jinho Lee, Hahyun Lee, Jung Won Kang, Jin Soo Choi. Realistic Media Research Team, ETRI 218 Gajeongno, Yuseong-gu Daejeon, South Korea | Tel: Email: | +82-42-860-5656 jinosoul@etri.re.kr hanilee@etri.re.kr jungwon@etri.re.kr jschoi@etri.re.kr |
| Source: | ETRI (Electronics and Telecommunications Research Institute) |  |  |

_____________________________

# Abstract

This contribution describes inter-layer motion vector prediction in tool experiment 5.2.3 of JCTVC-K1105, and reports the simulation results. The motion information of co-located PU in the base layer is employed as the first candidate for merge mode and motion vector prediction. It is reported that the average BD-rate is -2.4% for RA and -1.3% for LD-P 2x spatial scalability, and -2.5% for RA and -1.3% for LD-P 1.5x spatial scalability, respectively.

# Introduction

At the last meeting, TE5 [1] was set up for testing inter-layer syntax prediction tools for SHVC. TE5.2 includes inter-layer motion vector prediction, and the related proposal [2] is described in TE5.2.3.

# Inter-layer motion vector prediction

Since motion information of the current PU in the enhancement layer is likely to be similar to the co-located PU in the base layer (colBase), the motion information of the co-located PU in the base layer is assigned as the first candidate for both merge mode and motion vector prediction. By the way, the maximum number of candidate is not changed.

The center position (T2 in Fig. 1) is used to get the colBase, and motion vector of the colBase is scaled by the spatial resolution factor. Moreover, motion compression process for a base layer frame is performed after en/decoding an enhancement layer frame. Therefore the uncompressed motion vector of the colBase is used as the candidate.

Figure 1. Candidate position

Table 1 represents the proposed derivation process for merging candidates. The motion information of the colBase is added as the first merge candidate, and a duplication check between colBase and A1 is added when deriving A1. In addition, A0 is derived when the number of candidates is smaller than four.

Table 1. Merge candidate derivation process

Table 2 represents the proposed derivation process for motion vector predictor candidates. The motion information of the colBase is added as the first motion vector prediction candidate, and the motion vector of the colBase is scaled when the POC of reference frame of the colBase is different from the POC of reference frame of the current PU in enhancement layer. In addition, duplication check and pruning process are performed when the number of candidate is larger than two after deriving spatial candidates.

Table 2. AMVP candidate derivation process

# Simulation results

The proposed method was performed based on Smuc0.1.1 software under common test condition. The experiments are conducted on the cluster platform of 64-bit Linux version. The proposal is cross-checked by InterDigital and Sony.

Table 3 represents the simulation results of the proposed method including inter-layer motion prediction for both merge mode and AMVP.

The additional simulation results of inter-layer motion prediction for merge mode and AMVP are represented in Table 4 and 5, respectively.

Table 3. Inter-layer motion prediction for both merge mode and AMVP

Table 4. Inter-layer motion prediction for merge mode

Table 5. Inter-layer motion prediction for AMVP

# Conclusions

Inter-layer motion vector prediction in TE5.2.3 gives BD-rate of -2.4% for RA and -1.3% for LD-P 2x spatial scalability, and -2.5% for RA and -1.3% for LD-P 1.5x spatial scalability, respectively. In addition, the simulation runtime is reduced.

# Patent rights declaration(s)

ETRI (Electronics and Telecommunications Research Institute) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- V. Seregin, P. Onno, S. Liu, T. Lee, C. Kim, H. Yang “Description of Tool Experiment C5: Inter-layer syntax prediction using HEVC base layer”, JCTVC-K1105, Oct. 2012, Shanghai, China.

- J. W. Kang, H. Lee, J. Lee, J. S. Choi, J. W. Kim, J. Nam, H. Choi, D. Sim “Description of scalable video coding technology proposal by ETRI and Kwangwoon Univ.”, JCTVC-K0037, Oct. 2012, Shanghai, China.