| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0067_r1 |
| --- | --- |

| Title: | On transform selection for IntraBL mode in SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chuohao Yeo Yih Han Tan 1 Fusionopolis Way #21-01 Connexis (South Tower) Singapore 138632 | Tel: Email: | +65 6408 2000 chyeo@i2r.a-star.edu.sg |
| Source: | Institute for Infocomm Research |  |  |

_____________________________

# Abstract

This contribution proposes using the 4-point integer DST that is currently used for 4x4 intra luma residual in HEVC for 4x4 IntraBL coded luma TUs. Experimental results suggest that compared to using DCT, the proposed method shows some small coding gains for AI and no significant losses for RA and LD-P. It is argued that this would be a more consistent treatment of IntraBL as an Intra mode and lead to a simpler design. The implementation and results presented here are also reportedly the same as those presented in an independently proposed contribution JCTVC-L0204.

# Introduction

In SMUC 0.1.1, there was a bug related to the transform used for IntraBL coded 4x4 luma TUs that uses the DCT as the forward transform but the DST as the inverse transform. This has since been fixed in a bug-fix branch such that the DCT is used for both the forward and inverse transform.

In this contribution, we also consider the use of DST for coding IntraBL 4x4 luma TUs, and show results for this combination.

We have independently implemented the fix of using DCT for IntraBL 4x4 luma TUs, as well as using DST for coding IntraBL 4x4 luma TUs, in SMUC 0.1.1. The patch for this can be found in the attachment. The choice of transform is controlled by a macro INTRA_BL_TX; setting it to 1 uses the DCT fix while setting it to 2 uses the DST fix.

JCTVC-L0204 also proposed the use of DST for coding IntraBL 4x4 luma TUs. The implementation used in that proposal, which was independently performed, also matches the implementation used in this proposal.

# Experimental results

We implemented both the DCT fix and the DST fix in SMUC 0.1.1 and used the common conditions generated from AHG10 in our experiments.

The results for applying the DCT fix compared to the SMUC 0.1.1 anchor was previously made available on the reflector, and is reproduced below:

If the DST fix is applied instead, then compared to the SMUC 0.1.1 anchor, the results are:

Finally, we compare the DST fix against the DCT fix:

As seen above, there are some small coding gains for applying DST to IntraBL 4x4 luma TUs, particularly in the AI cases. In the RA and LD-P cases, there are no significant coding losses. We argue that to treat IntraBL consistently as an intra mode, we should apply the DST in order to make the design consistent and simpler.

We also found that the above results match those presented in JCTVC-L0204. We can therefore consider this proposal and JCTVC-L0204 to be mutual cross-checks of each other.

# Conclusion

In this contribution, we propose the use of DST for transform coding of IntraBL 4x4 luma TUs. The experimental results show there are some small coding gains in the AI cases, and no significant coding losses in the RA and LD-P cases. For a more consistent treatment of IntraBL as an intra mode and a simpler design, we recommend adopting this proposal in the SHVC design. The same proposal was also made in JCTVC-L0204, and both the implementation and results in that proposal match what was presented here.

# References

- V. Seregin, T.-D. Chuang, D.-K. Kwon, F. Le Leannec and P. Onno, “AHG11: DST transform for Intra-BL mode,” JCTVC-L0204, Geneva, CH, Jan. 2013.

# Patent rights declaration(s)

Institute for Infocomm Research may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).