| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0069 |
| --- | --- |

| Title: | TE2-3.1.7: Inter-layer intra prediction signaling with residual skip flag |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Yu-Wen Huang, Shawmin Lei MediaTek No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 Chulkeun Kim, Joonyoung Park, Byeongmoon Jeon LG Electronics #221 Yangjae-dong, Seocho-gu, Seoul 137-130, Korea | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {peter.chuang, yuwen.huang, shawmin.lei}@ mediatek.com Byeongmoon Jeon +82-10-8234-1402 {chulkeun.kim, jy.park, bm.jeon}@lge.com |
| Source: | MediaTek Inc. and LG Electronics |  |  |

_____________________________

# Abstract

In this contribution, a syntax coding method and a deblocking modification for inter-layer texture prediction are proposed for HEVC scalable extension. An intra_BL_flag is signaled at coding unit (CU) level after the skip_flag to indicate whether upsampled base-layer reconstructed samples are used as enhancement-layer sample predictors. For each inter-layer texture prediction coded CU, a residual skip flag, no_residual_syntax_flag, is used to indicate whether the CU has at least a nonzero coefficient. For deblocking, the boundary strengths of inter-layer texture prediction coded blocks are set to 1. Simulation results reportedly show 23.2%, 32.9%, 16.6%, 26.4%, 12.4, and 22.1% BD-rate savings on average for AI-2x, AI-1.5x, RA-2x, RA-1.5x, LDP-2x, and LDP-1.5x, respectively, compared with HM-8.1 simulcast anchors. The decoding time is reduced by 6-17% compared with SMuC-0.1.1 anchors.

# Introduction

In scalable video coding, upsampled base-layer reconstructed samples can be used as sample predictors for enhancement-layer coding. In JCTVC-K0033 [1], a syntax coding method and a deblocking modification for inter-layer texture prediction are proposed for HEVC scalable extension, and these techniques were decided to be investigated on top of SMuC-0.1.1 as a part of Tool Experiment 2 (TE2). The rest of this contribution is organized as follows. The proposed algorithm will be described in Section 2, simulation results will be shown in Section 3, and finally conclusions will be given in Section 4.

# Proposed methods

## Inter-layer texture prediction signaling

In this proposal, an intra_BL_flag is signaled at coding unit (CU) level after the skip_flag. If the intra_BL_flag is true, upsampled base-layer reconstructed samples are used as enhancement-layer sample predictors. The prediction mode is set to MODE_INTRA_BL, the partition mode is set to PART_2NxN, and the intra luma prediction mode is set to MODE_DC. Since the intra luma prediction mode is MODE_DC, discrete sine transform (DST) is used for the inter-layer texture prediction coded 4x4 transform units (TUs). For the inter-layer texture prediction coded CU, a residual skip flag, no_residual_syntax_flag, is used to indicate whether the CU has at least a nonzero coefficient. The contexts of the no_residual_syntax_flag of the inter-layer texture prediction block are shared with the no_residual_syntax_flag of the inter-coded block. No extra context or line buffer is required for the no_residual_syntax_flag. The syntax table of the proposed inter-layer texture prediction signaling is shown in Table 1.

Table 1.  Coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enable_flag ) { |  |
| cu_transquant_bypass_flag | ae(v) |
| } |  |
| if( slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] | ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, log2CbSize ) |  |
| else { |  |
| nCbS = ( 1 << log2CbSize ) |  |
| intra_BL_flag[ x0 ][ y0 ] | ae(v) |
| if (!intra_BL_flag[ x0 ][ y0 ]) { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| … |  |
| } |  |
| } |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if( PredMode[ x0 ][ y0 ] != MODE_INTRA && !(PartMode = = PART_2Nx2N && merge_flag[x0][y0]) ¦¦ intra_BL_flag[ x0 ][ y0 ]) |  |
| no_residual_syntax_flag | ae(v) |
| if( !no_residual_syntax_flag ) { |  |
| MaxTrafoDepth = ( PredMode[ x0 ][ y0 ] = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter ) |  |
| transform_tree( x0, y0 x0, y0, log2CbSize, 0, 0 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |

## Deblocking boundary strength modification for inter-layer texture prediction

In SMuC-0.1.1 deblocking, inter-layer texture prediction coded blocks are treated as the intra-coded blocks. However, sample predictors of inter-layer texture prediction are more accurate than those of intra prediction, the boundary strengths of inter-layer texture prediction coded blocks are modified. The deblocking filter at enhancement layer is basically the same as in HEVC, except the boundary strength (BS) decision method. Figure 1 represents the proposed BS decision process. If one or two of the adjacent blocks (P or Q) for a boundary is coded as inter-layer texture prediction, the boundary strength is set to 1.

Figure 1.  Boundary strength decision in the deblocking filter at enhancement layer

# Simulation results

Experiments were conducted according to the TE-A2 description defined in JCTVC-K1102 [2], and the software is based on SMuC-0.1.1. Simulation results of the proposed inter-layer texture prediction signaling are shown in Table 2. Simulations results of the deblocking boundary strength modification for inter-layer texture prediction are shown in Table 3. Simulation results of the combined solution (the proposed inter-layer texture prediction signaling and the deblocking boundary strength modification for inter-layer texture prediction) are shown in Table 4. The anchors in Table 2, 3, and 4 are the HM-8.1 simulcast. For the combined solution, simulation results reportedly show 23.2%, 32.9%, 16.6%, 26.4%, 12.4, and 22.1% BD-rate savings on average for AI-2x, AI-1.5x, RA-2x, RA-1.5x, LDP-2x, and LDP-1.5x, respectively.

Table 5 shows the results of combined solution compared with SMuC-0.1.1 simulcast anchors. Simulation results reportedly show 0.6%, 0.7%, 0.3%, 0.5%, 0.2, and 0.4% BD-rate savings on average for AI-2x, AI-1.5x, RA-2x, RA-1.5x, LDP-2x, and LDP-1.5x, respectively. The decoding time is reduced by 6-17% compared with SMuC-0.1.1 anchors.

# Conclusions

In this contribution, the syntax coding method and the deblocking modification for inter-layer texture prediction in JCTVC-K0033 are investigated on SMuC-0.1.1. Simulation results reportedly show 23.2%, 32.9%, 16.6%, 26.4%, 12.4, and 22.1% BD-rate savings on average for AI-2x, AI-1.5x, RA-2x, RA-1.5x, LDP-2x, and LDP-1.5x, respectively, compared with HM-8.1 simulcast anchors. The decoding time is reduced by 6-17% compared with SMuC-0.1.1 anchors.

# References

- C. Kim, J. Park, J. Kim, Hendry, S. Park, Y. Jeon, J. Lim, N. Park, B. Jeon, S. Liu, X. Zhang, M. Guo, Z. Chen, T.-D. Chuang, S.-T. Hsiang, C.-Y. Chen, C.-Y. Tsai, C.-M. Fu, C.W. Hsu, Y.-W. Huang, S. Lei, “Description of scalable video coding technology proposal by LG Electronics and MediaTek (differential coding mode on),” Document of Joint Collaborative Team on Video Coding, JCTVC-K0033, October, 2012.

- L. Guo, Y. He, D. Kown, J. Zan, H. Lakshman, J. Kang, “Description of Tool Experiment A2: Inter-layer Texture Prediction Signaling in SHVC,” Document of Joint Collaborative Team on Video Coding, JCTVC-K1102, October, 2012.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

LG Electronics may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Table 2.  Results of the proposed inter-layer texture prediction signaling (anchor: HM-8.1 simulcast)

Table 3.  Results of the deblocking boundary strength modification for inter-layer texture prediction (anchor: HM-8.1 simulcast)

Table 4.  Results of the proposed inter-layer texture prediction signaling and the deblocking boundary strength modification for inter-layer texture prediction (anchor: HM-8.1 simulcast)

Table 5.  Results of the proposed inter-layer texture prediction signaling and the deblocking boundary strength modification for inter-layer texture prediction (anchor: SMuC-0.1.1)