| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0070 |
| --- | --- |

| Title: | TE5-5.2.1: Inter-layer motion vector prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Yu-Wen Huang, Shawmin Lei MediaTek No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 Joonyoung Park, Byeongmoon Jeon LG Electronics #221 Yangjae-dong, Seocho-gu, Seoul 137-130, Korea | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {peter.chuang, yuwen.huang, shawmin.lei}@mediatek.com Byeongmoon Jeon +82-10-82031126 {jy.park, bm.jeon}@lge.com |
| Source: | MediaTek Inc. and LG Electronics |  |  |

_____________________________

# Abstract

In this contribution, the motion data of the corresponding block in base layer (BL) are used for enhancement layer (EL) motion vector prediction as follows. For skip mode and merge mode, the center and the bottom-right motion data of the corresponding block in BL are placed at the 1st and the 4th positions in the merging candidate list. For inter mode, the center motion vector of the corresponding block in BL is placed before the spatial candidates in the AMVP candidate list. In addition, the motion data compression is performed after the EL picture coding. Simulation results reportedly show 2.8%, 2.7%, 3.1%, 1.7%, 1.5%, and 2.5% BD-rate savings on average for RA-2x, RA-1.5x, RA-SNR, LDP-2x, LDP-1.5x, and LDP-SNR, respectively, compared with SMuC-0.1.1 anchors. The encoding time is reduced by 3-6%, and the decoding time is roughly unchanged.

# Introduction

In scalable video coding, the scaled base layer (BL) motion vectors can be used for enhancement layer (EL) coding. In JCTVC-K0033 [1], the scaled BL motion vectors of the corresponding block are put into the EL AMVP and merge candidate lists to improve the EL motion vector coding. These techniques were decided to be investigated on top of SMuC-0.1.1 as a part of Tool Experiment 5 (TE5). The rest of this contribution is organized as follows. The proposed algorithm will be described in Section 2, simulation results will be shown in Section 3, and finally conclusions will be given in Section 4.

# Proposed methods

## Using uncompressed base-layer motion data

In HEVC, motion data including motion vectors, reference indexes, and prediction modes are compressed after decoding a picture. The compressed motion data are utilized for deriving temporal AMVP/merge candidates. In SMuC-0.1.1, the BL motion data are compressed before the EL picture coding. However, the compressed motion data make the inter-layer motion prediction inefficient. In this contribution, uncompressed BL motion data are used for deriving EL AMVP and merging candidates. After decoding all the pictures of all layers in an access unit, motion data compression is performed.

## Inter-layer merging candidate list

In this contribution, two corresponding positions, IL-C and IL-H, are used for deriving merging candidates, as shown in Figure 1. The IL-C candidate is placed in the first place in the merging candidate list, and the IL-H is placed in the 4th position in the merging candidate list, which is after the B1 candidate. The number of merging candidates is unchanged, which is 5 in this contribution. If the resolutions of the BL and the EL are different, the motion vectors of IL-C and IL-H are scaled according to the spatial ratio. In Table 1, the left column shows the merging candidate list in HM-8.1, and the right column shows the proposed merging candidate list for EL. To reduce redundancy, the IL-H candidate or any spatial candidate will be removed if it is the same as the IL-C candidate.

Figure 1.  Derivation of corresponding positions for inter-layer motion candidate

Table 1.  Proposed merging candidate list

| HM-8.1 | Proposed |
| --- | --- |
| - A1 - B1 - B0 - A0 - if(N < 4), B2 - Temporal - first pruning - if(N < Max.), Combined bi- prediction candidates - if(N < Max.), Zero MV candidates | - IL-C - A1 - B1 - IL-H - if(N < 4), B0 - if(N < 4), A0 - if(N < 4), B2 - Temporal - first pruning - if(N < Max.), Combined bi-prediction candidates - if(N < Max.), Zero MV candidates |

## Inter-layer AMVP candidate list

In this contribution, one corresponding positions, IL-C, is used for deriving inter-layer AMVP candidates, as shown in Figure 1. The IL-C candidate is placed in the first place in the AMVP candidate list only when its reference list and reference picture are the same as those of the current PU in EL. The number of AMVP candidates is unchanged, which is 2 in this contribution. If the resolutions of the BL and the EL are different, the motion vector of IL-C is scaled according to the spatial ratio. In Table 2, the left column shows the AMVP candidate list in HM-8.1, and the right column shows the proposed AMVP candidate list for EL. To reduce redundancy, any spatial candidate will be removed if it is the same as the IL-C candidate.

Table 2.  Proposed AMVP candidate list

| HM-8.1 | Proposed |
| --- | --- |
| - A - B - Col - first pruning - if(N < Max.), Zero MV candidates | - IL-C - A - B - Temporal - first pruning - if(N < Max.), Zero MV candidates |

# Simulation results

Experiments were conducted according to the TE-C5 description defined in JCTVC-K1105 [2], and the software is based on SMuC-0.1.1. Simulation results of the proposed inter-layer motion vector prediction are summarized in Table 3. Test A includes the proposed inter-layer merging candidates and uses the uncompressed BL motion data. Test B includes the proposed inter-layer AMVP candidates and uses the uncompressed BL motion data. Test C includes the proposed inter-layer merging candidates and inter-layer AMVP candidates and uses the uncompressed BL motion data. Test D includes the proposed inter-layer merging candidates and inter-layer AMVP candidates but uses the compressed BL motion data. Simulation results of the Test A, Test B, Test C, and Test D are shown in Table 4, Table 5, Table 6, and Table 7, respectively. All the anchors are SMuC-0.1.1. For the combined solution (Test C), simulation results reportedly show 2.8%, 2.7%, 3.1%, 1.7%, 1.5%, and 2.5% BD-rate savings on average for RA-2x, RA-1.5x, RA-SNR, LDP-2x, LDP-1.5x, and LDP-SNR, respectively. The encoding time is reduced by 3-6%, and the decoding time is roughly unchanged.

Table 3.  Results of the proposed inter-layer motion vector prediction (BL+EL)

|  | Uncompr. BL MV | IL-Merge | IL-AMVP | RA-2x | RA-1.5x | RA-SNR | LDP-2x | LDP-1.5x | LDP-SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test A | O | O |  | -2.7% | -2.6% | -3.0% | -1.6% | -1.4% | -2.4% |
| Test B | O |  | O | -1.5% | -1.5% | -1.5% | -1.0% | -0.8% | -1.1% |
| Test C | O | O | O | -2.8% | -2.7% | -3.1% | -1.7% | -1.5% | -2.5% |
| Test D |  | O | O | -1.8% | -2.2% | -2.7% | -0.8% | -1.1% | -2.0% |

Table 8 shows the coding efficiency of the IL-H merging candidate in Section 2.2. In Table 8, the test includes the IL-C and IL-H merging candidates in Section 2.2, and the anchor includes the IL-C merging candidate. Both the test and anchor in Table 8 use compressed BL motion data. Simulation results reportedly show 0.6%, 0.3%, 0.6%, 0.4%, 0.3%, and 0.5% BD-rate savings on average for RA-2x, RA-1.5x, RA-SNR, LDP-2x, LDP-1.5x, and LDP-SNR, respectively. The encoding time and decoding time are roughly unchanged.

# Conclusions

In this contribution, the inter-layer motion vector prediction methods in JCTVC-K0033 are investigated on SMuC-0.1.1. Simulation results reportedly show 2.8%, 2.7%, 3.1%, 1.7%, 1.5%, and 2.5% BD-rate savings on average for RA-2x, RA-1.5x, RA-SNR, LDP-2x, LDP-1.5x, and LDP-SNR, respectively, compared with SMuC-0.1.1 anchors. The encoding time is reduced by 3-6%, and the decoding time is roughly unchanged compared with SMuC-0.1.1 anchors.

# References

- C. Kim, J. Park, J. Kim, Hendry, S. Park, Y. Jeon, J. Lim, N. Park, B. Jeon, S. Liu, X. Zhang, M. Guo, Z. Chen, T.-D. Chuang, S.-T. Hsiang, C.-Y. Chen, C.-Y. Tsai, C.-M. Fu, C.W. Hsu, Y.-W. Huang, S. Lei, “Description of scalable video coding technology proposal by LG Electronics and MediaTek (differential coding mode on),” Document of Joint Collaborative Team on Video Coding, JCTVC-K0033, October, 2012.

- V. Seregin, P. Onno, S. Liu, T. Lee, C. Kim, H. Yang, “Description of Tool Experiment C5: Inter-layer syntax prediction using HEVC base layer,” Document of Joint Collaborative Team on Video Coding, JCTVC-K1105, October, 2012.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

LG Electronics may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Table 4.  Results of Test A: using the proposed inter-layer merging candidates and uncompressed BL motion data

Table 5.  Results of Test B: using the proposed inter-layer AMVP candidates and uncompressed BL motion data

Table 6.  Results of Test C: using proposed inter-layer merging and AMVP candidates and uncompressed BL motion data

Table 7.  Results of Test D: using the proposed inter-layer merge and AMVP candidates and compressed base layer motion data

Table 8.  Results of the IL-H, where the test includes the IL-C and IL-H merging candidates and the anchor includes the IL-C merging candidate