| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0071 |
| --- | --- |

| Title: | AHG9: Inter-layer prediction flag and inter-layer syntax prediction flag |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {peter.chuang, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, an adaptive predictor compensation (APC) using reconstructed base layer (BL) texture is applied to enhancement layer (EL) sample predictors. For each inter-coded CU that has partition mode equal to 2Nx2N and at least one non-zero residue, the proposed APC can be applied with an explicit APC enabling flag at CU level. If the APC is applied, the EL sample predictors are generated by averaging the EL original sample predictors and the BL reconstructed texture of the corresponding block. Simulation results reportedly show 0.4%, 0.6%, 0.8%, 1.2%, 1.5%, and 2.4% BD-rate savings on average for RA-2x, RA-1.5x, RA-SNR, LDP-2x, LDP-1.5x, and LDP-SNR, respectively, compared with SMuC-0.1.1 anchors. The encoding time increase is 4%, and the decoding time increase is negligible.

# Prior art for constraining the inter-layer prediction

In SVC, a no_inter_layer_pred_flag is signaled in network abstraction layer (NAL) unit header, as shown in Table 1. The no_inter_layer_pred_flag specifies whether the base layer (BL) information can be used in this NAL unit. When no_inter_layer_pred_flag is equal to 1, inter-layer prediction is not used for EL coding. When no_inter_layer_pred_flag is equal to 0, inter-layer prediction may be used for EL coding as signaled in the macroblock layer. If no_inter_layer_pred_flag is equal to 0, three inter-layer prediction flags, which are adaptive_base_mode_flag, adaptive_motion_prediction_flag, and adaptive_residual_prediction_flag, are signaled in slice header, as shown in Table 2. If the adaptive_base_mode_flag is equal to 1, the base_mode_flag can be signaled in MB layer. If the adaptive_base_mode_flag is equal to 0, the base_mode_flag is inferred and equal to default_base_mode_flag. The same mechanism is applied for adaptive_motion_prediction_flag and adaptive_residual_prediction_flag. If adaptive_motion_prediction_flag is equal to 1, the motion_prediction_flag_l0 and motion_prediction_flag_l1 can be signaled; otherwise the motion_prediction_flag_l0 and motion_prediction_flag_l1 are inferred as the value of default_motion_prediction_flag. If adaptive_residual_prediction_flag is equal to 1, the residual_prediction_flag can be signaled; otherwise the residual_prediction_flag is inferred as the value of default_residual_prediction_flag. The motion_prediction_flag_l0 and motion_prediction_flag_l1 specify whether the BL MV is used as the MVP. The residual_prediction_flag specifies whether the residual prediction is applied on this MB.

- Table 1. NAL unit header SVC extension syntax

| nal_unit_header_svc_extension( ) { | C | Descriptor |
| --- | --- | --- |
| reserved_one_bit | All | u(1) |
| idr_flag | All | u(1) |
| priority_id | All | u(6) |
| no_inter_layer_pred_flag | All | u(1) |
| dependency_id | All | u(3) |
| quality_id | All | u(4) |
| temporal_id | All | u(3) |
| use_ref_base_pic_flag | All | u(1) |
| discardable_flag | All | u(1) |
| output_flag | All | u(1) |
| reserved_three_2bits | All | u(2) |
| } |  |  |

- Table 2. Slice header in scalable extension syntax

| slice_header_in_scalable_extension( ) { | C | Descriptor |
| --- | --- | --- |
| … |  |  |
| if( !no_inter_layer_pred_flag ) { |  |  |
| slice_skip_flag | 2 | u(1) |
| if( slice_skip_flag ) |  |  |
| num_mbs_in_slice_minus1 | 2 | ue(v) |
| else { |  |  |
| adaptive_base_mode_flag | 2 | u(1) |
| if( !adaptive_base_mode_flag ) |  |  |
| default_base_mode_flag | 2 | u(1) |
| if( !default_base_mode_flag ) { |  |  |
| adaptive_motion_prediction_flag | 2 | u(1) |
| if( !adaptive_motion_prediction_flag ) |  |  |
| default_motion_prediction_flag | 2 | u(1) |
| } |  |  |
| adaptive_residual_prediction_flag | 2 | u(1) |
| if( !adaptive_residual_prediction_flag ) |  |  |
| default_residual_prediction_flag | 2 | u(1) |
| } |  |  |
| if( adaptive_tcoeff_level_prediction_flag = = 1 ) |  |  |
| tcoeff_level_prediction_flag | 2 | u(1) |
| } |  |  |
| if( !slice_header_restriction_flag && !slice_skip_flag ) { |  |  |
| scan_idx_start | 2 | u(4) |
| scan_idx_end | 2 | u(4) |
| } |  |  |
| } |  |  |

# Proposed constraint for inter-layer prediction in HEVC scalable extension

## no_inter_layer_pred_flag

In HEVC scalable extension, a no_inter_layer_pred_flag is proposed to be signaled in NAL unit header,  video parameter set (VPS) or picture parameter set (PPS) to specifies whether the inter-layer prediction may be used for EL coding. Table 3 shows the syntax change that signaling the no_inter_layer_pred_flag in NAL unit header as the H.264/AVC scalable extension does. Note that the nuh_layer_id is reduced by 1 bit.

Table 3.  NAL unit header syntax

| nal_unit( NumBytesInNALunit ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_unit_type | u(6) |
| nuh_layer_id | u(5) |
| no_inter_layer_pred_flag | u(1) |
| nuh_temporal_id_plus1 | u(3) |
| NumBytesInRBSP = 0 |  |
| for( i = 2; i < NumBytesInNALunit; i++ ) { |  |
| if( i + 2 < NumBytesInNALunit && next_bits( 24 ) = = 0x000003 ) { |  |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| i += 2 |  |
| emulation_prevention_three_byte /* equal to 0x03 */ | f(8) |
| } else |  |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| } |  |
| } |  |

Since several video-level parameters are moved in the VPS, another position for signal the no_inter_layer_predictoin_flag is to put this flag in VPS extension. Table 4 shows the syntax change that signaling the no_inter_layer_pred_flag in VPS extension.

Table 4.  Video parameter set extension RBSP syntax

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| avc_base_codec_flag | u(1) |
| scalability_mask | u(16) |
| for( i = 0; i <NumScalabilityTypes; i++ ) { |  |
| dimension_id_len_minus1[ i ] | u(3) |
| } |  |
| vps_nuh_layer_id_present_flag | u(1) |
| if (nuh_layer_id > 0) |  |
| no_inter_layer_pred_flag | u(1) |
| … |  |
| } |  |

To reduce the error propagation, the no_inter_layer_pred_flag can be put into PPS and set equal to 1 when the current frame is the CRA picture. It can reduce the error propagation from BL data. Table 5 shows the syntax change that signaling the no_inter_layer_pred_flag in PPS.

Table 5.  Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if (nuh_layer_id > 0) |  |
| no_inter_layer_pred_flag | u(1) |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

## no_inter_layer_syntax_pred_flag

In HEVC scalable extension, the inter-layer prediction includes inter-layer texture prediction, which is investigate in TE-A1 [2], TE-A2 [3], TE-B3 [4], and TE-B4 [5], and inter-layer syntax prediction, which is investigate in TE-C5 [6], TE-C6 [7]. For syntax prediction, the decoder have to storage the BL information for EL coding, e.g. prediction mode, partition mode, intra prediction mode, motion vector. Some of them have different definition if the BL and EL bitstreams use different standard. For example, the definitions of intra prediction mode index are different between HEVC and H.264/AVC. some standards, e.g. MPEG-1 and MPEG-2, even do not have intra prediction mode.

To support the multi-standard decoding in HEVC scalable extension, a no_inter_layer_syntax_pred_flag is proposed to be signaled in PPS or in slice header. The no_inter_layer_syntax_pred_flag is signaled when no_inter_layer_pred_flag is equal to 0, or is inferred as 1 when no_inter_layer_pred_flag is equal to 1. When the no_inter_layer_pred_flag is equal to 1, all the syntax or mode information are not allowed in the EL coding. Only the BL reconstructed texture or residual can be used. When no_inter_layer_syntax_pred_flag is equal to 1, the data access bandwidth and storage for BL syntax or mode information can be reduced in decoder.

Table 6 shows the syntax change that signaling the no_inter_layer_pred_flag in PPS, and Table 7 shows the syntax change that signaling the no_inter_layer_pred_flag in slice header.

Table 6.  Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if (nuh_layer_id > 0 && !no_inter_layer_pred_flag) |  |
| no_inter_layer_syntax_pred_flag | u(1) |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

Table 7. General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if (nuh_layer_id > 0 && !no_inter_layer_pred_flag) |  |
| no_inter_layer_syntax_pred_flag | u(1) |
| if( slice_header_extension_present_flag ) { |  |
| slice_header_extension_length | ue(v) |
| for( i = 0; i < slice_header_extension_length; i++) |  |
| slice_header_extension_data_byte[ i ] | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

# References

- B. Bross, G. J. Sullivan, T. K. Tan, “Proposed editorial improvements for High Efficiency Video Coding (HEVC) text specification draft 8,” Document of Joint Collaborative Team on Video Coding, JCTVC-K0030, October, 2012.

- A. Segall, J. Chen, J. Dong, E. Alshina, “Description of Tool Experiment A1: Upsampling filter for SHVC,” Document of Joint Collaborative Team on Video Coding, JCTVC-K1101, October, 2012.

- L. Guo, Y. He, D. Kown, J. Zan, H. Lakshman, J. Kang, “Description of Tool Experiment A2: Inter-layer Texture Prediction Signaling in SHVC,” Document of Joint Collaborative Team on Video Coding, JCTVC-K1102, October, 2012.

- J. Chen, A. Segall, E. Alshina, S. Liu, J. Dong, J. Park, “Description of Tool Experiment B3: Combined Prediction in SHVC,” Document of Joint Collaborative Team on Video Coding, JCTVC-K1103, October, 2012.

- J. Chen, A. Segall, E. Alshina, S. Liu, J. Dong, J. Park, “Description of Tool Experiment B4: Inter-layer filtering in SHVC,” Document of Joint Collaborative Team on Video Coding, JCTVC-K1104, October, 2012

- V. Seregin, P. Onno, S. Liu, T. Lee, C. Kim, H. Yang, “Description of Tool Experiment C5: Inter-layer syntax prediction using HEVC base layer,” Document of Joint Collaborative Team on Video Coding, JCTVC-K1105, October, 2012

- J. Boyce, K. Kawamura, H. Lakshman, “Description of Tool Experiment C6: Inter-layer syntax prediction using AVC base layer,” Document of Joint Collaborative Team on Video Coding, JCTVC-K1106, October, 2012

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).