| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0071 |
| --- | --- |

| Title: | AHG9: Inter-layer prediction flag and inter-layer syntax prediction flag |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {peter.chuang, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, two high-level syntax changes are proposed. One proposed syntax change is to signal a no_inter_layer_pred_flag in video parameter set (VPS) extension to specify whether inter-layer prediction may be used for enhancement layer (EL) coding. The other proposed syntax change is to signal a no_inter_layer_syntax_pred_flag is in picture parameter set (PPS) extension or slice segment header extension in order to support a base layer (BL) format different from HEVC with no_inter_layer_syntax_pred_flag set to 1.

# Prior inter-layer prediction control in SVC

In SVC, a no_inter_layer_pred_flag is signaled in network abstraction layer (NAL) unit header, as shown in Table 1. The no_inter_layer_pred_flag specifies whether the base layer (BL) information can be used in this NAL unit. When no_inter_layer_pred_flag is equal to 1, inter-layer prediction is not used for enhancement layer (EL) coding. When no_inter_layer_pred_flag is equal to 0, inter-layer prediction may be used for EL coding as signaled in the macroblock layer. If no_inter_layer_pred_flag is equal to 0, three inter-layer prediction flags, which are adaptive_base_mode_flag, adaptive_motion_prediction_flag, and adaptive_residual_prediction_flag, are signaled in slice header, as shown in Table 2. If the adaptive_base_mode_flag is equal to 1, the base_mode_flag can be signaled in macroblock (MB) layer. If the adaptive_base_mode_flag is equal to 0, the base_mode_flag is inferred and equal to default_base_mode_flag. The same mechanism is applied for adaptive_motion_prediction_flag and adaptive_residual_prediction_flag. If adaptive_motion_prediction_flag is equal to 1, the motion_prediction_flag_l0 and motion_prediction_flag_l1 can be signaled; otherwise the motion_prediction_flag_l0 and motion_prediction_flag_l1 are inferred as the value of default_motion_prediction_flag. If adaptive_residual_prediction_flag is equal to 1, the residual_prediction_flag can be signaled; otherwise the residual_prediction_flag is inferred as the value of default_residual_prediction_flag. The motion_prediction_flag_l0 and motion_prediction_flag_l1 specify whether the BL motion vector (MV) is used as the motion vector predictor (MVP). The residual_prediction_flag specifies whether the residual prediction is applied to this MB.

Table 1.  Prior art: NAL unit header in SVC

| nal_unit_header_svc_extension( ) { | C | Descriptor |
| --- | --- | --- |
| reserved_one_bit | All | u(1) |
| idr_flag | All | u(1) |
| priority_id | All | u(6) |
| no_inter_layer_pred_flag | All | u(1) |
| dependency_id | All | u(3) |
| quality_id | All | u(4) |
| temporal_id | All | u(3) |
| use_ref_base_pic_flag | All | u(1) |
| discardable_flag | All | u(1) |
| output_flag | All | u(1) |
| reserved_three_2bits | All | u(2) |
| } |  |  |

Table 2.  Prior art: slice header in SVC

| slice_header_in_scalable_extension( ) { | C | Descriptor |
| --- | --- | --- |
| … |  |  |
| if( !no_inter_layer_pred_flag ) { |  |  |
| slice_skip_flag | 2 | u(1) |
| if( slice_skip_flag ) |  |  |
| num_mbs_in_slice_minus1 | 2 | ue(v) |
| else { |  |  |
| adaptive_base_mode_flag | 2 | u(1) |
| if( !adaptive_base_mode_flag ) |  |  |
| default_base_mode_flag | 2 | u(1) |
| if( !default_base_mode_flag ) { |  |  |
| adaptive_motion_prediction_flag | 2 | u(1) |
| if( !adaptive_motion_prediction_flag ) |  |  |
| default_motion_prediction_flag | 2 | u(1) |
| } |  |  |
| adaptive_residual_prediction_flag | 2 | u(1) |
| if( !adaptive_residual_prediction_flag ) |  |  |
| default_residual_prediction_flag | 2 | u(1) |
| } |  |  |
| if( adaptive_tcoeff_level_prediction_flag = = 1 ) |  |  |
| tcoeff_level_prediction_flag | 2 | u(1) |
| } |  |  |
| if( !slice_header_restriction_flag && !slice_skip_flag ) { |  |  |
| scan_idx_start | 2 | u(4) |
| scan_idx_end | 2 | u(4) |
| } |  |  |
| } |  |  |

# Proposed inter-layer prediction control in SHVC

## no_inter_layer_pred_flag

In SHVC, a no_inter_layer_pred_flag is proposed to be signaled in video parameter set (VPS) extension to specify whether inter-layer prediction may be used for EL coding

Table 3 shows the syntax change that signals no_inter_layer_pred_flag in VPS extension.

Table 3.  Video parameter set extension RBSP syntax

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| avc_base_codec_flag | u(1) |
| scalability_mask | u(16) |
| for( i = 0; i <NumScalabilityTypes; i++ ) { |  |
| dimension_id_len_minus1[ i ] | u(3) |
| } |  |
| vps_nuh_layer_id_present_flag | u(1) |
| if (nuh_layer_id > 0) |  |
| no_inter_layer_pred_flag | u(1) |
| … |  |
| } |  |

## no_inter_layer_syntax_pred_flag

In SHVC, inter-layer prediction includes inter-layer texture prediction, which is investigated in TE-A1 [1], TE-A2 [2], TE-B3 [3], and TE-B4 [4], and inter-layer syntax prediction, which is investigated in TE-C5 [5], TE-C6 [6]. For syntax prediction, the decoder has to storage BL information for EL coding (e.g., prediction mode, partition mode, intra prediction mode, motion vector).

To support multi-standard decoding in SHVC, where the BL can be encoded by a format different from HEVC, a no_inter_layer_syntax_pred_flag is proposed to be signaled in picture parameter set (PPS) extension or in slice segment header extension. The no_inter_layer_syntax_pred_flag is signaled when no_inter_layer_pred_flag is equal to 0, or is inferred as 1 when no_inter_layer_pred_flag is equal to 1. When the no_inter_layer_pred_flag is equal to 1, all the syntax or mode information from BL is not required in the EL coding. Only the BL reconstructed texture or residual can be used for EL coding. When no_inter_layer_syntax_pred_flag is equal to 1, the data access bandwidth and storage for BL syntax or mode information can be reduced at decoders.

Table 4 shows the syntax change that signals no_inter_layer_pred_flag in PPS extension, and Table 5 shows the syntax change that signals no_inter_layer_pred_flag in slice segment header extension.

Table 4.  Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ){ |  |
| if (nuh_layer_id > 0 && !no_inter_layer_pred_flag) |  |
| no_inter_layer_syntax_pred_flag | u(1) |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

Table 5.  Slice segment header syntax

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if (nuh_layer_id > 0 && !no_inter_layer_pred_flag) |  |
| no_inter_layer_syntax_pred_flag | u(1) |
| … |  |
| } |  |

# References

- A. Segall, J. Chen, J. Dong, E. Alshina, “Description of Tool Experiment A1: Upsampling filter for SHVC,” Document of Joint Collaborative Team on Video Coding, JCTVC-K1101, October, 2012.

- L. Guo, Y. He, D. Kown, J. Zan, H. Lakshman, J. Kang, “Description of Tool Experiment A2: Inter-layer Texture Prediction Signaling in SHVC,” Document of Joint Collaborative Team on Video Coding, JCTVC-K1102, October, 2012.

- J. Chen, A. Segall, E. Alshina, S. Liu, J. Dong, J. Park, “Description of Tool Experiment B3: Combined Prediction in SHVC,” Document of Joint Collaborative Team on Video Coding, JCTVC-K1103, October, 2012.

- J. Chen, A. Segall, E. Alshina, S. Liu, J. Dong, J. Park, “Description of Tool Experiment B4: Inter-layer filtering in SHVC,” Document of Joint Collaborative Team on Video Coding, JCTVC-K1104, October, 2012

- V. Seregin, P. Onno, S. Liu, T. Lee, C. Kim, H. Yang, “Description of Tool Experiment C5: Inter-layer syntax prediction using HEVC base layer,” Document of Joint Collaborative Team on Video Coding, JCTVC-K1105, October, 2012

- J. Boyce, K. Kawamura, H. Lakshman, “Description of Tool Experiment C6: Inter-layer syntax prediction using AVC base layer,” Document of Joint Collaborative Team on Video Coding, JCTVC-K1106, October, 2012

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).