| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0072 |
| --- | --- |

| Title: | TE3-4.3.1: Adaptive predictor compensation by using reconstructed BL texture |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Yu-Wen Huang, Polin Lai, Shan Liu, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {peter.chuang, yuwen.huang, polin.lai, shan.liu, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, adaptive predictor compensation (APC) using reconstructed base layer (BL) texture is applied to enhancement layer (EL) sample predictors. For each inter-coded CU that has partition mode equal to 2Nx2N and at least one non-zero residue, the proposed APC can be applied with an explicit APC enabling flag at CU level. If the APC is applied, the EL sample predictors are generated by averaging the EL original sample predictors and the BL reconstructed texture of the corresponding block. Simulation results reportedly show 0.4%, 0.6%, 0.8%, 1.2%, 1.5%, and 2.4% BD-rate savings on average for RA-2x, RA-1.5x, RA-SNR, LDP-2x, LDP-1.5x, and LDP-SNR, respectively, compared with SMuC-0.1.1 anchors. The encoding time increase is 4%, and the decoding time increase is negligible.

# Introduction

In scalable video coding, the current base layer (BL) reconstructed picture may predict better than the enhancement layer (EL) reference picture for regions that newly appear or have light changes. If the BL quantization distortion is not large, the EL original samples should be close to the BL reconstructed samples. Therefore, in the EL inter prediction, the BL reconstructed texture can be used to refine the EL motion compensated predictors, which was first proposed in JCTVC-K0033 [1] and decided to be investigated on top of SMuC-0.1.1 as a part of Tool Experiment 3 (TE3). In this contribution, the proposed technique is termed adaptive predictor compensation (APC). The rest of this contribution is organized as follows. The proposed algorithm will be described in Section 2, simulation results will be shown in Section 3, and finally conclusions will be given in Section 4.

# Proposed method

In JCTVC-K0033, prediction sample adaptive offset (PSAO) was proposed to use the BL reconstructed samples to refine the EL predictors. The adaptive offset is set equal to –a*(PredEL – RecBL) and added to the EL predictor as follows.

PredEL’ = PredEL – a * (PredEL – RecBL)							(1)

Equation (1) can be rewritten as

PredEL’ = (1 – a) * PredEL + a * RecBL = b * PredEL + a * RecBL, where a + b = 1		(2)

As indicated in Equation (2), PSAO can be simplified as a weighted sum of the original EL predictor and the BL reconstructed sample. In this contribution, both “a” and “b” are set to 0.5, and PASO is renamed APC with the following refined EL predictor.

PredEL’ = (PredEL + RecBL) >> 1								(3)

The proposed APC is only applied to each inter-coded CU with a partition mode equal to 2Nx2N and at least one non-zero residue. In addition, an APC_enable_flag is signaled after no_residual_prediction_flag to indicate whether the proposed APC is applied, as shown in Table 1.

Table 1.  Coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enable_flag ) { |  |
| cu_transquant_bypass_flag | ae(v) |
| } |  |
| if( slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] | ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, log2CbSize ) |  |
| else { |  |
| … |  |
| } |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if( PredMode[ x0 ][ y0 ] != MODE_INTRA && !(PartMode = = PART_2Nx2N && merge_flag[x0][y0])) |  |
| no_residual_syntax_flag | ae(v) |
| if( !no_residual_syntax_flag ) { |  |
| MaxTrafoDepth = ( PredMode[ x0 ][ y0 ] = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter ) |  |
| if ( PredMode[ x0 ][ y0 ] != MODE_INTRA && PredMode[ x0 ][ y0 ] != MODE_INTRA_BL) && PartMode = = PART_2Nx2N) |  |
| APC_enable_flag | ae(v) |
| transform_tree( x0, y0 x0, y0, log2CbSize, 0, 0 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |

# Simulation results

Experiments were conducted according to the TE-B3 description defined in JCTVC-K1103 [2], and the software is based on SMuC-0.1.1. Simulation results of the proposed APC are shown in Table 2. Simulation results reportedly show 0.4%, 0.6%, 0.8%, 1.2%, 1.5%, and 2.4% BD-rate savings on average for RA-2x, RA-1.5x, RA-SNR, LDP-2x, LDP-1.5x, and LDP-SNR, respectively, compared with SMuC-0.1.1 anchors. The encoding time is roughly increased by 4%, and the decoding time is roughly unchanged.

The results of two different APC encoding algorithms are also shown in Table 3 and Table 4 as the supplementary results, where all modifications from the APC with medium complexity rate-distortion optimization (RDO) in Table 2 are encoder-only and non-normative. The proposed APC with low complexity RDO is shown in Table 3, where the coding gain and encoding time are relatively reduced. The proposed APC with high complexity RDO is shown in Table 4, where the coding gain and encoding time are relatively increased. Table 5 shows a summary table of different RDO algorithms.

# Conclusions

In this contribution, adaptive predictor compensation (APC) originated from JCTVC-K0033 is investigated on SMuC-0.1.1. Simulation results reportedly show 0.4%, 0.6%, 0.8%, 1.2%, 1.5%, and 2.4% BD-rate savings on average for RA-2x, RA-1.5x, RA-SNR, LDP-2x, LDP-1.5x, and LDP-SNR, respectively, compared with SMuC-0.1.1 anchors. The encoding time is roughly increased by 4%, and the decoding time is roughly unchanged.

# References

- C. Kim, J. Park, J. Kim, Hendry, S. Park, Y. Jeon, J. Lim, N. Park, B. Jeon, S. Liu, X. Zhang, M. Guo, Z. Chen, T.-D. Chuang, S.-T. Hsiang, C.-Y. Chen, C.-Y. Tsai, C.-M. Fu, C.W. Hsu, Y.-W. Huang, S. Lei, “Description of scalable video coding technology proposal by LG Electronics and MediaTek (differential coding mode on),” Document of Joint Collaborative Team on Video Coding, JCTVC-K0033, October, 2012.

- J. Chen, A. Segall, E. Alshina, S. Liu, J. Dong, J. Park, “Description of Tool Experiment B3: Combined Prediction in SHVC,” Document of Joint Collaborative Team on Video Coding, JCTVC-K1103, October, 2012.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Table 2.  Results of the proposed APC with medium complexity RDO

Table 3.  Results of the proposed APC with low complexity RDO, where the decoding time is not measured on homogeneous clusters and should be similar to Table 2 if homogeneous clusters are used

Table 4.  Results of the proposed APC with high complexity RDO, where the decoding time is not measured on homogeneous clusters and should be similar to Table 2 if homogeneous clusters are used

Table 5.  Summary of APC using different RDO algorithms

|  | RA-2x | RA-1.5x | RA-SNR | LDP-2x | LDP-1.5x | LDP-SNR | Enc. Time | Dec. Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Low Complexity | -0.2% | -0.5% | -0.7% | -1.0% | -1.3% | -2.2% | 100% | 100% |
| Medium Complexity | -0.4% | -0.6% | -0.8% | -1.2% | -1.5% | -2.4% | 104% | 100% |
| High Complexity | -0.6% | -0.8% | -1.1% | -1.4% | -1.7% | -2.6% | 122% | 100% |