| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0073 |
| --- | --- |

| Title: | Non-TE5: On the effectiveness of temporal and base layer co-located motion vector prediction candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yih Han Tan Chuohao Yeo 1 Fusionopolis Way #21-01 Connexis (South Tower) Singapore 138632 | Tel: Email: | +65 6408 2000 yhtan@i2r.a-star.edu.sg |
| Source: | Institute for Infocomm Research |  |  |

_____________________________

# Abstract

This contribution studies the relative effectiveness of the temporal and base layer co-located motion vector prediction candidates in the enhancement layer. With the inclusion of the base layer co-located motion vector as a prediction candidate, it is claimed that an average coding performance improvement of 2% can be obtained. The subsequent exclusion of the temporal co-located candidate in the enhancement layers is claimed to lead to a 0.4% coding performance drop on average. This contribution advocates the use of the base layer co-located motion vector as a motion vector prediction and merge candidate and also the removal of the temporal co-located motion vector prediction candidate in the enhancement layers. In this case, it is asserted that the decoder does not have to retain mode and motion information of reference frames in the enhancement layers, reducing memory requirement.

# Introduction

In SMUC 0.1.1, the motion vector prediction (MVP) and merge mechanisms in the enhancement layers are identical to the base layers. The availability of the base layer co-located motion vector provides another possible motion vector predictor and merge candidate. The effectiveness of the base layer co-located motion vector as both a MVP and a merge candidate has been studied in TE5.

This contribution studies the relative effectiveness of the temporal and base layer co-located motion vector prediction candidates. The temporal co-located candidate is excluded in the enhancement layer and replaced with the base layer co-located candidate for MVP and merge.

# Experimental results

We experimented on the software for Test 5.2.2 of TE5 and used the common conditions generated from AHG10 in our experiments. Following Test 5.2.2, the base layer co-located motion vector was added as the first prediction candidate for both MVP and merge.

The results for Test 5.2.2, with base layer co-located motion vector added as a prediction candidate for both MVP and merge is reproduced below:

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -2.9% | -5.6% | -5.4% |  |  |  | -2.9% | -6.6% | -6.9% |
| Class B | -2.3% | -3.8% | -4.0% | -2.5% | -3.9% | -4.2% | -2.5% | -4.2% | -4.5% |
| Overall (EL+BL) | -2.4% | -4.3% | -4.4% | -2.5% | -3.9% | -4.2% | -2.6% | -4.9% | -5.2% |
| Overall (EL) | -4.6% | -7.9% | -8.0% | -6.6% | -9.6% | -10.4% | -5.3% | -9.3% | -9.9% |
| Enc Time[%] | 89.6% |  |  | 88.5% |  |  | 91.0% |  |  |
| Dec Time[%] | 91.5% |  |  | 94.2% |  |  | 96.9% |  |  |
| Enc Mem[%] | 100.0% |  |  | 100.0% |  |  | 100.0% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.8% | -3.5% | -3.4% |  |  |  | -2.6% | -4.3% | -4.4% |
| Class B | -1.3% | -2.4% | -2.4% | -1.4% | -2.2% | -2.3% | -1.9% | -2.7% | -2.9% |
| Overall (EL+BL) | -1.4% | -2.7% | -2.6% | -1.4% | -2.2% | -2.3% | -2.1% | -3.2% | -3.3% |
| Overall (EL) | -2.5% | -4.6% | -4.6% | -3.0% | -4.6% | -4.9% | -3.7% | -5.4% | -5.6% |
| Enc Time[%] | 91.0% |  |  | 87.6% |  |  | 93.1% |  |  |
| Dec Time[%] | 93.1% |  |  | 94.5% |  |  | 98.1% |  |  |
| Enc Mem[%] | 100.0% |  |  | 100.0% |  |  | 100.0% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

When the temporal co-located motion vector was subsequently excluded as a prediction candidate for both MVP and merge, the results are:

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -2.5% | -5.4% | -5.2% |  |  |  | -2.8% | -6.6% | -6.9% |
| Class B | -1.7% | -3.7% | -3.8% | -2.2% | -3.9% | -4.2% | -2.2% | -4.2% | -4.5% |
| Overall (EL+BL) | -1.9% | -4.2% | -4.2% | -2.2% | -3.9% | -4.2% | -2.3% | -4.9% | -5.2% |
| Overall (EL) | -3.8% | -7.7% | -7.8% | -5.9% | -9.6% | -10.4% | -4.8% | -9.4% | -9.9% |
| Enc Time[%] | 92.5% |  |  | 88.8% |  |  | 93.8% |  |  |
| Dec Time[%] | 96.5% |  |  | 95.0% |  |  | 97.8% |  |  |
| Enc Mem[%] | 100.0% |  |  | 100.0% |  |  | 100.0% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.5% | -3.1% | -2.9% |  |  |  | -2.5% | -4.2% | -4.2% |
| Class B | -0.6% | -2.2% | -2.2% | -0.9% | -2.1% | -2.2% | -1.5% | -2.5% | -2.7% |
| Overall (EL+BL) | -0.9% | -2.5% | -2.4% | -0.9% | -2.1% | -2.2% | -1.8% | -3.0% | -3.1% |
| Overall (EL) | -1.7% | -4.3% | -4.2% | -2.1% | -4.4% | -4.7% | -3.2% | -5.2% | -5.4% |
| Enc Time[%] | 96.0% |  |  | 93.7% |  |  | 96.1% |  |  |
| Dec Time[%] | 97.5% |  |  | 98.4% |  |  | 98.0% |  |  |
| Enc Mem[%] | 100.0% |  |  | 100.0% |  |  | 100.0% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

As seen above, most of the gains associated with including the base layer co-located motion vector are retained even with the subsequent exclusion of the temporal co-located candidate, indicating that the base layer co-located candidate is the better predictor in the enhancement layer.

# Conclusion

In this contribution, we studied the relative effectiveness of the temporal and base layer co-located motion vector prediction candidates in the enhancement layer. We recommend the inclusion of the co-located base layer motion vector as a MVP and merge candidate and also the exclusion of the temporal co-located candidate to reduce decoder memory requirement when decoding the enhancement layer.

# Patent rights declaration(s)

Institute for Infocomm Research may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).