| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0074 |
| --- | --- |

| Title: | Non-TE3: Adaptive predictor compensation with generalized residual prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Shan Liu, Mei Guo, Polin Lai, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {peter.chuang, shan.liu, mei.guo, polin.lai, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, the combined results of the adaptive predictor compensation (APC) in JCTVC-L0072 and the generalized residual prediction (GRP) test-2 in JCTVC-L0078 are shown. The APC uses the reconstructed base layer (BL) texture to refine the enhancement layer (EL) sample predictors. The GRP uses the inter prediction residual of the collocated BL block to predict the residual of the current block. The gains of the APC and GRP are additive. Simulation results reportedly show 2.2%, 3.1%, 2.6%, 4.0%, 5.4%, and 5.5% BD-rate savings on average for RA-2x, RA-1.5x, RA-SNR, LDP-2x, LDP-1.5x, and LDP-SNR, respectively, compared with SMuC-0.1.1 anchors. The encoding time increase is 25%, and the decoding time increase is 6%.

# Introduction

In the Tool Experiment 3 (TE3), the base layer (BL) reconstructed current pictures and the BL reconstructed reference pictures are utilized to help the enhancement layer (EL) picture coding. The adaptive predictor compensation (APC) in JCTVC-L0072 [1] (TE3-4.3.1) uses the collocated BL reconstructed texture to refine the EL sample predictors. The generalized residual prediction (GRP) in JCTVC-L0078 [2] (TE3-4.6.2.1) generates the inter prediction pseudo residual of the collocated BL block after upsampling by using the EL motion vectors (MVs) and uses the pseudo residual to predict the EL residual. In this proposal, these two techniques are combined and investigated on top of SMuC-0.1.1 to show the combined coding performance. The rest of this contribution is organized as follows. The proposed algorithm will be described in Section 2, simulation results will be shown in Section 3, and finally conclusions will be given in Section 4.

# Algorithm description

## Adaptive predictor compensation (APC)

In APC, the BL reconstructed texture is used to refine the EL sample predictors. The refined EL predictor is a weighted sum of the original EL predictor and the BL reconstructed sample as follows.

PredEL’ = (PredEL + RecBL_cur) >> 1								(1)

The APC is only applied to each inter-coded CU with a partition mode equal to 2Nx2N and at least one non-zero residue. In addition, an APC_enable_flag is signaled after no_residual_prediction_flag to indicate whether the APC is applied, as shown in Table 1.

Table 1.  Coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enable_flag ) { |  |
| cu_transquant_bypass_flag | ae(v) |
| } |  |
| if( slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] | ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, log2CbSize ) |  |
| else { |  |
| … |  |
| } |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if( PredMode[ x0 ][ y0 ] != MODE_INTRA && !(PartMode = = PART_2Nx2N && merge_flag[x0][y0])) |  |
| no_residual_syntax_flag | ae(v) |
| if( !no_residual_syntax_flag ) { |  |
| MaxTrafoDepth = ( PredMode[ x0 ][ y0 ] = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter ) |  |
| if ( PredMode[ x0 ][ y0 ] != MODE_INTRA && PredMode[ x0 ][ y0 ] != MODE_INTRA_BL) && PartMode = = PART_2Nx2N) |  |
| APC_enable_flag | ae(v) |
| transform_tree( x0, y0 x0, y0, log2CbSize, 0, 0 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |

## Generalized residual prediction (GRP)

In GRP, the inter prediction pseudo residual of the collocated BL block by using the EL motion vectors (MVs) is generated. The pseudo residual is used to predict the EL residual. The GRP is applied to inter CUs and skip mode CUs. The framework of this method is shown in Error: Reference source not found1, where a case of uni-prediction is illustrated. The CurEL and RecBL_cur denote the current block in the EL current picture and its collocated block in upsampled BL current picture, respectively. The PredEL and RecBL_ref denote the inter prediction block of CurEL in the EL reference picture and its collocated block in upsampled BL reference picture, respectively.

First the BL pseudo residual is generated as follows.

ResBL = RecBL_cur – RecBL_ref   		 						(2)

Then, the inter-layer residual prediction is applied. The weighted BL pseudo residual is used to predict the EL residual. So the refined EL predictor is as follow.

PredEL’ = PredEL + w * (RecBL_cur – RecBL_ref)							(3)

The weighting factor “w” is signaled at CU level as a weighting index, inter_resi_weight_idx. The weighting index 0, 1, and 2 are used to indicate the weighting factors 0, 0.5, and 1, respectively.

Figure 1.  Illustration of generalized residual prediction

## Combination of APC and GRP

In this proposal, the APC and GRP are implemented on top of SMuC-0.1.1 to show the combined coding performance. Two CU-level flags, APC_enable_flag and inter_resi_weight_idx are used to indicate whether the APC or GRP is applied. If both APC and GRP are applied, the refined EL predictor is as follow.

PredEL’ = (PredEL + w * (RecBL_cur – RecBL_ref) + RecBL_cur) >> 1				(4)

# Simulation results

Experiments were conducted according to the TE-B3 description defined in JCTVC-K1103 [3], and the software is based on SMuC-0.1.1. Table 2 summarizes the results of the APC in JCTVC-L0072, the GRP test-2 in JCTVC-L0078, and the combination of APC and GRP test-2. The combined results of the APC and the GRP test-2 are shown in Table 3 in more details. For the combination of APC and GRP test-2, simulation results reportedly show 2.2%, 3.1%, 2.6%, 4.0%, 5.4%, and 5.5% BD-rate savings on average for RA-2x, RA-1.5x, RA-SNR, LDP-2x, LDP-1.5x, and LDP-SNR, respectively, compared with SMuC-0.1.1 anchors. The encoding time and decoding time are roughly increased by 24% and 6%, respectively. From Table 2, we can see that the gains of APC and GRP are additive.

Table 2.  Summary of APC, GRP, and combined results

|  | RA-2x | RA-1.5x | RA-SNR | LDP-2x | LDP-1.5x | LDP-SNR |
| --- | --- | --- | --- | --- | --- | --- |
| APC in TE3-4.3.1 | -0.4% | -0.6% | -0.8% | -1.2% | -1.5% | -2.4% |
| GRP in TE3-4.6.2.1 test 2 | -1.9% | -2.8% | -2.1% | -2.7% | -3.9% | -3.4% |
| APC + GRP test 2 | -2.2% | -3.1% | -2.6% | -4.0% | -5.4% | -5.5% |

Table 3.  Results of the proposed combination of APC and GRP

# Conclusions

In this contribution, the proposed methods in TE3-4.3.1 (APC) and TE3-4.6.2.1 (GRP) are investigated together. The combined results reportedly show 2.2%, 3.1%, 2.6%, 4.0%, 5.4%, and 5.5% BD-rate savings on average for RA-2x, RA-1.5x, RA-SNR, LDP-2x, LDP-1.5x, and LDP-SNR, respectively, compared with SMuC-0.1.1 anchors. The gains of APC and GRP are additive.

# References

- T.-D. Chuang, Y.-W. Huang, P. Lai, S. Liu, S. Lei, “TE3-4.3.1: Adaptive predictor compensation by using reconstructed BL texture,” Document of Joint Collaborative Team on Video Coding, JCTVC-L0072, January, 2013.

- X. Li, J. Chen, K. Rapaka, M. Karczewicz, “TE3: Results of Test 4.6.2.1 on generalized residual prediction”, Document of Joint Collaborative Team on Video Coding, JCTVC-L0078, January, 2013.

- J. Chen, A. Segall, E. Alshina, S. Liu, J. Dong, J. Park, “Description of Tool Experiment B3: Combined prediction in SHVC,” Document of Joint Collaborative Team on Video Coding, JCTVC-K1103, October, 2012.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).