| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0075 |
| --- | --- |

| Title: | TE4.4: Inter-layer adaptive filter on upsampled base layer |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ching-Yeh Chen, Chia-Yang Tsai, Ximin, Zhang, Shan Liu, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {chingyeh.chen, chiayang.tsai, ximin.zhang, shan.liu, yuwen.huang, shawmin.lei}@ mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, inter-layer adaptive filtering is applied to upsampled base layer pictures for improving sample predictors of inter-layer intra prediction. The filter footprint is 7x7Cross+3x3Square, and only one filter is allowed per color component per slice. Compared to SMuC-0.1.1, simulation results show BD-rates of the proposed method are -0.1%, -0.3%, -0.1%, -0.3%, -2.1%, -0.3%, -0.8%, and -3.3% for AI-2x, AI-1.5x, RA-2x, RA-1.5x, RA-SNR, LDP-2x, LDP-1.5x, and LDP-SNR, respectively. The encoding time increase is negligible. The decoding time increase is 2.6% on average.

# Introduction

Inter-layer adaptive filter is introduced to further improve inter-layer intra prediction in responses to Call for Proposals (CfP) of HEVC scalable extension [1] [2]. Therefore, in the 11th JCT-VC meeting, a tool experiment, TE4: inter-layer filtering [3], is established to evaluate the performances of inter-layer filtering techniques. In this contribution, inter-layer adaptive filtering on upsampled base layer (BL) pictures is implemented by using Wiener filtering to reduce distortions between upsampled based-layer pictures and original enhancement-layer pictures, and the related experimental results are provided. The rest of this contribution is organized as follows. The proposed method will be introduced in Section 2, simulation results will be shown in Section 3, and conclusions will be given in Section 4.

# Proposed method

Figure 1 shows the block diagram of HEVC scalable extension with the proposed inter-layer adaptive filter. The inter-layer adaptive filter is applied after the upsampling process of the BL picture and before inter-layer intra prediction. The purpose of the inter-layer adaptive filter is to improve sample predictors of inter-layer intra prediction by reducing the distortion between the upsampled based-layer picture and the original picture in the enhancement layer (EL).

Figure 1.   Block diagram of HEVC scalable extension with the proposed inter-layer adaptive filter

In our proposed inter-layer adaptive filter, only single filter is allowed per color component per slice, and the selected filter footprint is 7x7Cross+3x3Square, as shown in Figure 2, where 1/2-symmetry is applied and the numbers denote filter coefficient indices. The filter coefficients are coded in slice header by using signed exponential Golomb codes, and the related syntax of our proposed inter-layer adaptive filter are summarized in Table 1.

Figure 2.  The proposed filter footprint is 7x7Cross+3x3Square with 1/2-symmetry

Table 1.  The proposed syntax of inter-layer adaptive filter

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| .. |  |
| if(!dependent_slice_segment_flag ) { |  |
| .. |  |
| if( loop_filter_across_slices_enabled_flag && ( slice_sao_luma_flag ¦ ¦ slice_sao_chroma_flag ¦ ¦ !slice_disable_deblocking_filter_flag ) ) |  |
| slice_loop_filter_across_slices_enabled_flag | u(1) |
| if( LayerId > 0) { |  |
| interlayer_adaptive_filter_enabled_flag | u(1) |
| if( interlayer_adaptive_filter_enabled_flag ) { |  |
| for( cIdx=0; cIdx< 3; cIdx++) { |  |
| interlayer_adaptive_filter_component_enabled_flag[cIdx] | u(1) |
| if( interlayer_adaptive_filter_component_enabled_flag[cIdx]) |  |
| for( i=0; i< 9; i++) |  |
| interlayer_adaptive_filter_coef[cIdx][i] | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| .. |  |
| } |  |

# Experimental results

The experimental results are shown in Table 2. The software is developed based on SMuC-0.1.1, and the test condition is described in JCTVC-K1104 [3]. Compared to SMuC-0.1.1, the proposed inter-layer adaptive filter can achieve BD-rates as -0.1%, -0.3%, -0.1%, -0.3%, -2.1%, -0.3%, -0.8%, and -3.3% for AI-2x, AI-1.5x, RA-2x, RA-1.5x, RA-SNR, LDP-2x, LDP-1.5x, and LDP-SNR, respectively. The encoding time increase is negligible. The decoding time increases are 1.9%, 3.6%, 1.2%, 2.7%, 3.7%, 1.1%, 2.2%, and 4.1% for AI-2x, AI-1.5x, RA-2x, RA-1.5x, RA-SNR, LDP-2x, LDP-1.5x, and LDP-SNR, respectively.

# Conclusion

It is proposed to apply inter-layer adaptive filtering on upsampled base layer pictures. The selected filter footprint is 7x7Cross+3x3Square, and only one filter is used for each color component per slice. Compared with SMuC-0.1.1, the proposed inter-layer adaptive filter can provide 0.1%, 0.3%, 0.1%, 0.3%, 2.1%, 0.3%, 0.8%, and 3.3% BD-rate reductions for AI-2x, AI-1.5x, RA-2x, RA-1.5x, RA-SNR, LDP-2x, LDP-1.5x, and LDP-SNR, respectively, with only 2.6% decoding time increase.

# References

- C. Kim, J. Park, J. Kim, Hendry, S. Park, Y. Jeon, J. Lim, N. Park, B. Jeon, S. Liu, X. Zhang, M. Guo, Z. Chen, T.-D. Chuang, S.-T. Hsiang, C.-Y. Chen, C.-Y. Tsai, C.-M. Fu, C.W. Hsu, Y.-W. Huang, S. Lei, “Description of scalable video coding technology proposal by LG Electronics and MediaTek (differential coding mode on),” Document of Joint Collaborative Team on Video Coding, JCTVC-K0033, October, 2012.

- C. Kim, J. Park, J. Kim, Hendry, S. Park, Y. Jeon, J. Lim, N. Park, B. Jeon, S. Liu, X. Zhang, M. Guo, Z. Chen, T.-D. Chuang, S.-T. Hsiang, C.-Y. Chen, C.-Y. Tsai, C.-M. Fu, C.W. Hsu, Y.-W. Huang, S. Lei, “Description of scalable video coding technology proposal by LG Electronics and MediaTek (differential coding mode off),” Document of Joint Collaborative Team on Video Coding, JCTVC-K0050, October, 2012.

- J. Chen, A. Segall, E. Alshina, S. Liu, J. Dong, J. Park, “Description of Tool Experiment B4: Inter-layer filtering in SHVC,” Document of Joint Collaborative Team on Video Coding, JCTVC-K1104, October, 2012.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Table 2.  Simulation results of the proposed inter-layer adaptive filter