| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0076 |
| --- | --- |

| Title: | Non-TE4.4: Inter-layer adaptive filter on upsampled base layer with CU on/off flags |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ching-Yeh Chen, Chia-Yang Tsai, Ximin, Zhang, Shan Liu, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {chingyeh.chen, chiayang.tsai, ximin.zhang, shan.liu, yuwen.huang, shawmin.lei}@ mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, it is proposed to add CU on/off flags to enable or disable the inter-layer adaptive filter on the upsampled BL in JCTVC-L0075 at CU level. It is reported that the inter-layer adaptive filter  on the upsampled BL with CU on/off flags can provide 0.5%, 0.6%, 0.3%, 0.5%, 2.2%, 0.3%, 0.9%, and 3.4% BD-rate reductions for AI-2x, AI-1.5, RA-2x, RA-1.5x, RA-SNR, LDP-2x, LDP-1.5x, and LDP-SNR, respectively. The encoding time increase is 5.9%, and the decoding time increase is 3.0%.

# Proposed method

An inter-layer adaptive filter on upsampled base layer (BL) is proposed in JCTVC-L0075 [1]. In this contribution, we added CU-level on/off flags on top of the inter-layer adaptive filter to enable or disable it at CU level.

# Experimental results

The experimental results are shown in Table 1. The software is developed based on SMuC-0.1.1, and the test condition is described in JCTVC-K1104 [2]. Compared to SMuC-0.1.1, the proposed inter-layer adaptive filter on upsampled BL with CU on/off flags can provide 0.5%, 0.6%, 0.3%, 0.5%, 2.2%, 0.3%, 0.9%, and 3.4% BD-rate reductions for AI-2x, AI-1.5, RA-2x, RA-1.5x, RA-SNR, LDP-2x, LDP-1.5x, and LDP-SNR, respectively. As for run time comparison, 13.3% encoding time and 3.3% decoding time are increased for intra test cases, and 3.5% encoding time and 2.9% decoding time are increased for inter test cases.

# References

- C.-Y. Chen, C.-Y. Tsai, X. Zhang, S. Liu, Y.-W. Huang, S. Lei, “TE4.4: Inter-layer adaptive filter on upsampled BL,” Document of Joint Collaborative Team on Video Coding, JCTVC-L0075, January, 2013.

- J. Chen, A. Segall, E. Alshina, S. Liu, J. Dong, J. Park, “Description of Tool Experiment B4: Inter-layer filtering in SHVC,” Document of Joint Collaborative Team on Video Coding, JCTVC-K1104, October, 2012.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Table 1.  Simulation results of the proposed inter-layer adaptive filter