| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0080 WG 11Number :_ m27408 |
| --- | --- |

| Title: | TE 3: Cross-Check of test 4.6.3 (JCTVC-L0038) in TE 3 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Ankur Saxena Elena Alshina Felix C. Fernandes 1301 E. Lookout Drive, Richardson, TX – 75082, USA. | Tel: Email: | 1-9727617761 asaxena@sta.samsung.com, elena_a.alshina@samsung.com felix.f@sta.samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

This contribution summarizes Samsung’s cross-check for JCTVC-L0038 proposal on base enhanced motion compensated prediction. The simulation results (BD Rates and PSNR’s) as verified by Samsung match closely with the results provided by the proponents in JCTVC-L0038.

# Introduction

JCTVC-L0038 [1] proposes a base-enhanced motion compensation prediction scheme for the enhancement layer prediction in Tool Experiment 3. We cross-checked the RA and LD-P settings for this tool in this cross-check document.

# Simulation Results

The test conditions as stipulated in common conditions for testing were followed for conducting the simulations.

## R-D Results

Table 1 shows the simulation results for JCTVC-L0038 proposal as cross-checked by Samsung. Detailed results are available in the attached Excel sheet. The BD Rates and PSNR’s for the tests matched with those provided by the proponents, while the encoding and decoding times are within cluster noise.

Table 1: BD-Rate and PSNR results for cross-check of Test 4.6.3 in Tool Experiment 3

|  |  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V | Y | U | V |  |
|  | Class A | -1.8% | -5.1% | -5.2% |  |  |  | -1.3% | -4.6% | -5.1% |  |
|  | Class B | -1.7% | -3.0% | -3.2% | -2.5% | -4.5% | -4.9% | -1.8% | -4.0% | -4.6% |  |
|  | Overall (EL+BL) | -1.7% | -3.6% | -3.8% | -2.5% | -4.5% | -4.9% | -1.6% | -4.2% | -4.7% |  |
|  | Overall (EL) | -3.2% | -6.9% | -7.2% | -5.8% | -10.6% | -11.6% | -3.1% | -8.2% | -9.1% |  |
|  | Enc Time[%] | 123% |  |  | 114% |  |  | 117% |  |  |  |
|  | Dec Time[%] | 109% |  |  | 105% |  |  | 109% |  |  |  |
|  | Enc Mem[%] | 100% |  |  | 100% |  |  | 100% |  |  |  |
|  | BL Match | Matched |  |  | Matched |  |  | Matched |  |  |  |
|  |  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |  |
|  |  | Y | U | V | Y | U | V | Y | U | V |  |
|  | Class A | -2.4% | -4.6% | -4.5% |  |  |  | -2.3% | -4.3% | -4.4% |  |
|  | Class B | -2.3% | -2.1% | -2.1% | -3.4% | -3.8% | -3.6% | -3.1% | -3.6% | -4.0% |  |
|  | Overall (EL+BL) | -2.3% | -2.9% | -2.8% | -3.4% | -3.8% | -3.6% | -2.9% | -3.8% | -4.2% |  |
|  | Overall (EL) | -4.1% | -5.4% | -5.5% | -7.5% | -8.5% | -8.6% | -5.0% | -6.7% | -7.4% |  |
|  | Enc Time[%] | 119% |  |  | 110% |  |  | 115% |  |  |  |
|  | Dec Time[%] | 106% |  |  | 104% |  |  | 108% |  |  |  |
|  | Enc Mem[%] | 100% |  |  | 100% |  |  | 100% |  |  |  |
|  | BL Match | Matched |  |  | Matched |  |  | Matched |  |  |  |

# Conclusions

We have cross-checked JCTVC-L0038, and verified the simulation results.

# References

1. J. Lainema and K. Ugur, “TE3: Results of test 4.6.3 on base enhanced motion compensated prediction,” JCTVC-L0038, Geneva, Switzerland, January 2013.