| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0083 |
| --- | --- |

| Title: | TEA1: Fixed up-sampling filter tests by Samsung (tests 4.1 and 4.4) |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Contribution |  |  |
| Author(s) or Contact(s): | Elena Alshina Alexander Alshin JeongHoon Park 416, Maetan 3-Dong, Gyeonggi-do, Suwon, Korea | Tel: Email: | +82 10 3026 1305 {elena_a.alshina, alexander_a.alshin, jeonghoon}@samsung.com |
| Source: | Samsung Electronics, Ltd. |  |  |

_____________________________

# Abstract

In this contribution default SMuC 0.1.1 up-sampling filter performance was tested (test 4.1 in TEA1). Additionally performance impact of accurate phase derivation for Chroma up-sampling was evaluated (test 4.4 in TEA1). Compare to HM8.1 simulcast tested up-sampling filter scheme which used for inter-layer texture prediction generation provides 17.1% (x2) and 26.9% (x1.5) Luma BD-rate reduction. BD-rate reduction for Chroma (U, V) components is 12.4%, 11.9% (x2) and 23.4%, 22.3% (x1.5). Without accurate Chroma phase alignment average there is 0.1% BD-rate degradation for Chroma in (x2) test, but there is 0.1% Chroma BD-rate gain. Average BD-rate for Luma is the same. Details of filter coefficients generation for these tests are provided in this contribution.

# Introduction

The major coding tool in scalable coding is inter-layer texture prediction. In case of spatial scalability interpolation for fractional positions is needed for inter-layer texture prediction. And so up-sampling filter is a key part of spatial scalable codec.

Up-sampling filter in reference s/w is implemented as 2D separable filter. Horizontal up-sampling is performed first and then followed by vertical up-sampling. In order to keep consistency with HEVC motion compensation interpolation module 8-taps filter is implemented for Luma and 4-taps for Chroma. Filter coefficients have the same precision as MC interpolation filter and for ½ (Luma), ¼, 3/8 and 7/8 (Chroma) positions filter coefficients are identical to HEVC MC interpolation.

According to BoG [1] only 2 specific spatial scalability factor were used in this TE x2 and x1.5. In order provide up-sampling with accurate Chroma phase alignment set of filters described in [2] and in Tables 1  and 2 is needed.

Table 1. Luma up-sampling filters for Test 4.1.

| Phase shift | Luma up-sampling filter | Comment |
| --- | --- | --- |
| 0 | { 0, 0, 0, 64, 0, 0, 0, 0}/64 | Trivial filter, equivalent to no filtering |
| 1/3 | {-1, 4, -11, 52, 26, -8, 3, -1}/64 | Used for up-sampling in x1.5 case |
| 1/2 | {-1, 4, -11, 40, 40, -11, 4, -1}/64 | Used for up-sampling in x2 case |
| 2/3 | {-1, 3, -8, 26, 52, -11, 4, -1}/64 | Used for up-sampling in x1.5 case |

Table 2. Chroma up-sampling filters for Test 4.1.

| Phase shift | Chroma up-sampling filter | Comment |
| --- | --- | --- |
| 0 | {0, 64, 0, 0}/64 | Trivial filter, equivalent to no filtering |
| 1/4 | {-4, 54, 16, -2}/64 | Used for up-sampling in x1.5 case (only for accurate Chroma alignment) |
| 1/3 | {-5, 50, 22, -3}/64 | Used for up-sampling in x1.5 case |
| 3/8 | {-6, 46, 28, -4}/64 | Used for up-sampling in x2 case (only for accurate Chroma alignment) |
| 1/2 | {-4, 36, 36, -4}/64 | Used for up-sampling in x2 case |
| 7/12 | {-4, 30, 43, -5}/64 | Used for up-sampling in x1.5 case (only for accurate Chroma alignment) |
| 2/3 | {-3, 22, 50, -5}/64 | Used for up-sampling in x1.5 case |
| 7/8 | {-2, 10, 58, -2}/64 | Used for up-sampling in x2 case (only for accurate Chroma alignment) |
| 11/12 | {-1, 5, 62, -2}/64 | Used for up-sampling in x1.5 case (only for accurate Chroma alignment) |

These filter coefficients were first proposed in [3] and used in the most part of CfP responses on scalable HEVC extensions submitted in October, 2012.

# Phase derivation process

The phase derivation process is explained on Fig. 1 for scalability ratio 1:2 and on Fig. 2 for scalability ratio 2:3. Notations are as follows:

|  | Low-resolution Luma samples |
| --- | --- |
|  | Low-resolution Chroma samples |
|  | High-resolution Luma samples |
|  | High-resolution Chroma samples |
|  | Accurate phase shift (no rounding to 1/16 accuracy) |

Figure 1. Phase derivation for up-sampling filter with accurate Chroma alignment. Scalability ratio 1:2.

Figure 2. Phase derivation for up-sampling filter with accurate Chroma alignment. Scalability ratio 3:2.

Phase derivation process uses the same rules for Luma horizontal and vertical and Chroma horizontal up-sampling. Only for vertical Chroma up-sampling phase derivation process is different. This is due to displacement of down-sampled Chroma position in 4:2:0 color format.

In order to avoid division by 3 in phase derivation process the following code is used in reference s/w:

Int   iShiftX = 16;  Int   iShiftY = 16;

(1)

Int   iPhaseX = 0;

Int   iPhaseY = 0; // Int   iPhaseY = 1; // in vertical Chroma up-sampling

Int   iAddX       = ( ( ( iBWidth * iPhaseX ) << ( iShiftX - 2 ) ) + ( iEWidth >> 1 ) ) / iEWidth + ( 1 << ( iShiftX - 5 ) );

Int   iAddY       = ( ( ( iBHeight * iPhaseY ) << ( iShiftY - 2 ) ) + ( iEHeight >> 1 ) ) / iEHeight+ ( 1 << ( iShiftY - 5 ) );

Int   iDeltaX     = 4 * iPhaseX;  Int   iDeltaY     = 4 * iPhaseY;

Int iShiftXM4 = iShiftX - 4;  Int iShiftYM4 = iShiftY - 4;

Int   iScaleX     = ( ( iBWidth << iShiftX ) + ( iEWidth >> 1 ) ) / iEWidth;

Int   iScaleY     = ( ( iBHeight << iShiftY ) + ( iEHeight >> 1 ) ) / iEHeight;

This code provides phase calculation with accuracy 1/16.

Filter coefficients are stored in table 8x16 for Luma up-sampling:

{

{  0,  0,  0, 64,  0,  0,  0,  0},

{CNU,CNU,CNU,CNU,CNU,CNU,CNU,CNU},

{CNU,CNU,CNU,CNU,CNU,CNU,CNU,CNU},

{CNU,CNU,CNU,CNU,CNU,CNU,CNU,CNU},

{CNU,CNU,CNU,CNU,CNU,CNU,CNU,CNU},

{ -1, 4, -11, 52, 26,  -8, 3, -1},

{CNU,CNU,CNU,CNU,CNU,CNU,CNU,CNU},

{CNU,CNU,CNU,CNU,CNU,CNU,CNU,CNU},

{ -1, 4, -11, 40, 40, -11, 4, -1},

{CNU,CNU,CNU,CNU,CNU,CNU,CNU,CNU},

{CNU,CNU,CNU,CNU,CNU,CNU,CNU,CNU},

{ -1, 3,  -8, 26, 52, -11, 4, -1},

{CNU,CNU,CNU,CNU,CNU,CNU,CNU,CNU},

{CNU,CNU,CNU,CNU,CNU,CNU,CNU,CNU},

{CNU,CNU,CNU,CNU,CNU,CNU,CNU,CNU},

{CNU,CNU,CNU,CNU,CNU,CNU,CNU,CNU}

};

and 4x16 table for Chroma up-sampling:

{

{  0, 64,  0,  0},

{CNU,CNU,CNU,CNU},

{CNU,CNU,CNU,CNU},

{CNU,CNU,CNU,CNU},

{ -4, 54, 16, -2},

{ -5, 50, 22, -3},

{ -6, 46, 28, -4},

{CNU,CNU,CNU,CNU},

{ -4, 36, 36, -4},

{ -4, 30, 43, -5},

{CNU,CNU,CNU,CNU},

{ -3, 22, 50, -5},

{CNU,CNU,CNU,CNU},

{CNU,CNU,CNU,CNU},

{ -2, 10, 58, -2},

{ -1,  5, 62, -2}

};

Abbreviation “CNU” means that up-sampling filter coefficients in these positions are not used. These up-sampling filters are reserved for spatial scalability with arbitrary ratio (not 1:2 or 2:3).

So up-sampling filter design [2] provides both accurate phase for x2 and x1.5 spatial scalability and doesn’t require division in implementation.

# Spectral analysis of tested filter

For scalability x2 up-sampling filter is the same with interpolation MC filter. Details of that filter generation can be found in [4].

For Luma up-sampling filter the frequency responses in amplitude and phase domain are shown on Figure 3 and Figure 4. We analysed test filter together with other filters-candidates tested in TEA1. Among them:

- {-1, 5, -12, 52, 26, -9, 4, -1} /64  TEA1 test 4.1

- {-1, 4, -11, 53, 25,  -8, 3, -1}/64 TEA1 test 4.2

- {-5, 11, -24,107,53, -18, 8, -4}/128 TEA1 test 4.1

- {-1, 4, -11, 52, 26,  -8, 3, -1}/64  TEA1 test 4.5

Ideal frequency response is straight line (both in amplitude and frequency domains), it is shown on both figures.  It is easy to see that frequency response of tested here filter {-1, 5, -12, 52, 26, -9, 4, -1} /64 has minimal discrepancy with ideal filter response both in amplitude and frequency domain. It was a criterion for filter coefficients selection when this filter was designed for [3]. According to our experience minimal deviation from ideal filter in terms of frequency response is a guaranty of best performance both in motion compensation and up-sampling interpolation. This was confirmed in this tool experiment one more time, since testes filter shows the best performance.

There is performance summary for up-sampling filters mentioned on figures 1-2.

| Luma UF | ×2 |  |  | ×1.5 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Test 4.1 {-1,4,-11,52,26,-8,3,-1}/64 | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Test 4.2 7b {-5,11,-24,107,53,-18,8,-4}/128 | 0,3% | 0,6% | 0,5% | 0,3% | 1,3% | 1,2% |
| Test 4.2 6b*{ -2,5,-12, 54,26, -9,4,-2} /64 | 0,0% | 0,4% | 0,4% | 0,1% | 1,1% | 1,1% |
| Test 4.3** {-1,5,-12,52,26,-9,4,-1}/64 | NA | NA | NA | 0,2% | 0,2% | 0,3% |
| Test 4.5**{-1,5,-11,53,25,-5,3,-1}/64 | 0,0% | 0,1% | 0,2% | 0,4% | 0,5% | 0,4% |

* not present in any CfP response

** withdrawn from TEA1

# Test results summary.

As required in TEA1 description proposed algorithm was tested vs HM8.1 simulcast in All Intra, Random Access and Low-Delay P (x2 , x1.5) tests. Simulation results for test 4.1 are summarised in Table 3 below. This is SMuC 0.1.1 reference s/w performance, independently tested by Qualcomm, I2R and Samsung.

Table 3. Summation results for SMuC 0.1.1 vs HM8.1 simulcast.

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -27.0% | -27.2% | -28.0% |  |  |  |
| Class B | -21.1% | -21.2% | -21.0% | -32.6% | -33.0% | -33.0% |
| Overall (EL+BL) | -22.8% | -22.9% | -23.0% | -32.6% | -33.0% | -33.0% |
| Overall (EL) | -34.8% | -35.2% | -35.3% | -58.1% | -58.2% | -58.3% |
| Enc Time[%] | 115.2% |  |  | 113.8% |  |  |
| Dec Time[%] | 107.3% |  |  | 107.4% |  |  |
| Enc Mem[%] | 110.8% |  |  | 109.6% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -18.5% | -7.8% | -9.3% |  |  |  |
| Class B | -15.5% | -9.2% | -7.9% | -26.1% | -20.3% | -18.8% |
| Overall (EL+BL) | -16.4% | -8.8% | -8.3% | -26.1% | -20.3% | -18.8% |
| Overall (EL) | -25.2% | -13.6% | -12.9% | -47.2% | -37.9% | -35.2% |
| Enc Time[%] | 109.0% |  |  | 111.7% |  |  |
| Dec Time[%] | 132.5% |  |  | 134.1% |  |  |
| Enc Mem[%] | 112.1% |  |  | 110.0% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -13.6% | -1.9% | -3.0% |  |  |  |
| Class B | -11.7% | -6.9% | -5.1% | -21.9% | -17.0% | -15.2% |
| Overall (EL+BL) | -12.3% | -5.5% | -4.5% | -21.9% | -17.0% | -15.2% |
| Overall (EL) | -19.6% | -8.6% | -7.2% | -40.6% | -32.2% | -29.2% |
| Enc Time[%] | 108.0% |  |  | 111.1% |  |  |
| Dec Time[%] | 134.8% |  |  | 137.6% |  |  |
| Enc Mem[%] | 112.2% |  |  | 110.5% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
| AVG | Y | U | V | Y | U | V |
| BD-rate | -17.1% | -12.4% | -11.9% | -26.9% | -23.4% | -22.3% |
| Enc Time[%] | 111% |  |  |  |  |  |
| Dec Time[%] | 126% |  |  |  |  |  |
| Enc Mem[%] | 111% |  |  |  |  |  |

It should be mentioned that in reference SW up-sampling filter is implemented with some redundant operations: int-pel samples are filtered by trivial filter { 0, 0, 0, 64, 0, 0, 0, 0}/64 which is equivalent to no filtering. So decoding time with better implementation should be lower: half filtration operations are not need in x2 case, 1/3 filtration operations are need in x1.5 case. The best way to optimize this part of SMuC s/w is just re-use existing generic interpolation filter implementation in MC framework [5].

In test 4.4 the performance impact of accurate Chroma phase alignment was evaluated. In order to do it SMuC 0.1.1 s/w was modified, namely in phase derivation process (1) for vertical Chroma up-sampling

Int   iPhaseY = 1;

was replaced by

Int   iPhaseY = 0;

After this modification phase derivation process is the same for all cases: Horizontal/Vertical, Luma/Chroma. This will simplify design and shorten description in the specification. Additionally 5 of 8 Chroma up-sampling filters mentioned in Table 2 are not needed w/o accurate Chroma phase alignment.

Test results are summarized in Table 4 below. Samsung would like to thank TI from the cross-check of these test results.

Table 4. Summation results for test 4.4 (non-accurate Chroma phase alignment) vs HM8.1 simulcast

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -27.0% | -27.1% | -27.8% |  |  |  |
| Class B | -21.0% | -21.1% | -20.9% | -32.6% | -33.0% | -33.1% |
| Overall (EL+BL) | -22.7% | -22.8% | -22.9% | -32.6% | -33.0% | -33.1% |
| Overall (EL) | -34.7% | -35.0% | -35.2% | -58.0% | -58.2% | -58.4% |
| Enc Time[%] | 114.6% |  |  | 113.5% |  |  |
| Dec Time[%] | 106.1% |  |  | 105.9% |  |  |
| Enc Mem[%] | 110.3% |  |  | 108.5% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -18.4% | -7.9% | -9.2% |  |  |  |
| Class B | -15.5% | -9.1% | -7.8% | -26.1% | -20.3% | -18.8% |
| Overall (EL+BL) | -16.3% | -8.7% | -8.2% | -26.1% | -20.3% | -18.8% |
| Overall (EL) | -25.2% | -13.5% | -12.8% | -47.2% | -38.0% | -35.4% |
| Enc Time[%] | 108.7% |  |  | 111.5% |  |  |
| Dec Time[%] | 130.5% |  |  | 133.3% |  |  |
| Enc Mem[%] | 112.0% |  |  | 110.0% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -13.6% | -1.9% | -2.7% |  |  |  |
| Class B | -11.7% | -6.8% | -5.0% | -21.9% | -17.0% | -15.3% |
| Overall (EL+BL) | -12.2% | -5.4% | -4.4% | -21.9% | -17.0% | -15.3% |
| Overall (EL) | -19.5% | -8.4% | -7.0% | -40.5% | -32.2% | -29.2% |
| Enc Time[%] | 107.7% |  |  | 110.9% |  |  |
| Dec Time[%] | 133.3% |  |  | 137.1% |  |  |
| Enc Mem[%] | 112.2% |  |  | 110.5% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
| AVG | Y | U | V | Y | U | V |
| BD-rate | -17.1% | -12.3% | -11.8% | -26.9% | -23.5% | -22.4% |
| Enc Time[%] | 111% |  |  |  |  |  |
| Dec Time[%] | 124% |  |  |  |  |  |
| Enc Mem[%] | 111% |  |  |  |  |  |

This is easy to see that there is no average BD-rate change for Luma and 0.1% performance degradation for Chroma in x2 case, but 0.1% Chroma performance improvement in x1.5 case.

In order to simplify comparison in Table 5 results are presented in a form when anchor uses accurate Chroma phase alignment, but modification is not.

Table 5. Summation results for test 4.4 (non-accurate Chroma phase alignment) vs SMuC 0.1.1

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.2% | 0.3% |  |  |  |
| Class B | 0.1% | 0.2% | 0.1% | 0.0% | -0.1% | -0.1% |
| Overall (EL+BL) | 0.1% | 0.2% | 0.2% | 0.0% | -0.1% | -0.1% |
| Overall (EL) | 0.1% | 0.4% | 0.4% | 0.1% | -0.3% | -0.4% |
| Enc Time[%] | 99.5% |  |  | 99.7% |  |  |
| Dec Time[%] | 98.8% |  |  | 98.6% |  |  |
| Enc Mem[%] | 99.5% |  |  | 99.0% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | 0.2% |  |  |  |
| Class B | 0.1% | 0.2% | 0.1% | 0.0% | -0.1% | -0.1% |
| Overall (EL+BL) | 0.0% | 0.1% | 0.2% | 0.0% | -0.1% | -0.1% |
| Overall (EL) | 0.1% | 0.2% | 0.3% | 0.1% | -0.2% | -0.3% |
| Enc Time[%] | 99.7% |  |  | 99.8% |  |  |
| Dec Time[%] | 98.5% |  |  | 99.4% |  |  |
| Enc Mem[%] | 100.0% |  |  | 100.1% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.5% |  |  |  |
| Class B | 0.1% | 0.2% | 0.2% | 0.0% | 0.0% | -0.1% |
| Overall (EL+BL) | 0.0% | 0.1% | 0.3% | 0.0% | 0.0% | -0.1% |
| Overall (EL) | 0.1% | 0.3% | 0.5% | 0.1% | 0.0% | -0.2% |
| Enc Time[%] | 99.7% |  |  | 99.8% |  |  |
| Dec Time[%] | 98.9% |  |  | 99.6% |  |  |
| Enc Mem[%] | 100.0% |  |  | 100.0% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
| AVG | Y | U | V | Y | U | V |
| BD-rate | 0.0% | 0.1% | 0.2% | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 100% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  |  |  |  |
| Enc Mem[%] | 100% |  |  |  |  |  |

Again there is no effect for Luma BD-rate and under-noise level performance change for Chroma. Based on these test data we conclude that accurate Chroma phase alignment during up-sampling doesn’t have crucial effect on scalable HEVC extension performance. For design simplification we would suggest to unify phase derivation process and not to use accurate Chroma phase alignment. It should be mentioned that accurate Chroma phase alignment is not considered in HEVC motion compensation process.

# Conclusion

Performance of up-sampling filter first published 1.5 years ago and use in most part of CfP responses on HEVC scalable extensions was verified in this test. Compare to HM8.1 simulcast tested up-sampling filter scheme provides 17.1% (x2) and 26.9% (x1.5) Luma BD-rate reduction. BD-rate reduction for Chroma (U, V) components is 12.4%, 11.9% (x2) and 23.4%, 22.3% (x1.5). Samsung suggests keeping this up-sampling filter as default filter in SMuC reference s/w.

Without accurate Chroma phase alignment average there is 0.1% BD-rate degradation for Chroma in (x2) test, but there is 0.1% Chroma BD-rate gain. Average BD-rate for Luma is the same. Samsung suggests removing accurate Chroma phase alignment from SMuC reference for design simplification.

# References

[1] A. Segall, BoG report on HEVC scalable extensions, JCTVC-K0354, October, 2012.

[2] E. Alshina, H. Lakshman, J.Dong, J.Chen, A.Luthra, Suggested up-sampling filter design for tool experiments on HEVC scalable extension, JCTVC-K0378, October, 2012

[3] J. Boyce, D. Hong, W. Jang, A. Abbas (Vidyo) , Information for HEVC scalability extension, JCTVC-G078, November, 2011

[4] E. Alshina, A.Alshin (Samsung), CE3: DCT derived interpolation filter test by Samsung, JCTVC-F247, July, 2011

[5] F. Bossen, On the motion compensation process, JCTVC-F537, July, 2011

# Patent rights declaration(s)

Samsung Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).