| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0084 |
| --- | --- |

| Title: | Non TEB3: Bi-directional optical flow for inter-layer texture prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexander Alshin Elena Alshina Zhan Ma JeongHoon Park 416, Maetan 3-Dong, Gyeonggi-do, Suwon, Korea | Tel: Email: | +82 10 3026 1305 { alexander_a.alshin, elena_a.alshina, zhan.ma, jeonghoon}@samsung.com |
| Source: | Samsung Electronics, Ltd. |  |  |

_____________________________

# Abstract

New method improving B-slice prediction on enhancement layer is proposed here. They key idea of proposed algorithm is combination of inter-layer texture prediction with optical flow concept and high accuracy gradients evaluation. The proposed technique allows pixel-wise refinement of motion in bi-predictive Pus on enhancement layer. This approach does not require any signaling of motion vector refinement to decoder.

# Introduction

Decoder side motion vector derivation (DMVD) techniques have a great potential in terms of video compression. If more than one reference is used for current block prediction it is possible to refine the motion within the prediction unit (PU). This motion refinement is estimated in same manner in encoder and decoder sides. More than 2 predictions are available in bi-predictive PUs. In case of scalable video codec one more prediction (from the base layer) is available. In this contribution we propose the motion refinement based on combination of 2 EL predictions and inter-layer texture prediction.

Proposed algorithm allows estimating of motion vector for each pixel. Initially this idea was proposed in JCTVC-C204 [1] and then studied in CE1 [2] which was devoted to DMVD methods. More details of described algorithm can be found in [3]. Since main idea came from optical flow concept [4] and was applied to B-slices this algorithm was called Bi-directional Optical Flow (BIO). If we can compensate this small pixel displacement without additional partitioning and additional motion vector coding it will allow using of larger sizes partitions. Thus, having pixel-wise compensation with no additional syntax, we may expect considerable increase in coding efficiency.

# Algorithm description.

Two blocks from two reference frames (past  and future), displaces relatively to currently predicted block according to proper motion vectors are our input. It means that integer pixel values from Reference0 () and Reference1 () should are processed by interpolation filter, providing blocks  and  of the same size with fractional position values (according to the fractional part of motion vector). After that, conventional bi-directional prediction P is given by averaging

Pconventional[i,j]=(P(0)[i,j]+ P(1)[ i,j]+ +offset)>>shiftNum ,                         (1)

where “>>” is bit-wise right shift.

The main idea of this method is to modify (1) by introducing pixel-wise motion vector refinement in order to obtain more accurate bi-directional prediction.

The proposed method essentially uses several assumptions.

- Let’s assume that objects move with locally constant speed. This means that so called “steady motion” model can be used (refer to Fig. 1).

Fig. 1. Steady motion assumption.

- Let’s assume that the luminance of moving objects does not change and well known optical flow PDE [1] is valid:

.                           (2)

Here I(x,y,t) is luminance, (x,y) are spatial coordinates, t is time, vx and vy are components pixel displacement vector.

- Let’s assume main motion compensation is already done by standard block based prediction and only fine motion less than one pixel is not compensated. For fine motion compensation only linear part of local Taylor expansion can be used.

Let us introduce theto be the difference between values in points A and B (Fig. 1):

Δi,j=P0i,j−P1i,j+vxi,jIx0i,j−Ix1i,j+vyi,jIy0i,j−Iy1i,j (3)

Here vx[i,j] and vy[i,j,] pixel-wise motion vector refinement components, and  are gradients of luminance I in the position [i,j] on 0 and 1 reference frame correspondently.

Of course the accuracy of gradient calculation is limited due to availability integer samples only. Let’s I[i,j] be the luminance value in position [i,j].  It is possible to estimate the gradient using only integer pixel values. For example using the concept described in [2] during DCT based interpolation filter construction we may construct DCT based filter directly providing gradient component

Ixli,j=m=−M+1MFmαxlRi+m,jl; Iyli,j=m=−M+1MFmαylRi+m,jl.                  (4)

Here  is block motion vector, R(l)i,j are reference picture values in integer position [i,j] for references l=0.1, Fmα is filter directly providing derivatives.

For different fractional position following 6 taps filter coefficients is used for gradient calculation

Frac =0:  {  8,      -39,       -3,       46,      -17,        5 },

Frac =1/4:{  4,      -17,      -36,       60,      -15,        4 },

Frac =1/2:{  -1,       4,      -57,       57,       -4,        1 },     (12)

Frac =3/4:{  -4,      15,      -60,       36,       17,       -4 }

In order to make motion vector estimation for point [i, j] more stable the assumption of locally steady motion inside some window  centered in [i, j] is involved. This window contains pixel in process and 5x5 neighbors. Optimal motion vector refinement can be estimated as solution of the least-squares problem. Let’s minimize the discrepancy (3) inside the window  with center in position [i,j]: i',j'∈Ω if i−L≤i'≤i+L,j−L≤j'≤j+L:

i',j∈ΩΔ2i',j=min..                                       (5)

Under assumptions listed above the problem (5) has explicit solution for the local pixel displacement vector :

(6)

where

Note that formula (6) can be substituted by approximate solution

vx = s3 /s1,  vy = (s6 - vx s2)/s5.

In spite of this formula is less accurate than formula (8) calculations can be done using 32 bit integer arithmetic without overflow. Finally bi-directional prediction based on BIO concept is

PBIO=P0+P1+vxIx0−Ix1+vyIy0−Iy1+offset>>shiftNum  (7)

Fig. 2 PBL- Inter-layer texture prediction (left), PBIOBIO prediction (right).

The Figure 2 shows comparison between inter-layer texture prediction which is too blurred and BIO prediction.

In suggested design the motion refinement is applied in bi-predictive PUs on enhancement layer and combined as weighted sum with inter-layer texture prediction:

(8)

Theoretically optimal weight is given by following formula

,        (9)

Where 1, 2 are corresponding variances for Inter-layer texture prediction (PBL) and BIO prediction (PBIO). However it’s fixed weight is preferable for implementation , so α is predetermined in suggested design.

# Tests results

As required in TEB3 description proposed algorithm was tested vs SMuC 0.1.1 reference s/w. Proposed algorithm works in random access test only. Random Access (x2, x1.5 and SNR) simulation results are summarised in Table 1 below. In average 2.5% (x2), 2.9% (x1.5) and 2.5% (SNR) Luma BD-rate gain is obtained.

It is known that majority of DMVD techniques has significant complexity in decoder side and this is also valid for BIO. However estimation of motion vectors for each pixel is independent and can be easily parallelized.

Table 1. Performance of combined prediction with pixel-wise motion refinement for RA scenario.

The most similar tool studied in TEB3 is test 4.6.2.2 which combines BL and EL prediction in a simple way:

p = ( pEL+pBL+1)>>1.

Performance of this test is summaries in Table 2. From comparison it is clear that pixel-wis motion refinement in combine prediction significantly improves performance of combined prediction.

Table 2. Performance of test TEB3_test4.6.2.2 for RA scenario.

# Conclusion

Proposed algorithm BIO shows noticeable gain for Random Access scenario

2.7% for Luma and 1.0% for Chroma

which is comparable with performance of generalized residual prediction tested in TEB3. In opposite to GRILP proposed algorithm doesn’t require storage of BL reference frames.

# Patent rights declaration(s)

Samsung Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).