| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0085 |
| --- | --- |

| Title: | TEB4: Inter-layer SAO (test 4.3.2). |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Contribution |  |  |
| Author(s) or Contact(s): | Elena Alshina Alexander Alshin JeongHoon Park 416, Maetan 3-Dong, Gyeonggi-do, Suwon, Korea | Tel: Email: | +82 10 3026 1305 {elena_a.alshina, alexander_a.alshin, jeonghoon}@samsung.com |
| Source: | Samsung Electronics, Ltd. |  |  |

_____________________________

# Abstract

In this contribution inter-layer SAO technique from Samsung’s CfP response was tested vs SMuC 0.1.1.  An average performance improvement from tested tool is 0.3% (Luma), 0.1% (Chroma) BD-rate reduction in spatial scalability tests (x2 and x1.5). In SNR scalability tests in average tested tool provides 0.4% Luma BD-rate gain, but 0.1% Chroma BD-rate drop. Encoding time doesn’t increase (100%), but there is increment of decoding time 9% in spatial scalability tests and 11% in SNR tests.

# Introduction

Sample Adaptive Offset (SAO) is one of the new tools entered video codec standard from HEVC [1]. In HEVC SAO is applied as in-loop filter which minimizes the difference between reconstructed (after de-blocking) and original signal.

In two CfP responses on scalable HEVC extensions (Samsung&Vidyo [2] and MediaTek&LG [3]) SAO was applied as inter-layer filter, which minimizes the difference between up-sampled (if needed) reconstructed Base Layer (BL) and original Enhancement Layer (EL). The motivation of this inter-layer filtering is correction of systematic errors might appear during BL compression and up-sampling interpolation.

Figure 1. Inter-layer SAO application.

The block diagram of inter-layer SAO application is shown on Figure 1.

Inter-layer SAO was designed targeting to re-use existing in HEVC SAO kernel for samples processing. So both sample processing module and memory are shared between in-loop SAO and inter-layer SAO.

In the variant tested in TEB4_test 4.3.2 SAO parameters are encoded in slice header. Both 5 SAO types 4 directions of Edge Offset (EO) and Band Offset (BO) are allowed. Offsets values are in a range -31...31. Inter-layer SAO is applied for Luma component only in tested design.

# Test results summary.

As required in TEB4 description proposed algorithm was tested vs SMuC 0.1.1 in All Intra, Random Access and Low-Delay P tests for both spatial (x2 and x1.5) and SNR scalability scenario. Simulation results are summarised in Table 1 below.

Table 1. Inter-layer SAO (TEB4 test 4.3.2) performance.

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | -0.9% | -1.2% | -1.3% |  |  |  |  |  |  |
| Class B | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |  |  |  |
| Overall (EL+BL) | -0.3% | -0.4% | -0.4% | 0.0% | -0.1% | -0.1% |  |  |  |
| Overall (EL) | -0.6% | -0.8% | -0.9% | -0.2% | -0.2% | -0.2% |  |  |  |
| Enc Time[%] | 100.1% |  |  | 100.3% |  |  |  |  |  |
| Dec Time[%] | 108.4% |  |  | 107.5% |  |  |  |  |  |
| Enc Mem[%] | 102.2% |  |  | 101.1% |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.4% | -0.2% | -0.4% |  |  |  | -0.7% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall (EL+BL) | -0.4% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% |
| Overall (EL) | -0.9% | -0.1% | -0.2% | 0.0% | 0.0% | 0.0% | -0.6% | 0.0% | 0.0% |
| Enc Time[%] | 100.3% |  |  | 100.1% |  |  | 100.1% |  |  |
| Dec Time[%] | 109.8% |  |  | 107.2% |  |  | 110.8% |  |  |
| Enc Mem[%] | 100.6% |  |  | 100.5% |  |  | 112.5% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.5% | -0.2% | -0.3% |  |  |  | -1.0% | 0.1% | 0.1% |
| Class B | -0.2% | -0.1% | 0.1% | -0.2% | 0.1% | 0.1% | -0.3% | 0.1% | 0.1% |
| Overall (EL+BL) | -0.6% | -0.1% | 0.0% | -0.2% | 0.1% | 0.1% | -0.5% | 0.1% | 0.1% |
| Overall (EL) | -1.3% | -0.2% | 0.0% | -0.6% | 0.3% | 0.4% | -1.1% | 0.1% | 0.2% |
| Enc Time[%] | 100.2% |  |  | 100.1% |  |  | 100.1% |  |  |
| Dec Time[%] | 111.1% |  |  | 107.5% |  |  | 112.0% |  |  |
| Enc Mem[%] | 100.7% |  |  | 100.7% |  |  | 112.7% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | -0.3% | -0.1% |  |  |  |  | -0.4% | 0.1% |  |
|  | 100% |  |  |  |  |  | 100% |  |  |
|  | 109% |  |  |  |  |  | 111% |  |  |

The best performance was observed for highest resolution video PeopleOnStreet in RA, x2 test (-3.0% BD-rate reduction).

Authors would like to thank Qualcomm for verification of these test results in TEB4.

# Conclusion

The performance of inter-layer SAO with slice based parameters selection was verified in this test. Tested algorithm provides an average performance improvement 0.3% (Luma), 0.1% (Chroma) in terms of BD-rate reduction in spatial scalability tests (x2 and x1.5). In SNR scalability tests in average tested tool provides 0.4% Luma BD-rate gain, but 0.1% Chroma BD-rate drop. Encoding time doesn’t increase (100%), but there is increment of decoding time 9% in spatial scalability tests and 11% in SNR tests. In tested design the sample processing module and memory is shared with HEVC in-loop SAO.

# References

[1] Chih-Ming Fu, Elena Alshina, Alexander Alshin, Yu-Wen Huang, Ching-Yeh Chen, Chia-Yang Tsai,, Chih-Wei Hsu, Shaw-Min Lei, JeongHoon Park, and Woo-Jin Han, Sample Adaptive Offset in the HEVC Standard; IEEE Transactions on Circuits and Systems for Video Technology , Volume 99.

[2] K. McCann, J. H. Park, J. Kim, C. Kim, J.-H. Min, E. Alshina, A. Alshin, I.-K. Kim, T. Lee, B. Choi, Y. Piao, S. Jeong, S. Lee, Y Cho, J. Y. Choi, F. C. A. Fernandes, Z. Ma (Samsung), J. Boyce, D. Hong, W. Jang, A. Abbas, S. Reddy (Vidyo) Description of high efficiency scalable video coding technology proposal by Samsung and Vidyo, JCTVC-K0044, October, 2012

[3] C. Kim, J. Park, J. Kim, Hendry, S. Park, Y. Jeon, J. Lim, N. Park, B. Jeon (LG Electronics), S. Liu, X. Zhang, M. Guo, Z. Chen, T.-D. Chuang, S.-T. Hsiang, C.-Y. Chen, C.-Y. Tsai, C.-M. Fu, C.W. Hsu, Y.-W. Huang, S. Lei (MediaTek), Description of scalable video coding technology proposal by LG Electronics and MediaTek (differential coding mode on), JCTVC-K0033, October, 2012

# Patent rights declaration(s)

Samsung Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).