| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0086 |
| --- | --- |

| Title: | non-TEB4: Band Offset correction for inter-layer texture prediction. |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Elena Alshina Alexander Alshin JeongHoon Park 416, Maetan 3-Dong, Gyeonggi-do, Suwon, Korea | Tel: Email: | +82 10 3026 1305 {elena_a.alshina, alexander_a.alshin, jeonghoon}@samsung.com |
| Source: | Samsung Electronics, Ltd. |  |  |

_____________________________

# Abstract

In this contribution inter-layer SAO technique from Samsung’s CfP response was simplified. Namely only Band Offset correction is allowed during inter-layer filtering. This modification reduces decoding run-time compare to anchor from 109% (spatial scalability) and 111% (SNR scalability) to 101% but preserves the most part of the gain. An average performance improvement from tested tool is 0.14% (Luma), 0.10% (Chroma) BD-rate reduction in spatial scalability tests (x2 and x1.5). In SNR scalability tests in average tested tool provides 0.12% Luma BD-rate gain.

# Introduction

Sample Adaptive Offset (SAO) is one of the new tools entered video codec standard from HEVC [1]. In HEVC SAO is applied as in-loop filter which minimizes the difference between reconstructed (after de-blocking) and original signal.

In two CfP responses on scalable HEVC extensions (Samsung&Vidyo and MediaTek&LG) SAO was applied as inter-layer filter, which minimizes the difference between up-sampled (if needed) reconstructed Base Layer (BL) and original Enhancement Layer (EL). The motivation of this inter-layer filtering is correction of systematic errors might appear during BL compression and up-sampling interpolation.

Figure 1. Inter-layer SAO application.

The block diagram of inter-layer SAO application is shown on Figure 1.

Inter-layer SAO was designed targeting to re-use existing in HEVC SAO kernel for samples processing.

In the variant tested in TEB4_test 4.3.2 [2] SAO parameters are encoded in slice header. Both 5 SAO types: 4 directions of Edge Offset (EO) and Band Offset (BO), are allowed. Inter-layer SAO applied for Luma component only in our TEB4 test. Offsets values are in a range -31...31.

For this contribution we have restricted inter-layer SAO application and use Band Offset Correction only. All 32 bands are allowed to be corrected. Offset values are in a range -15...15. Band Offset parameter are encoded in EL Slice Header.

So both sample processing module and memory are shared between in-loop SAO on EL and inter-layer SAO.

Band Offset correction is applied only for Luma inter-layer texture prediction in SMuC 0.1.1.

# Test results summary.

Following TEB4 requirements proposed algorithm was tested vs SMuC 0.1.1 in All Intra, Random Access and Low-Delay P tests for both spatial (x2 and x1.5) and SNR scalability scenario. Simulation results are summarised in Table 1 below.

Table 1. Inter-layer BO performance.

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |  |
| Class A | -1.0% | -1.3% | -1.3% |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |  |  |  |  |
| Overall (EL+BL) | -0.3% | -0.4% | -0.4% | 0.0% | -0.1% | -0.1% |  |  |  |  |
| Overall (EL) | -0.7% | -0.9% | -0.9% | -0.1% | -0.2% | -0.2% |  |  |  |  |
| Enc Time[%] | 101.1% |  |  | 100.8% |  |  |  |  |  |  |
| Dec Time[%] | 103.4% |  |  | 101.9% |  |  |  |  |  |  |
| Enc Mem[%] | 101.9% |  |  | 103.3% |  |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |  |
|  | Y | U | V | Y | U | V | Y | U |  | V |
| Class A | -1.0% | -0.3% | -0.3% |  |  |  | -0.3% | -0.1% |  | -0.1% |
| Class B | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  | 0.0% |
| Overall (EL+BL) | -0.2% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |  | 0.0% |
| Overall (EL) | -0.6% | -0.1% | -0.1% | 0.0% | 0.1% | 0.1% | -0.3% | -0.1% |  | 0.0% |
| Enc Time[%] | 100.7% |  |  | 100.1% |  |  | 100.4% |  |  |  |
| Dec Time[%] | 99.8% |  |  | 100.3% |  |  | 101.2% |  |  |  |
| Enc Mem[%] | 103.1% |  |  | 104.2% |  |  | 112.0% |  |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |  |
|  | Y | U | V | Y | U | V | Y | U |  | V |
| Class A | -1.0% | -0.2% | -0.2% |  |  |  | -0.3% | -0.2% |  | -0.2% |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% | 0.0% | -0.1% | 0.1% |  | 0.1% |
| Overall (EL+BL) | -0.3% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | 0.0% |  | 0.0% |
| Overall (EL) | -0.6% | -0.1% | -0.1% | 0.0% | -0.2% | -0.1% | -0.3% | -0.1% |  | 0.0% |
| Enc Time[%] | 100.7% |  |  | 100.0% |  |  | 100.5% |  |  |  |
| Dec Time[%] | 100.3% |  |  | 99.8% |  |  | 101.4% |  |  |  |
| Enc Mem[%] | 103.1% |  |  | 104.1% |  |  | 112.2% |  |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |  |
|  | -0.14% | -0.10% |  |  |  |  | -0.12% |  | 0.00% |  |
|  | 101% |  |  |  |  |  | 100% |  |  |  |
|  | 101% |  |  |  |  |  | 101% |  |  |  |

The best performance again was observed for highest resolution video PeopleOnStreet in AI, x2 test (-2.30% BD-rate reduction). This confirms the efficiency of Band Offset Correction for noise reduction.

Authors would like to thank Qualcomm for verification of these test results.

Compare to TEB4 inter-layer SAO this simplified version provides better trade-off between complexity and performance improvement.

Table 2. Comparison between inter-layer SAO and inter-layer BO.

|  | TEB4 test 432 |  | BO correction for IntraBL |  |
| --- | --- | --- | --- | --- |
|  | ×2, ×1.5 | SNR | ×2, ×1.5 | SNR |
| Avg. Luma BD-rate | -0.25% | -0.38% | -0.14% | -0.12% |
| Avg. Chroma BD-rate | -0.08% | 0.05% | -0.10% | 0.00% |
| Avg. Enc. Time | 100% | 100% | 101% | 100% |
| Avg. Dec. Time | 109% | 111% | 101% | 101% |

# Conclusion

The performance of Band Offset Correction applied to inter-layer texture prediction with slice based parameters selection was tested in this contribution. Tested algorithm provides an average performance improvement 0.14% (Luma), 0.10% (Chroma) in terms of BD-rate reduction in spatial scalability tests (x2 and x1.5). In SNR scalability tests in average tested tool provides 0.12% Luma BD-rate gain w/o .1% Chroma BD-rate drop. Encoding time doesn’t increase (100%), there is only 1% increment of decoding time. The sample processing module and memory for proposed tool are shared with HEVC in-loop SAO.

# References

[1] Chih-Ming Fu, Elena Alshina, Alexander Alshin, Yu-Wen Huang, Ching-Yeh Chen, Chia-Yang Tsai,, Chih-Wei Hsu, Shaw-Min Lei, JeongHoon Park, and Woo-Jin Han, Sample Adaptive Offset in the HEVC Standard; IEEE Transactions on Circuits and Systems for Video Technology , Volume 99.

[2] Elena Alshina, Alexander Alshin, JeongHoon Park, SHVC TEB4: Inter-layer SAO (test 4.3.2), JCTVC-L0085, January, 2013.

# Patent rights declaration(s)

Samsung Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).