| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0087 |
| --- | --- |

| Title: | On integer samples filtering during up-sampling |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Elena Alshina Alexander Alshin JeongHoon Park 416, Maetan 3-Dong, Gyeonggi-do, Suwon, Korea | Tel: Email: | +82 10 3026 1305 {elena_a.alshina, alexander_a.alshin, jeonghoon}@samsung.com |
| Source: | Samsung Electronics, Ltd. |  |  |

_____________________________

# Abstract

In this contribution de-noising filter for SNR scalability proposed and tested vs SMuC 0.1.1. De-noising filter is applied for Luma only. An average performance improvement from de-noising of reconstructed base layer signal is 1.6% (Luma) and 0.5% (Chroma) BD-rate reduction. Encoding time doesn’t increase (100%), but there is increment of decoding time ~22% (similar to adaptive up-sampling filter techniques in TEB4). Additionally in up-sampling filter function of reference s/w SMuC 0.1.1 redundant filtration of integer samples was removed. This losses change leads to ~5% decoding time reduction.

# Introduction

In case of spatial scalability SHVC tests original signal is captured in high resolution and then down-sampled applying filters described in [1]. In particular both in case of x2 and x1.5 scalability integer pel positions were filtered during down-sampling using following 12-taps filters:

| x1.5 | [ 0 | 5 | -6 | -10 | 37 | 76 | 37 | -10 | -6 | 5 | 0 | 0] /128 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x.2 | [2 | -3 | -9 | 6 | 39 | 58 | 39 | 6 | -9 | -3 | 2 | 0] /128 |

Frequency responses in amplitude domain of these filters are shown on Figure 1. It is easy to notice that both they have strong de-noising effect: x1.5 filter cuts about half of high frequency harmonics, x2 filter cuts almost 2/3 of high frequency spectrum.

The effect of this integer samples filtration during down-sampling can be considered as pre-processing of original base layer (BL) signal which removes the noise. Up-sampling filter in case of spatial scalability also works as noise-killer.

There is no such kind of pre-processing in case of SNR scalability: Original BL and Original EL signals are identical. The difference of coding schemes in spatial and SNR scalability process is summarized on Figures 2-3.

Figure 1. Frequency response in amplitude domain of down-sampling filters (x2 and x1.5 scalability) and proposed de-noising filter.

| Figure 2. The scheme of spatial scalability process | Figure 3. The scheme of SNR scalability process |
| --- | --- |

De-noising filter for SNR scalability. In order to equalize de-noising effect of down-sampling and up-sampling procedures for spatial and SNR scalability we have designed de-noising filter for SNR scalability test. Suggested filter is 2D separable FIR with coefficients:

{ 1,	-5,	11,	50,	11,	-5,	1,	0}/64.

In case of SNR test it is applied to int-pel Luma samples (Fig. 4).

Figure 4. Suggested modification of SNR scalability process scheme.

This int-pel filter was implemented in SMuC 0.1.1.

Removal of int-pel filtration in spatial scalability. In addition int-pel filtering with trivial filter’s coefficients

{  0,  0,  0, 64,  0,  0,  0,  0}/64

in spatial scalability test, which is equivalent to no filtering, was removed from the code. Table 1 below summarize the amount of filter application actually needed for up-sampling and in SMuC 0.1.1 s/w.

Table 1. Up-sampling filter process application summary.

| Up-sampling filter application for x2 scalability |  | Up-sampling filter application or x1.5 scalability |  |
| --- | --- | --- | --- |
| Actually needed | SMuC 0.1.1 | Actually needed | SMuC 0.1.1 |

In case of ×2 spatial scalability the half of up-sampling filter application is redundant: 8  4, in case of ×1.5 spatial scalability the one third of up-sampling filter operations is not needed: 18  12.

We would suggest modifying reference s/w further and re-using generic MC framework for up-sampling. This will reduce the code size and speed-up the s/w.

# Test results summary.

Suggested below modification of int-pel filtration in SMuC 0.1.1 s/w is loss-less in case of spatial scalability. Removal of redundant int-pel filtration leads to decoding time reduction 5%. In case of SNR scalability de-noising filter applied to Luma component provides 1.6% average Luma BD-rate reduction. Since in SNR case int-pel filtration is added and additional temporal buffer is needed (in SMuC reconstructed BL and inter-layer texture prediction share the same memory in SNR case) there is decoding time increment ~22%. Very similar decoding time increment in SNR tests is observed in TEB4 tests for adaptive up-sampling filtering.

Table 1. Int-pel filtration modified in SMuC 0.1.1.

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall (EL+BL) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall (EL) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 99.8% |  |  | 99.6% |  |  |  |  |  |
| Dec Time[%] | 97.3% |  |  | 97.0% |  |  |  |  |  |
| Enc Mem[%] | 102.2% |  |  | 102.5% |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |  |  |  | -3.5% | -0.6% | -0.5% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.3% | -0.2% |
| Overall (EL+BL) | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | -1.1% | -0.4% | -0.3% |
| Overall (EL) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -2.1% | -0.3% | -0.1% |
| Enc Time[%] | 99.9% |  |  | 99.3% |  |  | 99.1% |  |  |
| Dec Time[%] | 92.9% |  |  | 93.6% |  |  | 123.4% |  |  |
| Enc Mem[%] | 103.1% |  |  | 104.2% |  |  | 112.0% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |  |  |  | -4.6% | -0.7% | -0.5% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -1.0% | -0.5% | -0.3% |
| Overall (EL+BL) | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | -2.0% | -0.6% | -0.4% |
| Overall (EL) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -3.7% | -0.7% | -0.4% |
| Enc Time[%] | 99.8% |  |  | 99.3% |  |  | 99.3% |  |  |
| Dec Time[%] | 93.4% |  |  | 94.6% |  |  | 123.0% |  |  |
| Enc Mem[%] | 103.1% |  |  | 104.2% |  |  | 112.2% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | 0.0% | 0.0% | 0.0% |  |  |  | -1.6% | -0.5% | -0.3% |
|  | 100% |  |  |  |  |  | 99% |  |  |
|  | 95% |  |  |  |  |  | 123% |  |  |

The best performance was observed for highest resolution video PeopleOnStreet in LDP, SNR test (-7.3% BD-rate reduction).

Authors would like to thank InterDigital for verification of these test results.

# Conclusion

Integer samples filtration is not needed for spatial scalability. Loss-less removal of these redundant operations leads to at least 5% decoding time reduction. Further code speed-up is possible is generic motion compensation interpolation framework will be re-used for up-sampling module of SMuC. De-noising filter for SNR scalability with fixed coefficients suggested in this contribution. It allows achieving 1.6% BD-rate reduction without any content adaptivity.

# References

[1] Jie Dong, Yong He, Yan Ye, Downsampling filters for anchor generation for scalable extensions of HEVC, MPEG2012/M24499, May, 2012.

# Patent rights declaration(s)

Samsung Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).