| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0090 |
| --- | --- |

| Title: | TEA2: Cross-check of refIdx performance (tests 3.2.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Elena Alshina Alexander Alshin JeongHoon Park 416, Maetan 3-Dong, Gyeonggi-do, Suwon, Korea | Tel: Email: | +82 10 3026 1305 {elena_a.alshina, alexander_a.alshin, jeonghoon}@samsung.com |
| Source: | Samsung Electronics, Ltd. |  |  |

_____________________________

# Abstract

This is verification of TEA2 tests 3.2.1 from InterDigital. Alternative to IntraBL approach is tested here. Up-sampled (if needed) base layer picture is added into the reference list and so used as prediction on enhancement layer. Among 5 variants of algorithm presented by Inter Digital in TEA2 so-called “settings 2”, “settings 2 + zero MVD” and “setting 2 + zero MV + uni-prediction” are verified in this report. An average Luma BD-rate gain of different variations tested compare to HM8.1 simulcast is from 27.8% to 28.0%, Chroma BD-rate gain is from 27.4% to 27.7% in case of all intra tests. For MC test scenario average Luma BD-rate gain of different variations tested compare to HM8.1 simulcast is from 19.1% to 19.3%, Chroma BD-rate gain is from 10.9% to 12.4%

# Introduction

As result of JCTVC-K meeting BoG on scalable HEVC extensions only one tool which is inter-layer texture prediction was included into the reference s/w. By default inter-layer texture prediction is enabled as special mode for enhancement layer CU encoding. This is analogue of IntraBL from previous standard.

Among CfP responses was an alternative implementation of inter-layer texture prediction. This is so-called “redIdx” framework. Up-sampled (if needed) reconstructed base layers picture is included into the reference list and so in motion compensation prediction on enhancement layer following cases are:

- Collocated base layer block (up-sampled if needed) is used as predictor (uni-prediction with MV restricted to 0)

- Interpolated base layer block (after up-sampling in case of spatial scalability) is used as predictor (uni-prediction with non-zero)

- The combination of 2 different predictions:

  - Both from BL

  - Both from EL

  - One form EL, but another from BL

is used as prediction.

So “refIdx” framework is an elegant implementation which includes not only IntraBL but combined prediction as well.

In order to enable inter-layer texture prediction in case of “all intra” test slice type is changed from I to P-type.

“RefIDx” framework was implemented in reference s/w SMuC 0.1.1, but disable by default. In this “ref_idx” framework the inter-layer reference picture is added to the end of list L0 if the enhancement-layer picture is coded as P-Slice, or is added to the end of both lists L0 and L1 if the enhancement-layer picture is coded as B-Slice. This is so-called “settings 1” of Inter Digital’s test.

In the “settings 2, the inter-layer reference picture is placed at the same position as the first setting for AI and LD-P coding structure. But, for RA case, the inter-layer reference picture is added into both lists L0 and L1 of the enhancement-layer pictures that are located in the first temporal level, but is only added into the list L0 of the enhancement-layer pictures that are located in the higher temporal levels.

This test is marked as “setting 2” below.

Additionally proponent provided an option to restrict of MV to 0 in “refIdx” framework. With this option reconstructed base layer signal will be only up-sampled (if needed) but no additional interpolation due to non-zero MV will be needed. The test for this option is marked as “setting 2 + zero MV” below.

One more option provided by proponent is allowing only uni-prediction in “refIdx” framework. In this case no combination prediction from base layer with prediction from enhancement layer is possible. Test results for both 3 options together are marked “setting 2 + zero MV + uni-prediction” below.

# Test results for verified algorithm.

As required in TEA2 description verified algorithm was tested vs HM8.1 simulcast in spatial (x2 and x1.5) scalability tests. SNR test was optional but was presented by proponent and verified here.  Simulation results are summarised in Tables 1-3.

Table 1. Summation test results for “setting 2” (vs HM8.1 simulcast).

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | -26.9% | -26.2% | -27.1% |  |  |  |  |  |  |
| Class B | -21.2% | -20.1% | -20.0% | -32.8% | -32.9% | -33.1% |  |  |  |
| Overall (EL+BL) | -22.8% | -21.9% | -22.0% | -32.8% | -32.9% | -33.1% |  |  |  |
| Overall (EL) | -34.8% | -33.7% | -33.9% | -58.5% | -58.2% | -58.7% |  |  |  |
| Enc Time[%] | 241.4% |  |  | 190.0% |  |  |  |  |  |
| Dec Time[%] | 91.1% |  |  | 87.4% |  |  |  |  |  |
| Enc Mem[%] | 126.8% |  |  | 123.1% |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -18.3% | -6.7% | -8.1% |  |  |  | -21.4% | -10.8% | -10.1% |
| Class B | -15.5% | -7.4% | -5.8% | -26.4% | -20.2% | -18.5% | -19.5% | -9.5% | -5.5% |
| Overall (EL+BL) | -16.3% | -7.2% | -6.4% | -26.4% | -20.2% | -18.5% | -20.1% | -9.9% | -6.8% |
| Overall (EL) | -25.2% | -11.3% | -10.3% | -47.7% | -38.2% | -35.3% | -31.7% | -16.9% | -12.4% |
| Enc Time[%] | 112.5% |  |  | 110.0% |  |  | 108.4% |  |  |
| Dec Time[%] | 119.2% |  |  | 115.8% |  |  | 87.3% |  |  |
| Enc Mem[%] | 116.0% |  |  | 113.4% |  |  | 102.2% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -13.3% | -1.1% | -2.3% |  |  |  | -13.7% | -6.4% | -5.4% |
| Class B | -11.6% | -5.6% | -3.9% | -22.1% | -16.9% | -15.2% | -12.7% | -5.7% | -1.8% |
| Overall (EL+BL) | -12.1% | -4.3% | -3.5% | -22.1% | -16.9% | -15.2% | -13.0% | -5.9% | -2.9% |
| Overall (EL) | -19.4% | -6.8% | -5.7% | -41.2% | -32.1% | -29.1% | -20.9% | -11.2% | -6.9% |
| Enc Time[%] | 105.3% |  |  | 103.9% |  |  | 103.7% |  |  |
| Dec Time[%] | 120.6% |  |  | 117.7% |  |  | 88.4% |  |  |
| Enc Mem[%] | 117.7% |  |  | 115.3% |  |  | 103.8% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

Table 2. Summation test results for “setting 2 + zero MV” (vs HM8.1 simulcast).

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | -27.1% | -26.5% | -27.3% |  |  |  |  |  |  |
| Class B | -21.3% | -20.4% | -20.2% | -32.9% | -33.0% | -33.2% |  |  |  |
| Overall (EL+BL) | -23.0% | -22.1% | -22.2% | -32.9% | -33.0% | -33.2% |  |  |  |
| Overall (EL) | -35.1% | -34.0% | -34.3% | -58.8% | -58.5% | -58.9% |  |  |  |
| Enc Time[%] | 219.2% |  |  | 172.1% |  |  |  |  |  |
| Dec Time[%] | 90.4% |  |  | 86.6% |  |  |  |  |  |
| Enc Mem[%] | 127.0% |  |  | 123.1% |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -18.5% | -7.2% | -8.6% |  |  |  | -21.5% | -11.3% | -10.6% |
| Class B | -15.7% | -7.9% | -6.2% | -26.5% | -20.5% | -18.8% | -19.7% | -9.9% | -6.0% |
| Overall (EL+BL) | -16.5% | -7.7% | -6.9% | -26.5% | -20.5% | -18.8% | -20.2% | -10.3% | -7.3% |
| Overall (EL) | -25.4% | -11.9% | -11.0% | -48.0% | -38.6% | -35.8% | -31.9% | -17.5% | -13.2% |
| Enc Time[%] | 101.1% |  |  | 100.5% |  |  | 101.0% |  |  |
| Dec Time[%] | 117.6% |  |  | 116.1% |  |  | 87.8% |  |  |
| Enc Mem[%] | 116.0% |  |  | 113.4% |  |  | 102.2% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -13.3% | -1.4% | -2.6% |  |  |  | -13.7% | -6.8% | -5.9% |
| Class B | -11.7% | -5.8% | -4.1% | -22.2% | -16.9% | -15.3% | -12.8% | -6.0% | -2.3% |
| Overall (EL+BL) | -12.2% | -4.5% | -3.7% | -22.2% | -16.9% | -15.3% | -13.0% | -6.2% | -3.3% |
| Overall (EL) | -19.5% | -7.1% | -6.0% | -41.4% | -32.2% | -29.2% | -21.0% | -11.6% | -7.6% |
| Enc Time[%] | 99.5% |  |  | 99.0% |  |  | 99.8% |  |  |
| Dec Time[%] | 120.6% |  |  | 117.7% |  |  | 87.3% |  |  |
| Enc Mem[%] | 117.7% |  |  | 115.2% |  |  | 103.8% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

Table 3. Summation test results for “setting 2 + zero MV + uni-prediction” (vs HM8.1 simulcast).

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | -27.1% | -26.5% | -27.3% |  |  |  |  |  |  |
| Class B | -21.3% | -20.4% | -20.2% | -32.9% | -33.0% | -33.2% |  |  |  |
| Overall (EL+BL) | -23.0% | -22.1% | -22.2% | -32.9% | -33.0% | -33.2% |  |  |  |
| Overall (EL) | -35.1% | -34.0% | -34.3% | -58.8% | -58.5% | -58.9% |  |  |  |
| Enc Time[%] | 217.8% |  |  | 170.0% |  |  |  |  |  |
| Dec Time[%] | 93.2% |  |  | 89.3% |  |  |  |  |  |
| Enc Mem[%] | 126.8% |  |  | 123.0% |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -18.2% | -6.9% | -8.4% |  |  |  | -21.1% | -11.0% | -10.5% |
| Class B | -15.3% | -7.8% | -6.3% | -26.1% | -20.2% | -18.7% | -19.1% | -10.4% | -7.0% |
| Overall (EL+BL) | -16.1% | -7.5% | -6.9% | -26.1% | -20.2% | -18.7% | -19.7% | -10.5% | -8.0% |
| Overall (EL) | -25.0% | -11.6% | -10.9% | -47.4% | -38.2% | -35.5% | -31.2% | -17.8% | -14.1% |
| Enc Time[%] | 98.5% |  |  | 98.6% |  |  | 99.5% |  |  |
| Dec Time[%] | 121.2% |  |  | 121.2% |  |  | 93.6% |  |  |
| Enc Mem[%] | 115.9% |  |  | 113.3% |  |  | 102.2% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -13.3% | -1.4% | -2.6% |  |  |  | -13.7% | -6.8% | -5.9% |
| Class B | -11.7% | -5.8% | -4.1% | -22.2% | -16.9% | -15.3% | -12.8% | -6.0% | -2.3% |
| Overall (EL+BL) | -12.2% | -4.5% | -3.7% | -22.2% | -16.9% | -15.3% | -13.0% | -6.2% | -3.3% |
| Overall (EL) | -19.5% | -7.1% | -6.0% | -41.4% | -32.2% | -29.2% | -21.0% | -11.6% | -7.6% |
| Enc Time[%] | 98.4% |  |  | 98.3% |  |  | 98.6% |  |  |
| Dec Time[%] | 124.6% |  |  | 123.6% |  |  | 95.3% |  |  |
| Enc Mem[%] | 117.7% |  |  | 115.2% |  |  | 103.8% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

Both HM8.1 simulcast and verified test were simulated using the same homogeneous cluster, so time measurement is more or less accurate in our tests.

In addition Samsung was one of the cross-checkers of SMuC 0.1.1 performance, so we can compare default SMuC 0.1.1 settings (which is IntraBL) performance /run time with “refIdx” approach. This comparison is shown in Table 4 (for all intra tests) and Table 5 (RA and LDP spatial scalability tests).

Table 4. Comparison between IntraBL in SMuC 0.1.1 and verified algorithms in All Intra tests.

| Test | Avg BD-rate |  |  | Avg. compare to HM8.1 simulcast |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Enc. Time | Dec. Time | Enc. Mem. |
| SMuC 0.1.1 | -27.7% | -28.0% | -28.0% | 114% | 107% | 110% |
| “settings 2” | -27.8% | -27.4% | -27.5% | 216% | 89% | 125% |
| “setting 2 + zero MV” | -28.0% | -27.6% | -27.7% | 196% | 88% | 125% |
| “setting 2 + zero MV + uni-prediction” | -28.0% | -27.6% | -27.7% | 194% | 91% | 125% |

It is easy to notice that “refIdx” framework performs slightly better (0.1…0.3%) in terms of Luma BD-rate, but there is a same drop in terms of Chroma component compare to IntraBL in SMuC 0.1.1.

Important positive effect is noticeably less decoding time “refIdx” framework has (88...91%) compare to IntraBL (107%) in SMuC 0.1.1. This effect is common for all contribution is TEA2 which utilise “inter residual coding” for IntraBL residual.

Restrictions of MV and bi-prediction in “refIdx” framework make encoder 20...22%  faster w/o any harm to the performance (even with additional 0.2% Luma BD-rate gain).

There is additional ~15% memory usage in current “refIdx” implementation compare to SMuC 0.1.1.

Table 5. Comparison between IntraBL in SMuC 0.1.1 and verified algorithms in RA&LDP tests.

| Test | Avg BD-rate |  |  | Avg. compare to HM8.1 simulcast |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Enc. Time | Dec. Time | Enc. Mem. |
| SMuC 0.1.1 | -19.2% | -12.9% | -11.7% | 110% | 135% | 111% |
| “settings 2” | -19.2% | -12.2% | -10.9% | 108% | 118% | 116% |
| “setting 2 + zero MV” | -19.3% | -12.4% | -11.2% | 100% | 118% | 116% |
| “setting 2 + zero MV + uni-prediction” | -19.1% | -12.3% | -11.1% | 98% | 123% | 116% |

In MC coding scenario “refIdx” framework performs more or less equally with IntraBL  (-0.1…0.1%) in terms of Luma BD-rate, but there is consistent drop (0.6%...0.8%) in terms of Chroma component compare to IntraBL in SMuC 0.1.1.

Again decoder is faster when “inter residual coding” is used for inter-layer prediction residual.

Encoder run-time is comparable (w/o MV and bi-pred restriction) or 10...12% less (with restriction) compare to SMuC 0.1.1.

Additional memory usage is ~5% compare to SMuC 0.1.1.

It should be mentioned that SMuC 0.1.1 has DCT/DST transform switch related a bug in residual coding for inter-layer texture prediction. Performance effect of this bug correction is ~0.2% in terms of Luma BD-rate. This bug doesn’t effect refIdx” framework performance since “inter residual coding” is used.

# Conclusions

The performance of an elegant implementation of inter-layer texture prediction is verified this report. Among 5 variants of “refIdx framework” presented by Inter Digital in TEA2 so-called “settings 2”, “settings 2 + zero MVD” and “setting 2 + zero MV + uni-prediction” are verified in this report. An average Luma BD-rate gain of different variations tested compare to HM8.1 simulcast is from 27.8% to 28.0%, Chroma BD-rate gain is from 27.4% to 27.7% in case of all intra tests. For MC test scenario average Luma BD-rate gain of different variations tested compare to HM8.1 simulcast is from 19.1% to 19.3%, Chroma BD-rate gain is from 10.9% to 12.4%. Allowing of uni-prediction only with MV restricted to 0 seems reasonable for “refIdx framework”: there is almost no performance degradation but up to 22% encoder run-time reduction due to these restrictions.