| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0091 |
| --- | --- |

| Title: | TEB4: Cross-check of IMDDF performance (test 4.1.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Elena Alshina Alexander Alshin JeongHoon Park 416, Maetan 3-Dong, Gyeonggi-do, Suwon, Korea | Tel: Email: | +82 10 3026 1305 {elena_a.alshina, alexander_a.alshin, jeonghoon}@samsung.com |
| Source: | Samsung Electronics, Ltd. |  |  |

_____________________________

# Abstract

This is verification of TEB4 test 4.1.2 from Inter Digital. In this test fixed set of up-sampling filters switched depending on Intra Prediction Direction on base layer was tested vs fixed SMuC 0.1.1 filter. Average BD-rate improvement 0.1% (in all intra x2) and 0.2% (in all intra x1.5) is confirmed in this cross-check report.

# Introduction

Intra mode dependent switchable up-sampling filter was part of Inter Digital’s CfP response (JCTVC-K0034). In this TEB4 test performance of this tool was studied. S/w was released by proponent before dead-line.

After careful s/w analysis we have found following details of tested tool:

- Up-sampling procedure is implemented as 2 stage process:

  - For I-slices:

    - SMuC 0.1.1 up-sampling for Chroma is performed first

    - Interpolation of base layer with mode-dependent filters is applied for Luma next

  - For B and P-slices:

    - SMuC 0.1.1 up-sampling for both Luma and Chroma is performed first

    - Interpolation of base layer with mode-dependent filters is applied for Luma next

As a result in B and P slices base layer Luma signal is filtered twice.

- Mode dependent filtration is applied for Luma signal only.

- Filter bank consist of 4×34= 136 2D non-separable filters with sizes from 7×7 to 8×8. Due to the limited time we don’t provide spectral characteristics analysis for all these filters.

- Filter coefficients are scaled by 256 (8 bits precision).

- Only 1:2 and 2:3 spatial scalability ratio is supported.

- There is slice –level flag to enable/disable this mode-dependent up-sampling. Decision about use or not to use mode-dependent up-sampling is made on encoder side by comparison of distortion between up-sampled with fixed or mode-dependent filters set and original EL signal.

# Test results for verified algorithm.

As required in TEB4 description proposed algorithm was tested vs fixed up-sampling filter is reference s/w. All Intra (x2 and x1.5), Random Access (x2, x1.5 and SNR) and Low-Delay P (x2 , x1.5 and SNR) tests were performed. Simulation results are summarised in Table 1 below

Table 1. Summation test results for verified algorithm.

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |
| Class B | -0.2% | -0.2% | -0.1% | -0.2% | -0.3% | -0.2% |  |  |  |
| Overall (EL+BL) | -0.1% | -0.1% | -0.1% | -0.2% | -0.3% | -0.2% |  |  |  |
| Overall (EL) | -0.2% | -0.2% | -0.1% | -0.6% | -0.7% | -0.7% |  |  |  |
| Enc Time[%] | 102.1% |  |  | 102.3% |  |  |  |  |  |
| Dec Time[%] | 102.3% |  |  | 104.6% |  |  |  |  |  |
| Enc Mem[%] | 100.0% |  |  | 99.7% |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |  |  |  | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall (EL+BL) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall (EL) | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100.6% |  |  | 100.5% |  |  | 100.8% |  |  |
| Dec Time[%] | 99.6% |  |  | 100.9% |  |  | 101.5% |  |  |
| Enc Mem[%] | 100.0% |  |  | 100.0% |  |  | 100.0% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |  |  |  | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Overall (EL+BL) | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Overall (EL) | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100.8% |  |  | 100.5% |  |  | 100.9% |  |  |
| Dec Time[%] | 101.3% |  |  | 101.0% |  |  | 100.7% |  |  |
| Enc Mem[%] | 100.0% |  |  | 100.0% |  |  | 100.1% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | 101% |  |  |  |  |  | 101% |  |  |
|  | 102% |  |  |  |  |  | 101% |  |  |

Run time reported in first and second release of this cross-check was not adequate. Up-dated version is accurate.

# Conclusion

The performance of switchable dependent on Intra prediction direction 2D up-sampling filter was verified in this report. Noticeable gain was observed in all intra test case: 0.1% (×2) and 0.2% (×1.5).

# Patent rights declaration(s)

Samsung Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).