| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0092 |
| --- | --- |

| Title: | SHVC TEB4: Cross-check report for adaptive up-sampling filter (test 4.2.1). |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Elena Alshina Alexander Alshin JeongHoon Park 416, Maetan 3-Dong, Gyeonggi-do, Suwon, Korea | Tel: Email: | +82 10 3026 1305 {elena_a.alshina, alexander_a.alshin, jeonghoon}@samsung.com |
| Source: | Samsung Electronics, Ltd. |  |  |

_____________________________

# Abstract

This is verification of TEB4 test 4.2.1 from Qualcomm. Adaptive up-sampling filter was tested vs up-sampling filter with predetermined coefficients. Average BD-rate improvement 0.3% (in ×2 tests), 0.7% (×1.5) and 2.5% (SNR) was confirmed.  Additional test results which will help to identify where gain comes from are also provided in this cross-check report.

# Introduction

Adaptive up-sampling filter was part of Qualcomm’s CfP response (JCTVC-K0036). In this TEB4 test performance of this tool was studied. S/w was released by proponent before dead-line, but ~3 weeks later an implementation bug was fixed by proponent. The performance of bug fix version is studied in this cross-check report.

After careful s/w analysis we have found following details of tested tool:

- Coefficients of up-sampling filter for current frame are determined at encoder side based on current frame statistics. Whole frame should be loaded into the memory and analysed before starting encoding process.

- In case of spatial scalability adaptive up-sampling filter coefficients are selected in order to minimize the difference between up-sampled BL and smoothed original EL. The smoothing of original EL requires additional processing vertical and horizontal filtration with 10 taps filter:

{ 2,     0,   -14,    33,    86,    33,   -14,     0,     2,     0 };

- Integer pel positions are modified by adaptive up-sampling filter in opposite to fixed up-sampling filter in reference s/w which doesn’t modify int-pel positions during up-sampling.

| Up-sampling filter application for x2 scalability |  | Up-sampling filter application or x1.5 scalability |  | Up-sampling filter application for SNR scalability |  |
| --- | --- | --- | --- | --- | --- |
| SMuC 0.1.1 filter | Proposed | SMuC 0.1.1 filter | Proposed | SMuC 0.1.1 filter | Proposed |

In case of x2 spatial scalability due to filtering of int-pel positions up-sampling filter application is doubled: 4  8, in case of x1.5 spatial scalability due to filtering of int-pel positions up-sampling filter application increases 1.5 times: 12  18

In SNR test fixed up-sampling filter of reference is not applied at all, but in case adaptive up-sampling filtration is needed (∞ increment for up-sampling process complexity). It courses at 25% decoding time increment.

- Adaptive up-sampling filter is applied for Luma only. Chroma is filtered using SMuC 0.1.1 default filter.

- Horizontal and vertical up-sampling filters are allowed to be different.

- The symmetry of filter coefficients is preserved in adaptive up-sampling: filter for phases α and 1-α are mirrored.

- Precision of filter coefficients is the same as in reference s/w.

- The sum of filter coefficients is 64 (one coefficient in a set is not signalled, but derived from normalization condition).

- Adaptive filter coefficients are signalled in APS (which doesn’t exist in HEVC specification).

- The difference between fixed up-sampled filter from reference s/w

{  0,  0,  0, 64,  0,  0,  0,  0},

{ -1, 4, -11, 52, 26,  -8, 3, -1},

{ -1, 4, -11, 40, 40, -11, 4, -1},

and adaptive one is signalled.

- For SNR scalability one horizontal and one vertical filter are needed. Following filter coefficients are allowed to be changed:

{  0,  0,  0, 64,  0,  0,  0,  0},

equal due to the symmetry

In total 6 coefficients need to be signalled in this case: 3 for vertical + 3 for horizontal.

- For x1.5 scalability 2 horizontal and 2 vertical filters are needed. Following filter coefficients are allowed to be changed:

{  0,  0,  0, 64,  0,  0,  0,  0},

equal due to the symmetry

{  -1, 4, -11, 52, 26,  -8, 3, -1},

In total 20 coefficients need to be signalled in this case: 10 for vertical + 10 for horizontal.

- For x2 scalability 2 horizontal and 2 vertical filters are needed. Following filter coefficients are allowed to be changed:

{  0,  0,  0, 64,  0,  0,  0,  0},

equal due to the symmetry

{  -1, 4, -11, 40, 40, -11, 4, -1},

equal due to the symmetry

In total 12 coefficients need to be signalled in this case: 6 for vertical + 6 for horizontal

- As explained in 11-13 adaptive filter coefficients signalling format depends on scalability ratio and defined for 3 cases only. It is not clear how to extend proposed algorithm for arbitrary scalability ration.

- Filter coefficient decision requires linear algebraic system solving. In verified implementation double arithmetic is used on encoder side. It is not clear if it is possible to implement this method using integer arithmetic and how much will be gain degradation.

# Test results for verified algorithm.

As required in TEB4 description proposed algorithm was tested vs fixed up-sampling filter is reference s/w. All Intra (x2 and x1.5), Random Access (x2, x1.5 and SNR) and Low-Delay P (x2 , x1.5 and SNR) tests were performed. Simulation results are summarised in Table 1 below. In average 0.3% (x2), 0.7% (x1.5) and 2.5% (SNR) Luma BD-rate gain is verified.

Table 1. Summation test results for verified algorithm.

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |
| Class B | -0.2% | 0.0% | 0.0% | -0.6% | -0.4% | -0.3% |  |  |  |
| Overall (EL+BL) | -0.1% | 0.0% | 0.0% | -0.6% | -0.4% | -0.3% |  |  |  |
| Overall (EL) | -0.3% | 0.0% | 0.0% | -2.4% | -0.9% | -0.9% |  |  |  |
| Enc Time[%] | 102.8% |  |  | 103.1% |  |  |  |  |  |
| Dec Time[%] | 99.3% |  |  | 99.4% |  |  |  |  |  |
| Enc Mem[%] | 106.8% |  |  | 107.1% |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.1% |  |  |  | -4.2% | -0.5% | -0.5% |
| Class B | -0.4% | 0.1% | 0.0% | -0.6% | 0.1% | 0.1% | -1.1% | 0.0% | 0.1% |
| Overall (EL+BL) | -0.3% | 0.1% | 0.0% | -0.6% | 0.1% | 0.1% | -2.0% | -0.2% | -0.1% |
| Overall (EL) | -0.5% | 0.1% | 0.1% | -2.0% | 0.4% | 0.3% | -4.1% | -0.1% | -0.1% |
| Enc Time[%] | 101.9% |  |  | 102.1% |  |  | 101.7% |  |  |
| Dec Time[%] | 99.4% |  |  | 100.5% |  |  | 124.0% |  |  |
| Enc Mem[%] | 104.1% |  |  | 105.1% |  |  | 112.8% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.2% | 0.3% |  |  |  | -5.8% | -0.3% | -0.1% |
| Class B | -0.8% | 0.2% | 0.2% | -1.0% | 0.5% | 0.6% | -1.8% | 0.7% | 0.9% |
| Overall (EL+BL) | -0.6% | 0.2% | 0.2% | -1.0% | 0.5% | 0.6% | -3.0% | 0.4% | 0.6% |
| Overall (EL) | -1.0% | 0.4% | 0.4% | -2.9% | 1.3% | 1.4% | -5.5% | 0.8% | 1.1% |
| Enc Time[%] | 101.9% |  |  | 102.3% |  |  | 102.0% |  |  |
| Dec Time[%] | 100.1% |  |  | 100.9% |  |  | 127.2% |  |  |
| Enc Mem[%] | 104.6% |  |  | 105.4% |  |  | 113.3% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
| Y BD-rate | -0.3% |  |  | -0.7% |  |  | -2.5% |  |  |
| Enc | 102% |  |  | 102% |  |  | 102% |  |  |
| Dec | 100% |  |  | 100% |  |  | 126% |  |  |
| Mem | 105% |  |  | 106% |  |  | 113% |  |  |

Encoding time increment is ~2% (due to adaptive up-sampling filter coefficients derivation).

Decoding time increment is 26% in SNR test.

In current reference s/w up-sampling for integer-pel position is done using filter with only one non-trivial coefficient (and other are zeros) that is why decoding complexity of verified algorithm in spatial scalability test is “hidden”.

Memory usage was not mandatory to report. In our tests memory usage increases by 5% (x2), 6% (x1.5) and 13% (SNR) compare to predetermined up-sampling filter in reference s/w.

Main gain is observed in SNR scalability test where adaptive up-sampling works as de-noising filter. The typical frequency response of filters chosen in SNR test is shown in Figure 1. For comparison we show 1/3 phase filter from reference s/w and down-sampling filter applied to int-pel positions in ×2. “Ideal” frequency response (straight line). It is easy to see that adaptive “up-sampling” filter in SNR case has very early high frequency cut similar to down-sampling filter used in spatial scalability. Both ×2 down-sampling filter and typical “up-sampling” filter in SNR test work as de-noising filters. But down-sampling ×2 filters operates as pre-processing (applied before BL coding). The adaptive “up-sampling” filter in SNR works as post-filter for base-layer signal. It kills coding noise present in reconstructed BL which

- was not pre-processes by de-noising filter (as it happens in spatial scalability)

- was encoded with poor quality (in SNR test).

In spatial scalability tests the gain from adaptive up-sampling filter is much lower compare to SNR test: 0.3% (×2) and 0.7% (×1.5).

There is no gain from adaptive up-sampling filter for class A (high resolution) sequences in spatial scalability tests. It seems one filter set per high resolution picture is not enough to adapt up-sampling process for particular content.

# Additional tests results

No int-pel filtration. Since reference s/w doesn’t modify int-pel positions during up-sampling fair comparison between fixed and adaptive up-sampling filter cannot be done within test 4.2.1 in TEB4. In order to estimate performance impact of integer positions filtration we disabled modification of integer pel samples in test 4.2.1 s/w. Performance test results are summarized in table 2 below.

There is no gain in SNR test at all since algorithm is not applied (2.5%  0.0%). In spatial scalability tests gain degrades as follows: 0.3  0.1% (x2), 0.7%  0.4% (x1.5). Our conclusion is w/o int-pel filtration adaptive up-sampling filter gain is minor. These numbers are similar to difference in performance between fixed up-sampling filters announced for testing in TEA1 (they were used on CfP stage, so considered more or less adequate by proponents, but most of them were withdrawn in TEA1).

Table 2. Performance of verified algorithm w/o int-pel filtration.

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | -0.3% | -0.2% | -0.2% |
| Overall (EL+BL) | 0.0% | 0.0% | 0.0% | -0.3% | -0.2% | -0.2% |
| Overall (EL) | -0.1% | 0.0% | 0.0% | -1.3% | -0.5% | -0.5% |
| Enc Time[%] | 103.0% |  |  | 103.0% |  |  |
| Dec Time[%] | 99.4% |  |  | 99.4% |  |  |
| Enc Mem[%] | 107.0% |  |  | 107.1% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | -0.2% | 0.1% | 0.0% | -0.4% | 0.1% | 0.1% |
| Overall (EL+BL) | -0.1% | 0.1% | 0.0% | -0.4% | 0.1% | 0.1% |
| Overall (EL) | -0.2% | 0.1% | 0.1% | -1.3% | 0.4% | 0.3% |
| Enc Time[%] | 101.9% |  |  | 102.0% |  |  |
| Dec Time[%] | 99.4% |  |  | 100.8% |  |  |
| Enc Mem[%] | 104.1% |  |  | 105.0% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.2% | 0.3% |  |  |  |
| Class B | -0.3% | 0.2% | 0.2% | -0.6% | 0.5% | 0.5% |
| Overall (EL+BL) | -0.2% | 0.2% | 0.2% | -0.6% | 0.5% | 0.5% |
| Overall (EL) | -0.3% | 0.3% | 0.4% | -1.7% | 1.2% | 1.2% |
| Enc Time[%] | 101.9% |  |  | 102.2% |  |  |
| Dec Time[%] | 100.6% |  |  | 100.9% |  |  |
| Enc Mem[%] | 104.6% |  |  | 105.4% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
| Avg Gain | -0.1% |  |  | -0.4% |  |  |

No smoothing of original EL signal. As mentioned above the smoothing of original El signal is used during adaptive up-sampling coefficients derivation (only for spatial scalability). In order to estimate the performance impact of this encoder hint we have disabled it and ran the sane tests. Performance test results are summarized in table 3 below. There is very small performance degradation in x1.5 tests (0.7%  0.6%) compare to full version. Our conclusion is this encoder hint is questionable.

Table 3. Performance of verified algorithm w/o original EL signal smoothing.

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | -0.2% | 0.0% | 0.0% | -0.4% | -0.1% | -0.1% |
| Overall (EL+BL) | -0.1% | 0.0% | 0.0% | -0.4% | -0.1% | -0.1% |
| Overall (EL) | -0.3% | 0.0% | 0.0% | -1.8% | -0.5% | -0.4% |
| Enc Time[%] | 102.6% |  |  | 102.7% |  |  |
| Dec Time[%] | 99.5% |  |  | 99.5% |  |  |
| Enc Mem[%] | 106.4% |  |  | 106.1% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.1% |  |  |  |
| Class B | -0.4% | 0.1% | 0.0% | -0.5% | 0.1% | 0.0% |
| Overall (EL+BL) | -0.3% | 0.1% | 0.0% | -0.5% | 0.1% | 0.0% |
| Overall (EL) | -0.5% | 0.1% | 0.1% | -1.9% | 0.3% | 0.1% |
| Enc Time[%] | 101.9% |  |  | 102.0% |  |  |
| Dec Time[%] | 99.6% |  |  | 100.6% |  |  |
| Enc Mem[%] | 104.2% |  |  | 105.1% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.2% | 0.3% |  |  |  |
| Class B | -0.8% | 0.3% | 0.2% | -0.9% | 0.5% | 0.4% |
| Overall (EL+BL) | -0.6% | 0.3% | 0.2% | -0.9% | 0.5% | 0.4% |
| Overall (EL) | -1.0% | 0.4% | 0.4% | -2.8% | 1.4% | 1.1% |
| Enc Time[%] | 101.8% |  |  | 102.2% |  |  |
| Dec Time[%] | 100.4% |  |  | 100.7% |  |  |
| Enc Mem[%] | 104.5% |  |  | 105.4% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
| Avg Gain | -0.3% |  |  | -0.6% |  |  |

# Conclusion

The performance of adaptive up-sampling filter from Qualcomm was verified in this cross-check report. The gain 0.3% (x2), 0.7% (x1.5) and 2.5% (SNR) was verified. Verified algorithm leads to significant increment of up-sampling process complexity since int-pel samples are filtered. W/o filtration of integer-pel positions the gain is minor 0.1% (x2), 0.4% (x1.5) and 0.0% (SNR), but not-fixed up-sampling filter coefficients will cause significant increment of h/w implementation cost.

# Patent rights declaration(s)

Samsung Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).