| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0094 |
| --- | --- |

| Title: | non TEC5: Cross-check of assigning intra prediction mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Elena Alshina Tammy Lee JeongHoon Park 416, Maetan 3-Dong, Gyeonggi-do, Suwon, Korea | Tel: Email: | +82 10 3026 1305 {elena_a.alshina, tammy.lee, jeonghoon}@samsung.com |
| Source: | Samsung Electronics, Ltd. |  |  |

_____________________________

# Abstract

This is verification of non-TEC5 contribution from LG (JCTVC-L0239). Proponent modified IntraBL handling in mode-dependent scan, transform, Intra direction prediction in neighbor blocks by synchronization with corresponding block on base layer.  In average 0.3% (Luma) and 0.2% (Chroma) BD-rate gain for the case when mode-dependent scan is allowed in IntraBL blocks is verified in this report. If parsing dependency is resolved by disabling mode-dependent scan in IntraBL blocks then an average gain is 0.2% for Luma and 0.0% for Chroma.

# Introduction

In the reference s/w SMuC0.1.1, if block is coded by inter layer intra prediction then it is handled intra prediction mode assuming DC prediction. It means that in mode-dependent scan, transform and neighbour blocks Intra direction prediction IntraBL is equal to Intra DC mode.

In verified contribution IntraBL handling is changed. If collocated block on BL was encoded as Intra then IntraBL block on enhancement layer will be handled that in mode-dependent scan, transform and neighbour blocks Intra direction according to the intra prediction direction of collocated BL block.

By this modification:

- DCT/DST transform related bug in SMuC 0.1.1 s/w is fixed

- Mode-dependent scan is allowed for IntraBL residual coding

- Indirectly intra prediction direction from base layer is included into MPM set:

Since utilizing of base layer intra prediction direction in mode-dependent scan on enhancement layer leads to the un-desirable parsing dependency the proponent provided additional test data when MDCS is disabled on enhancement layer.

# Test results for verified algorithm.

Verified algorithm was tested vs SMuC 0.1.1 s/w in All Intra (x2, x1.5), Random Access (x2, x1.5 and SNR) and Low Delay-P (x2, x1.5 and SNR) tests. Simulation results for assigned mode only and assigned mode w/o parsing dependency are summarised in Tables 1-2 below.

Table 1. Summation results for assigned mode.

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | -0.5% | -0.2% | -0.1% |  |  |  |  |  |  |
| Class B | -0.6% | -0.3% | -0.4% | -0.6% | -0.1% | -0.4% |  |  |  |
| Overall (EL+BL) | -0.6% | -0.3% | -0.3% | -0.6% | -0.1% | -0.4% |  |  |  |
| Overall (EL) | -1.0% | -0.4% | -0.5% | -1.5% | 0.1% | -0.4% |  |  |  |
| Enc Time[%] | 101.6% |  |  | 101.2% |  |  |  |  |  |
| Dec Time[%] | 101.9% |  |  | 101.5% |  |  |  |  |  |
| Enc Mem[%] | 99.2% |  |  | 99.6% |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.2% | -0.1% |  |  |  | -0.2% | -0.1% | -0.1% |
| Class B | -0.3% | -0.1% | -0.2% | -0.3% | 0.1% | -0.1% | -0.3% | 0.1% | -0.1% |
| Overall (EL+BL) | -0.3% | -0.1% | -0.2% | -0.3% | 0.1% | -0.1% | -0.2% | 0.0% | -0.1% |
| Overall (EL) | -0.4% | -0.2% | -0.2% | -0.7% | 0.3% | -0.1% | -0.3% | 0.3% | 0.1% |
| Enc Time[%] | 100.8% |  |  | 100.4% |  |  | 100.7% |  |  |
| Dec Time[%] | 100.1% |  |  | 100.6% |  |  | 102.4% |  |  |
| Enc Mem[%] | 99.9% |  |  | 100.0% |  |  | 100.0% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.2% | -0.2% |  |  |  | -0.1% | -0.4% | -0.3% |
| Class B | -0.1% | -0.1% | -0.3% | -0.1% | 0.0% | -0.1% | -0.1% | -0.1% | -0.3% |
| Overall (EL+BL) | -0.1% | -0.2% | -0.3% | -0.1% | 0.0% | -0.1% | -0.1% | -0.2% | -0.3% |
| Overall (EL) | -0.1% | -0.3% | -0.4% | -0.2% | 0.0% | -0.2% | -0.1% | -0.2% | -0.3% |
| Enc Time[%] | 100.7% |  |  | 100.4% |  |  | 100.7% |  |  |
| Dec Time[%] | 101.2% |  |  | 101.4% |  |  | 102.3% |  |  |
| Enc Mem[%] | 100.0% |  |  | 100.0% |  |  | 100.1% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

Table 2. Summation results for assigned mode w/o parsing dependency.

|  | AI HEVC 2x |  |  |  |  | AI HEVC 1.5x |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y |  | U | V |  |  |  |  |  |  |  |
| Class A |  | -0.3% | 0.1% | 0.0% |  |  |  |  |  |  |  |  |  |  |  |
| Class B |  | -0.4% | 0.0% | 0.0% | -0.5% |  | 0.2% | 0.0% |  |  |  |  |  |  |  |
| Overall (EL+BL) |  | -0.4% | 0.0% | 0.0% | -0.5% |  | 0.2% | 0.0% |  |  |  |  |  |  |  |
| Overall (EL) |  | -0.7% | 0.1% | 0.0% | -1.3% |  | 0.7% | 0.4% |  |  |  |  |  |  |  |
| Enc Time[%] |  | 101.1% |  |  | 100.9% |  |  |  |  |  |  |  |  |  |  |
| Dec Time[%] |  | 101.3% |  |  | 101.3% |  |  |  |  |  |  |  |  |  |  |
| Enc Mem[%] |  | 99.7% |  |  | 99.3% |  |  |  |  |  |  |  |  |  |  |
| BL Match |  | Matched |  |  | Matched |  |  |  |  |  |  |  |  |  |  |
|  |  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |  |  | RA HEVC SNR |  |  |  |  |  |
|  |  | Y | U | V | Y |  | U | V |  | Y |  | U |  | V |  |
| Class A |  | -0.1% | -0.1% | -0.1% |  |  |  |  |  | -0.2% |  | 0.0% |  | 0.0% |  |
| Class B |  | -0.2% | 0.2% | 0.2% | -0.3% |  | 0.2% | 0.1% |  | -0.2% |  | 0.3% |  | 0.2% |  |
| Overall (EL+BL) |  | -0.2% | 0.1% | 0.1% | -0.3% |  | 0.2% | 0.1% |  | -0.2% |  | 0.2% |  | 0.1% |  |
| Overall (EL) |  | -0.3% | 0.2% | 0.1% | -0.6% |  | 0.5% | 0.3% |  | -0.3% |  | 0.6% |  | 0.4% |  |
| Enc Time[%] |  | 100.3% |  |  | 100.3% |  |  |  |  | 100.4% |  |  |  |  |  |
| Dec Time[%] |  | 99.5% |  |  | 101.4% |  |  |  |  | 102.1% |  |  |  |  |  |
| Enc Mem[%] |  | 100.0% |  |  | 100.1% |  |  |  |  | 100.0% |  |  |  |  |  |
| BL Match |  | Matched |  |  | Matched |  |  |  |  | Matched |  |  |  |  |  |
|  |  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |  |  | LD-P HEVC SNR |  |  |  |  |  |
|  |  | Y | U | V | Y |  | U | V |  | Y |  | U |  | V |  |
| Class A |  | 0.0% | -0.2% | -0.2% |  |  |  |  |  | -0.1% |  | -0.4% |  | -0.2% |  |
| Class B |  | -0.1% | 0.0% | -0.1% | -0.1% |  | 0.0% | 0.0% |  | -0.1% |  | 0.0% |  | -0.1% |  |
| Overall (EL+BL) |  | -0.1% | -0.1% | -0.1% | -0.1% |  | 0.0% | 0.0% |  | -0.1% |  | -0.1% |  | -0.1% |  |
| Overall (EL) |  | -0.1% | -0.1% | -0.2% | -0.1% |  | 0.1% | 0.0% |  | -0.1% |  | -0.1% |  | -0.1% |  |
| Enc Time[%] |  | 100.3% |  |  | 100.2% |  |  |  |  | 100.3% |  |  |  |  |  |
| Dec Time[%] |  | 100.6% |  |  | 101.6% |  |  |  |  | 101.5% |  |  |  |  |  |
| Enc Mem[%] |  | 100.0% |  |  | 100.0% |  |  |  |  | 100.0% |  |  |  |  |  |
| BL Match |  | Matched |  |  | Matched |  |  |  |  | Matched |  |  |  |  |  |

In average 0.3% Luma and 0.2% Chroma BD-rate gain is confirmed in verified test. There is no noticeable increment of encoding or decoding time.

# Conclusion

The performance of assigned mode proposed by LG is verified in this cross-check report. Average gain is 0.3% (Luma) and 0.2%(Chroma) in terms of BD-rate. Verified algorithm doesn’t increase encoding or decoding time. The variant w/o parsing dependency provides 0.2% (Luma) and 0.0%(Chroma) gain vs SMuC 0.1.1 in terms of BD-rate.