| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0095 |
| --- | --- |

| Title: | SHVC TEB3: Cross-check of generalized residual prediction from Canon (test 4.6.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Alexander Alshin Elena Alshina JeongHoon Park 416, Maetan 3-Dong, Gyeonggi-do, Suwon, Korea | Tel: Email: | +82 10 3026 1305 { alexander_a.alshin, elena_a.alshina, jeonghoon}@samsung.com |
| Source: | Samsung Electronics, Ltd. |  |  |

_____________________________

# Abstract

This is verification of TE_B3 test 4.6.1 from Canon. Generalized Residual Inter-Layer Prediction was tested. Average BD-rate improvement 1.6% (Luma) and 2.6% (Chroma) is confirmed.

# Introduction

Generalized Residual Inter-Layer Prediction (GRILP) method comprises of linear combination enhancement layer temporal prediction and up-sampling temporal residual in the base layer. The prediction is obtained as follows:

PRED = MC[REFEL, MVEL] + UP[RECBL] - MC[UP[REFBL], MVEL]

where PRED is the prediction signal, REFEL and REFBL are the corresponding reference pictures in the Enhancement and Base layers, RECBL is the base layer reconstructed picture corresponding to the current enhancement layer picture, UP[.] is the up-sampling operator, MC[.,MV] is the motion compensation using the motion vector MV. We have found following details of tested tool:

- For this algorithm additional Grilpflag is needed which is written as one bit on CU level only if two conditions are valid: current layer is enhancement and PredictionMode is INTER.

- One additional context model is introduced for this flag.

# Test results for verified algorithm.

As required in TEB3 description proposed algorithm was tested vs reference s/w SMuC 0.1.1. Random Access (x2, x1.5 and SNR) and Low-Delay P (x2 , x1.5 and SNR) tests were performed. Simulation results are summarised in Table 1 below. Both bit-rate and PSNR match with data from proponent.

Table 1. Summary of test results for verified algorithm.

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.3% | -3.2% | -3.2% |  |  |  | -0.7% | -2.9% | -3.1% |
| Class B | -1.6% | -2.2% | -2.2% | -1.8% | -3.2% | -3.3% | -1.2% | -2.6% | -2.8% |
| Overall (EL+BL) | -1.5% | -2.5% | -2.5% | -1.8% | -3.2% | -3.3% | -1.0% | -2.7% | -2.9% |
| Overall (EL) | -2.7% | -4.7% | -4.8% | -4.6% | -7.8% | -8.2% | -2.0% | -5.4% | -5.7% |
| Enc Time[%] | 177.0% |  |  | 164.2% |  |  | 152.5% |  |  |
| Dec Time[%] | 100.6% |  |  | 98.1% |  |  | 101.4% |  |  |
| Enc Mem[%] | 100.3% |  |  | 100.3% |  |  | 100.3% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.4% | -3.0% | -3.0% |  |  |  | -0.9% | -2.6% | -2.7% |
| Class B | -1.9% | -2.3% | -2.2% | -2.3% | -3.5% | -3.5% | -1.4% | -2.3% | -2.6% |
| Overall (EL+BL) | -1.7% | -2.5% | -2.4% | -2.3% | -3.5% | -3.5% | -1.3% | -2.4% | -2.6% |
| Overall (EL) | -3.1% | -4.6% | -4.7% | -5.7% | -8.4% | -8.5% | -2.4% | -4.6% | -4.9% |
| Enc Time[%] | 171.7% |  |  | 158.5% |  |  | 149.6% |  |  |
| Dec Time[%] | 100.1% |  |  | 97.1% |  |  | 100.1% |  |  |
| Enc Mem[%] | 100.3% |  |  | 100.3% |  |  | 100.3% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | -1.6% | -2.6% |  |  |  |  |  |  |  |

Following main effects were observed for verified algorithm:

- Encoding time increment is ~60% compare to SMuC 0.1.1.

- Decoding time is almost equal to SMuC 0.1.1.

- Memory usage was not mandatory to report. In our tests memory usage increases by 0.3 compare to SMuC 0.1.1.

- The highest gain is observed in Low-Delay P x1.5 scalability test (2.3%) however performance improvement is consistent for all test scenarios.

# Conclusion

The performance of Generalized Residual Inter-Layer Prediction (Canon) was verified in this cross-check report. The gain 1.6% (x2), 2.1% (x1.5) and 1.2% (SNR) was confirmed. Encoding time increase for verified algorithm is not small however no decoding time increment is observed. No significant memory increment is noticed in current s/w implementation. A careful comparison with other techniques from generalized residual prediction branch (4.6) is needed. We recommend paying attention to this algorithm.