| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 12th Meeting: Geneva, CH, 14-23 January, 2013 | Document: JCTVC-L0096 |
| --- | --- |

| Title: | Hardware Implementation of a HEVC Decoder |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Seunghyun Cho Hyunmi Kim 138 Gajeongno, Yuseong-gu, Daejeon, 305-700, KOREA | Tel: Email: | +82-42-860-6274 shcho@etri.re.kr chaos0218@etri.re.kr |
| Source: | Electronics and Telecommunications Research Institute (ETRI) |  |  |

_____________________________

# Abstract

This document introduces a hardware implementation of a HEVC decoder developed using Verilog HDL and fully synthesizable. The decoder is capable of decoding a 1080p HEVC bitstream at 60fps while running at 266MHz.

# Introduction

ETRI has implemented on hardware, a real-time HEVC decoder codenamed Stellar. Almost all decoding processes for the HEVC main profile is accelerated by dedicated hardware in Stellar for enhanced decoding performance and reduced power consumption. Table 1 summarizes Stellar and its system environment.

Table 1. Summary of the HEVC decoder, Stellar

| Reference HM version | 6.0 |
| --- | --- |
| Target profile / level | Main profile / 4.3 |
| Implementation scheme | Dedicated decoder hardware cooperating with a RISC processor - RISC processor: parses HEVC high level syntaxes including slice header and controls the decoder H/W. - Decoder H/W: manipulates HEVC bitstream and performs most of decoding process starting from slice data parsing. |
| Decoder H/W pipeline | CTU based |
| Cache for motion compensation | Implemented |
| Interface of decoder H/W for SoC integration | - A 64-bit AMBA AXI bus master: used for the decoder to access DDR3 SDRAM - A 32-bit AMBA AHB bus slave: used for the processor to configure the decoder and to diagnose the status of decoder. - An interrupt pin to the processor |
| Performance | - 1080p@60fps running at 266MHz - 1080p@30fps running at 133MHz - 720p@30fps running at 58MHz - 480p@30fps running at 20MHz |
| Decoder H/W size | Synthesis result in 65nm technology: - Logic: 1.76 Mgates - SRAM: 178KB |

Figure 1 shows the overview of system environment used to develop Stellar. SNAKE, a 32-bit RISC processor developed by ETRI, is used for parsing HEVC high level syntaxes including slice header and control of other hardware modules. The AMBA Network Interconnect (NIC-301) is used as the network infrastructure of the system.

Stellar is configured and controlled by SNAKE through a 32-bit AMBA AHB slave interface. A 64-bit wide DDR3 memory is used to store the HEVC bitstream, motion data, and reference pictures. Stellar accesses the DDR3 memory via a 64-bit AMBA AXI master interface.

Figure 1. System environments overview

# Hardware Structure of Stellar

Figure 2 shows a block diagram of Stellar including necessary system bus interfaces. A 64-bit AXI bus master is used to read the bitstream, co-located motion data, and reference picture.

Figure 2. Block diagram of Stellar

Figure 3 illustrates the CTU based pipeline structure of Stellar.

Figure 3. CTU based pipeline structure of Stellar

Explanation of the module names used in Figure 2 and 3 are below:

- CMV_LD (co-located motion vector load): loads motion data of co-located CUs for TMVP from external memory.

- SEP (syntax element parsing): parses syntax elements for a CTU.

  - BSC (bitstream controller): reads and manipulates bistream from external memory.

  - IMPRD (intra mode prediction): generates intra prediction modes for a CTU.

  - MVPRD (motion vector prediction): generates motion data for a CTU.

- CMV_ST (co-located motion vector store): stores motion data of CUs into external memory.

- ITQ (inverse transform and quantization): performs scaling and transformation to obtain residual samples.

- IPRD (intra prediction): performs intra prediction to obtain predicted samples.

- MC (motion compensation): performs inter prediction to obtain predicted samples.

- DBF (deblocking filter): performs deblocking filtering

- SAO (sample adaptive offset): performs SAO filtering

- RST (reconstructed picture store): stores the reconstructed picture into external memory.

Several design issues encountered during development of Stellar are

- While decoding a CTU, exploring hierarchies of CUs, PUs, and TUs consumes a small but not negligible number of clock cycles.

- CTU based pipeline may require a large amount of internal SRAM.

- Outputs of DBF and SAO are not aligned to the CTU boundary and they are irregular at picture boundaries.

- The IPRD and MC should share with each other their decoding status of CUs in a CTU.

# Hardware Synthesis Results

Synthesis results of sub-modules of Stellar are listed in Table 2.

Table 2. Synthesis results of Stellar (65nm technology, target freq.: 266MHz)

| Module name | Gate count (Kgates) | SRAM (Kbytes) |
| --- | --- | --- |
| SEP (w/ CABAC) | 138 | 1.9 |
| ITQ | 779 | 2 |
| IPRD | 270 | 4.2 |
| MC (w/ Cache) | 440 | 50 |
| DBF | 84 | 16.5 |
| SAO | 32 | 10.3 |
| RST | 10 | 15 |
| Etc. (control & communication) | 10 | 78 |
| Total | 1,763 | 177.9 |

Although the hardware area is dependent on implementation circumstances, from our experience, HEVC needs about twice the logic count and 50% more SRAM compared to H.264/AVC when implemented in hardware.

Given the capability of current state of the art semiconductor technology, the described work confirms the feasibility of hardware implementation of the HEVC decoder as an IP to be integrated in a SoC.

# Demonstration on a FPGA prototyping board

Figure 4 shows a snapshot of the FPGA prototyping board used for developing Stellar and the supporting system. A Xilinx Virtex-6 FPGA on the prototyping board provides the necessary hardware resources for the system environment shown in Figure 1 including a SNAKE processor, Stellar, video controllers, SDRAM controllers, and other peripherals. A live demonstration of the HEVC decoder running on the FPGA prototyping board will be made during the JCT-VC meeting.

Figure 4. a snapshot of FPGA prototyping board for Stellar.

# Conclusion

ETRI implemented a fully synthesizable hardware HEVC decoder, which is capable of decoding a 1080p HEVC bitstream at 60fps while running at 266MHz. Substantial increase in hardware area compared to H.264/AVC hardware decoder is required, but it can be said that it is acceptable. The implementation result confirms the feasibility of the current HEVC design.

# References

- B. Bross, W.–J. Han, J.-R. Ohm, G. J. Sullivan and T. Wiegand, “High efficiency video coding (HEVC) text specification draft 6”, JCTVC-H1003, JCT-VC 8th Meeting, San José, CA, USA, February, 2012