| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0097_v2 |
| --- | --- |

| Title: | Sample scale factor in VUI |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Arturo A. Rodriguez 5030 Sugarloaf Parkway Lawrenceville GA 30044 USA | Tel: Email: | +1 770 236 6483 rodrigaa@cisco.com |
| Source: | Cisco Systems |  |  |

_____________________________

# Abstract

A sample scale factor is proposed to support applications that require the intended display area produced from the conformance cropping window be kept constant throughout coded video sequences that have different picture resolutions.  The sample aspect ratio may be used to scale the intended display area when the picture resolution changes, except when the active sample aspect ratio does not change, such as when transitioning between 1920x1080 and 1280x720 picture resolutions.  This limitation is addressed by using the proposed sample scale factor to multiply the width and height of the sample aspect ratio to produce the intended horizontal and vertical distances between the columns and rows of the samples in the intended display area. The sample scale factor defaults to a value equal to one when it is not present.

# Introduction

Some applications require that the picture resolution of the intended display area be kept constant across transitions of coded video sequences (CVSes) with different picture resolutions [1].  The intended display area herein refers to the corresponding pictures produced from the respective frames of the conformance cropping window.  The resolution of the intended display area can be kept constant when the picture resolution changes by performing any necessary upconversion or downcoversion on the samples of the frames of the conformance cropping window according to the specified sample aspect ratio.

The sample aspect ratio (SAR) identified by aspect_ratio_idc is sufficient to configure the necessary upconversion or downconversion to maintain the display area constant when the picture resolution changes, except when consecutive CVSes have the same SAR, such as when transitioning between 1920x1080 and 1280x720 picture resolutions.  A sample scale factor (SSF) is proposed to address this limitation. Specifically, when necessary the SSF is signalled in VUI to multiply the width and height of the sample aspect ratio to produce the intended horizontal and vertical distances between the columns and rows of the samples in the intended display area.

The proposed SSF is signaled only as needed and set equal to one when not present.

The following examples illustrate the usage of the proposed sample scale factor.

Example 1 - Transitions between 1920x1080 and 1280x720 with predominant 1920x1080

The CVSes for the 1920x1080 portions signal aspect_ratio_idc equal to 1 (i.e., SAR = 1) and SSF equal to 1.   CVSes with 1280x720 picture resolution are signaled also with SAR equal to 1 but with a SSF equal to 3 2 to keep the intended display area constant.

Example 2 - Transitions between 1920x1080 and 1280x720 with predominant 1280x720

The CVSes for the 1280x720 portions signal aspect_ratio_idc equal to 1 (i.e., SAR = 1) and an implied SSF equal to 1.   CVSes with 1920x1080 resolution are signaled with SAR equal to 1 SSF equal to 2 3 to keep the display area constant.

Example 3 - Transitions between 1920x1080 and 960x540 with predominant 1920x1080

The CVSes for the 1920x1080 portions signal aspect_ratio_idc equal to 1 (i.e., SAR = 1) and SSF equal to 1.   CVSes in 960x540 resolution are signaled with SAR equal to 1 and  SSF equal to 2 to keep the display area constant.

# Proposed syntax and semantics

Relevant VUI parameters syntax (truncated after overscan_appropriate_flag)

| vui_parameters( ) { | Descriptor |
| --- | --- |
| aspect_ratio_info_present_flag | u(1) |
| if( aspect_ratio_info_present_flag ) { |  |
| aspect_ratio_idc | u(8) |
| if( aspect_ratio_idc = = Extended_SAR ) { |  |
| sar_width | u(16) |
| sar_height | u(16) |
| } |  |
| sample_scale_factor_flag | u(1) |
| if(sample_scale_factor_flag) |  |
| sample_scale_factor_index | u(3) |
| } |  |
| } |  |
| overscan_info_present_flag | u(1) |
| if( overscan_info_present_flag ) |  |
| overscan_appropriate_flag | u(1) |
| …. |  |

Table E.X - Sample Scale Factor Table

| sample_scale_factor_index | SSF |
| --- | --- |
| 0 | 1.0 |
| 1 | 3 2 |
| 2 | 2 3 |
| 3 | 2.0 |
| 4-7 | Reserved |

sample_scale_factor_flag equal to 1 specifies that sample_scale_factor_index is present and identifies the sample scale factor (SSF) in Table EX to multiply the horizontal width and vertical height of the sample aspect ratio (SAR), expressed by h:v, to produce the resolution of the intended display area from each constituent frame of the conformance cropping window as illustrated in Figure E-X1 and Figure E-X2. sample_scale_factor_flag equal to 0 specifies that sample_scale_factor_index is not present.

sample_scale_factor_index identifies the corresponding SSF in Table E.X to multiply the horizontal and vertical distances between the columns and rows of the samples of each constituent frame of the conformance cropping window to produce the intended display area as illustrated in Figure E- X1 and Figure E-X2.  When sample_scale_factor_index is not present, its value is equal to 0.

Figure E-X1

Figure E-X2

- References

[1] A. Rodriguez , “Sample scale factor in VUI”, JCTVC- K0263, Shanghai, China, October 2012.

# Patent rights declaration(s)

Cisco Systems may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).