| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 January 2013 | Document: JCTVC-L0098 |
| --- | --- |

| Title: | On software complexity: decoding 4K60p content on a laptop |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | TK Tan1 Yoshinori Suzuki1 3-6 Hikarinooka, Yokosuka-shi Kanagawa, 239 8536 Japan Frank Bossen2 3240 Hillview Ave Palo Alto CA 94304 USA | Tel: Email: | +65 9799 3816 tktan@m-sph.com suzukiyos@rd.nttdocomo.co.jp +1-650-496-4742 bossen@docomoinnovations.com |
| Source(s): | 1 NTT DOCOMO, Inc. & 2 DOCOMO Innovations, Inc. |  |  |

_____________________________

# Abstract

This contribution provides an update on HEVC software decoding. Previously presented contributions have shown the HEVC software decoding of 480p to 1080p at 25 or 30 fps, running on a single core of an ARM processor.  In contrast, this contribution demonstrates that software playback of 4K sequences at 60 fps is possible on a laptop with a quad-core Core-i7 processor when multiple decoding threads are used.

# Introduction

Screen resolutions are poised to advance to the next level for televisions and laptops.  Several 4K resolution (3840x2160 luma samples) flat panel televisions have been announced and are commercially available.  For laptops, resolutions of 2880x1800 luma samples have been available since June 2012.

Several prior contributions [1][2][3] have reported real-time HEVC software decoding of 480p video on mobile devices. This was extended to 720p in [4] and 1080p resolution in [5]. Considering the mobile application domain, video is constrained to 25/30 frames per second.

In previous contributions [1], [4] and [5], the decoding process is running on a single core of an ARM processor. The player application itself is multi-threaded, with separate threads for decoding and display. OpenGL is used to display video in real time. Resampling (scaling to fit screen size) and YUV to RGB color conversion are done on the GPU during shading.

In this contribution, multiple decoding threads were added to the above to enable higher resolution and frame rates to demonstrate the decoding capability of the software decoder when running on a quad-core Core-i7 (Ivy Bridge) processor clocked at 2.7 GHz.

# Experimental results

HM 9.0 bitstreams (random access (RA main) configuration from the common test conditions [6]) were used. The highest playback rate possible without causing the display buffer to underflow (smooth playback) was measured as a function of the number of decoding threads, regardless of the original frame rate (and bitrate) that the bitstreams were encoded for.

The table below summarizes the sequences that were used, including their length and their original bitrate (not adjusted for playback rate).  To ensure that a sufficient number of continuous frames were used in the test, the 4K sequences that consist of approximately 1000 frames were repeated 5 times.

Table 1: Sequences used for the test.

| Sequence | Resolution | Original frame rate [fps] | # frames x repeats | QP setting | Bit rate [Mbps] |
| --- | --- | --- | --- | --- | --- |
| diving | 3840 x 2160 | 60 | 1046 x 5 | 31 | 12.7 |
| pool | 3840 x 2160 | 60 | 1019 x 5 | 32 | 11.7 |
| nebuta | 3840 x 2160 | 60 | 1062 x 5 | 33 | 12.6 |

## Frame rate as a function of decoder threads

In this test, the video sequences were played with different number of decoding threads and the frame rate where smooth playback (display buffer does not underflow) was recorded.  The table and the graph below show the smooth playback frame rate achieved.

Table 2: Smooth playback frame rate achieved as a function of parallel decoding threads used.

| Sequence | Frame rate [fps] where smooth playback is achieved with |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1 decoding thread | 2 decoding threads | 3 decoding threads | 4 decoding threads |
| diving | 40 | 70 | 90 | 100 |
| pool | 30 | 56 | 71 | 80 |
| nebuta | 27 | 52 | 68 | 76 |

Figure 1: Plot of playback frame rate as a function of number of decoding threads

# Conclusion

Experiments have shown that real-time decoding and display of a variety of 4K sequences is feasible on a laptop where the multiple cores are utilised for the decoding process. The frame rate of 60 frames per second or more can be achieved for sequences encoded with HM9.0 random access main profile at a bitrate of approximately 12 Mbps, by using 3 parallel decoding threads on laptop with a quad core, 2.7 GHz, Core-i7 processor.  Up to 100 fps was achieved with 4 parallel decoding threads on the same laptop.

# Acknowledgements

The original sources of the video sequences were provided by NHK, Japan.  The resolution of the original source was 8K UHDTV content.

# References

- F. Bossen, “On software complexity”, JCT-VC document G757, Geneva, November 2011.

- K. McCann, J.-Y. Choi, K. Pachauri, K. P. Choi, C. Kim, Y. Park, Y. J. Kwak, S. Jun, M. Choi, H. Yang, and J. Park, “HEVC software player Demonstration on Mobile devices,” JCT-VC document G988, Geneva, November 2011.

- K. Veera, R. Ganguly, B. Zhou, N. Kamath, S. Chowdary, J. Du, I.-S. Chong, and M. Coban, “A real-time ARM HEVC decoder implementation,” JCT-VC document H0693, San Jose CA, February 2012.

- F. Bossen, “On software complexity: decoding 720p content on a tablet”, JCT-VC document J0128, Stockholm, July 2012

- F. Bossen, “On software complexity: decoding 1080p content on a smartphone”, JCT-VC document K0327, Shanghai, October 2012

- F. Bossen, “Common test conditions and software reference configurations”, JCT-VC document J1100, Stockholm, July 2012.

# Patent rights declaration(s)

NTT DOCOMO, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).