| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0100 |
| --- | --- |

| Title: | TE3: Results of test 4.6.1 on the Generalized Residual Inter-Layer Prediction (GRILP) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Christophe Gisquet Fabrice Le Léannec Jonathan Taquet Edouard François Guillaume Laroche Patrice Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33(0)299876800 christophe.gisquet@crf.canon.fr fabrice.leleannec@crf.canon.fr edouard.francois@crf.canon.fr guillaume.laroche@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon Research Centre France |  |  |

_____________________________

# Abstract

This contribution provides the results of the test 4.6.1 of TE3 related to Combined Prediction in SHVC. More precisely, this contribution is about the Generalized Residual Inter-Layer Prediction (GRILP) method presented by Canon in JCTVC-K0041 and integrated in the SMuC version 0.1.1 for further evaluation. It is reported that the Y-BDR performance of the GRILP method respectively brings -1.5%(RA 2x), -1.8%(RA 2x), -1.0%(RA SNR) and -1.7%(LD-P 2x), -2.3%(LD-P 1.5x), -1.3%(LD-P SNR) over the SMuC 0.1.1.

# Introduction

The GRILP method has been proposed in JCTVC-K0041[2] as a new inter-layer coding mode for the enhancement layer. In the previous SVC standard[3], the base layer’s residual data that serves as reference data for Inter-layer residual prediction corresponds to base Inter macro-blocks residual in their decoded form. This was in-line with the single-loop decoding architecture chosen for the SVC standard.

Here, in the future SHVC codec, a multiple-loop decoding architecture is chosen. Therefore, more freedom is allowed in the codec design, since fully reconstructed base pictures are available. Moreover, one notices that in the former SVC approach, a decoded temporal residual block is associated with one motion vector. Then, in the temporal prediction process that takes place in the enhancement layer, if the enhancement motion vector is not correlated to the base CU’s motion vector, then it is very likely that the residual associated to the base CU will not serve for Inter-layer residual prediction. In addition, GRILP works without inheriting any other information from the Base Layer than the texture.

As a consequence, we proposed in K0041 an improvement of SVC residual Inter-layer prediction which is adapted to a multiple-loop decoding approach. This GRILP tool is further described in the following section.

# GRILP tool description

Generalized Residual Inter-Layer Prediction (GRILP) mode consists in generating, given a Prediction Unit and motion vector MV being considered, a corresponding temporal residual CU in the base layer that is added to enhancement layer temporal prediction PU.

The GRILP prediction mode is selected on a CU basis. It can be activated inside Inter CUs and is signaled by a flag at the CU level. When activated, it is used for all PUs of the CU. The GRILP prediction mode proceeds as follows.

The prediction signal is derived as follows (shown for the mono-prediction case, the concept is easily generalized for the bi-prediction case):

PREDEL = MC[REFEL, MVEL] + UP{RECBL} - MC[UP{REFBL}, MVEL]

where PREDEL is the prediction signal of the enhancement layer, REFEL and REFBL are the reference pictures in the Enhancement and Base layers, RECBL is the base layer reconstructed picture corresponding to the current enhancement layer picture, MVEL is the motion vector selected, UP{.} is the up-sampling operator, MC[I,MV] is the motion compensation step of the current picture using I as reference picture and MV for the motion vectors.

For this test 4.6.1, the interpolation filters for motion compensation are those of the HEVC core design. The up-sampling interpolation filters are those defined in the TE 2.

The principle of the GRILP mode can be summarized by the following Figure 1.

Figure 1: Schematic principle of the GRILP mode

# Results

The testing conditions are those defined by TE3 description document[1]. The version of the SMuC is 0.1.1. The encoding and decoding runtime are not reliable.

# Conclusion

This contribution reports the performance of the GRILP tool initially presented in contribution K0041 for the Call for Proposal of Scalable technologies. The average BDR performance gain of the GRILP method is about -1.6%(Y), -2.8%(U), - 2.9%(V) under the TE3 testing conditions over the SMuC 0.1.1.

# References

- X. Li, E. Francois, P. Lai, D. Kwon, A. Saxena, “Description of Tool Experiment B3: Combined Prediction in SHVC”, Joint Collaborative Team on Video Coding, JCTVC-K1103, October, 2012.

- Sébastien Lasserre, Fabrice Le Léannec, Jonathan Taquet, Naël Ouedraogo, Stéphane Pautet, Christophe Gisquet, Guillaume Laroche, Tangi Poirier, Yann Verdavaine, Edouard Francois, Eric Nassor, “Description of the scalable video coding technology proposal by Canon Research Centre France” Joint Collaborative Team on Video Coding, JCTVC-K0041, October, 2012.

- ITU-T Recommendation H.264, “Advanced Video Coding for Generic Audiovisual Services,” March, 2010

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).