| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0101 |
| --- | --- |

| Title: | TE5: Results of test 5.2.6 on the Inter-Layer Motion Vector Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guillaume Laroche Tangi Poirier Patrice Onno Christophe Gisquet Edouard François Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33(0)299876800 guillaume.laroche@crf.canon.fr patrice.onno@crf.canon.fr christophe.gisquet@crf.canon.fr edouard.francois@crf.canon.fr |
| Source: | Canon Research Centre France |  |  |

_____________________________

# Abstract

This contribution provides the results of the test 5.2.6 of TE5 related to Inter-layer syntax prediction using HEVC base layer in SHVC. More precisely, this contribution is about the Inter-layer motion vector prediction method presented in JCTVC-K0041 and integrated in the SMuC version 0.1.1 for further evaluation. It is reported, under the TE5 test conditions, that the Y-BDR performance of the proposed inter-layer motion vector prediction method brings -1.7%(RA 2x), -2.2%(RA 1.5x), -2.2%(RA SNR) and -0.7%(LD-P 2x), -1.1%(LD-P 1.5x), -1.6%(LD-P SNR) over the SMuC 0.1.1. Furthermore, this contribution brings additional results about the combination of this motion vector prediction tool and the Base Mode method presented in JCTVC-L0102. It is reported that the combination shows the following performance -3.6%(RA 2x), -4.3%(RA 2x), -4.0%(RA SNR) and -2.7%(LD-P 2x), -3.4%(LD-P 1.5x), -3.7%(LD-P SNR) over the SMuC 0.1.1.

# Introduction

In JCTVC-K0041 [2], an Inter-layer motion vector prediction method is proposed to take into account the motion vector of the collocated CU used in the base layer. This motion vector is used as motion vector candidate in the enhancement layer.

In addition, this contribution brings additional results about the combination of this motion vector prediction and the Base Mode presented in JCTVC-L0102 [4]. Indeed when the Base Mode is enabled, it is very similar to introduce the base motion vector as motion vector predictor for the enhancement layer. The modification of the Merge process for this proposal takes into account the enabling/disabling of the Base Mode method to avoid preponderance of the base layer motion vector in the candidate list.

# Inter-layer motion prediction tool description

In this contribution, the MERGE derivation predictors specified for HEVC is adapted to the enhancement layer prediction in order to improve the coding efficiency of the Merge and Merge Skip modes. For this TE experiment, the AMVP process remains unchanged for the enhancement layer.

## Modification of the Enhancement layer Merge mode

The derivation process of the Merge, Merge Skip presented in JCTVC-K0041 [2] has been modified compared to the HM8.1, the number of candidates and the order of candidates are modified. Moreover, new candidates are added for the enhancement layer. These adaptations are listed below.

- The number of candidates in the final list is increased from 5 to 6.

- The temporal candidate is on the first position if the temporal collocated block is selected in a forward reference. In that case only the centered position is considered for the enhancement layer. Please note that it affect only the random access case.

- The derivation process of spatial candidates is the same as for the base layer, except that the duplicate check is the same as the MVP hook duplicate check.

- Then the motion vector of the co-located base layer is added to the list of candidates. In that case the bottom right position is considered.

- The base motion vector is followed by 4 offset candidates derived according to derived according to  the first candidate of the list. The generation of these candidates consists in adding one offset (offset value: 4 or -4) alternatively to the horizontal or to the vertical component of the motion vector from list L0 and its inverse value to components of motion vector from list L1.

- Then the combined candidates derivation and the zero candidates derivation are applied if at least one position is available. Yet the number of possible combination of combined candidates have been modify to match 6 candidates in the list.

- Moreover, the MV data compression was removed for the Enhancement layer as it was definitively removed for HM9.

# Results

The testing conditions are those defined by TE5 description document [1]. The version of the SMuC is 0.1.1. The encoding and decoding runtime are reliable. In average for the mandatory tests, 1.4% gain is obtained.

# Additional results

MV compression for temporal motion vector

In addition to the previous results, we enable the memory compression for the temporal motion vector in order to be comparable to the other TE5.2 experiments.

In average for the mandatory sequences 1.4% gain is obtained. As the conclusion, the temporal motion vector compression doesn’t need to be removed.

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -1.1% | -2.3% | -2.2% |  |  |  |
| Class B | -1.7% | -2.5% | -2.6% | -2.1% | -2.8% | -3.1% |
| Overall (EL+BL) | -1.6% | -2.5% | -2.5% | -2.1% | -2.8% | -3.1% |
| Overall (EL) | -3.0% | -4.5% | -4.7% | -5.6% | -7.1% | -7.7% |
| Enc Time[%] | 98.7% |  |  | 96.5% |  |  |
| Dec Time[%] | 99.9% |  |  | 96.2% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | -0.9% | -0.9% |  |  |  |
| Class B | -0.9% | -1.5% | -1.5% | -1.1% | -1.6% | -1.8% |
| Overall (EL+BL) | -0.7% | -1.3% | -1.3% | -1.1% | -1.6% | -1.8% |
| Overall (EL) | -1.2% | -2.2% | -2.3% | -2.4% | -3.4% | -3.7% |
| Enc Time[%] | 99.7% |  |  | 97.3% |  |  |
| Dec Time[%] | 97.8% |  |  | 97.0% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

Table 1: Results of TE5.2.6 with the motion vector compression for the temporal motion vector compared to the SMuC 0.1.1

Combination with TE5.3.3

The following results are provided to demonstrate the good synergy of the interlayer motion vector prediction presented in this contribution in combination with the Base Mode presented in JCTVC-L0102  [4]. These results are presented by activating the interlayer motion vector prediction method and the Base Mode by using the SMuC 0.1.1 as reference. These results have been cross-checked.

Table 2: Results on SMuC 0.1.1

The following table shows the gain of the method when the reference is the SMuC 0.1.1 + the Base Mode. This demonstrates the good synergy between the Inter-layer motion prediction and the Base Mode.

Table 3: Results over SMuC 0.1.1 with Base Mode of [5]

In addition to these results, the following table shows the performances of the proposed test modified in order that the spatial merge candidate cannot be obtained from the block coded with Base Mode. This offers a complexity reduction of the merge mode derivation process. Compared to the results of previous table, it is shows that it doesn’t affect the result of the previous experiment.

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -3.9% | -8.8% | -8.4% |  |  |  |
| Class B | -3.5% | -5.7% | -6.1% | -4.3% | -6.8% | -7.6% |
| Overall (EL+BL) | -3.6% | -6.6% | -6.8% | -4.3% | -6.8% | -7.6% |
| Overall (EL) | -6.9% | -12.3% | -12.6% | -11.4% | -17.6% | -19.4% |
| Enc Time[%] | 101.2% |  |  | 98.4% |  |  |
| Dec Time[%] | 128.0% |  |  | 145.7% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -2.5% | -6.0% | -5.5% |  |  |  |
| Class B | -2.8% | -3.9% | -4.0% | -3.4% | -4.8% | -5.0% |
| Overall (EL+BL) | -2.7% | -4.5% | -4.4% | -3.4% | -4.8% | -5.0% |
| Overall (EL) | -5.0% | -8.2% | -8.1% | -7.9% | -11.8% | -12.3% |
| Enc Time[%] | 101.8% |  |  | 98.6% |  |  |
| Dec Time[%] | 125.1% |  |  | 145.3% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

Table 4: Results on SMuC 0.1.1

# Conclusion

This contribution reports the performance of Inter-layer motion prediction tool initially presented in contribution K0041 for the Call for Proposal of Scalable technologies.

The average BDR performance gain of the inter-layer motion vector prediction method is about -1.6%(Y), -2.3%(U) and -2.5% (V) under the TE5 testing conditions over the SMuC 0.1.1. When combining with the Base Mode method proposed in JCTVC-L0102 [4] the performance are about -3.6%(Y), -5.9%(U), -6.3%(V).

# References

- Vadim Seregin, Patrice Onno, Shan Liu, Tammy Lee, Chulkeun Kim, Haitao Yang, Haricharan Laksman, “Description of Tool Experiment C5: Inter-Layer syntax prediction using the HEVC base layer”, Joint Collaborative Team on Video Coding, JCTVC-K1105, October, 2012.

- Sébastien Lasserre, Fabrice Le Léannec, Jonathan Taquet, Naël Ouedraogo, Stéphane Pautet, Christophe Gisquet, Guillaume Laroche, Tangi Poirier, Yann Verdavaine, Edouard Francois, Eric Nassor, “Description of the scalable video coding technology proposal by Canon Research Centre France” Joint Collaborative Team on Video Coding, JCTVC-K0041, October, 2012.

- ITU-T Recommendation H.264, “Advanced Video Coding for Generic Audiovisual Services,” March, 2010.

- Christophe Gisquet, Tangi Poirier, Jonathan Taquet, Fabrice Le Léannec, Edouard François, Guillaume Laroche, Patrice Onno, “TE5: Results of test 5.3.3 on the Base Mode”, Joint Collaborative Team on Video Coding, JCTVC-L0102, January, 2013.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).