| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0102 |
| --- | --- |

| Title: | TE5: Results of test 5.3.3 on the Inter-Layer Prediction Mode (Base Mode) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Christophe Gisquet Tangi Poirier Jonathan Taquet Fabrice Le Léannec Edouard François Guillaume Laroche Patrice Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33(0)299876800 christophe.gisquet@crf.canon.fr fabrice.leleannec@crf.canon.fr edouard.francois@crf.canon.fr guillaume.laroche@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon Research Centre France |  |  |

_____________________________

# Abstract

This contribution provides the results of the test 5.3.3 of TE5 related to Inter-layer inferred prediction mode in SHVC. More precisely, this contribution is about the Base Mode method presented in JCTVC-K0041 and integrated in the SMuC version 0.1.1 for further evaluation. It is reported that the Y-BDR performance of the Base Mode method respectively brings -3.2%(RA 2x), -3.7%(RA 2x), -3.4%(RA SNR) and -2.5%(LD-P 2x), -3.0%(LD-P 1.5x), -3.3%(LD-P SNR) over the SMuC 0.1.1.

# Introduction

In JCTVC-K0041 [2], an inter-layer prediction mode is proposed for the coding/decoding of enhancement inter picture. This mode, called the “Base Mode” prediction mode consists in predicting an entire enhancement layer CU from some prediction texture data coming from the base layer. The Base Mode prediction data is computed through:

- either temporal prediction of derived Inter CUs (with derived BL motion info and residual prediction)

- or Intra BL prediction derived from Intra CUs, in combination with the compensation of enhancement reference pictures.

The Base Mode prediction mode is signaled at the CU level in the enhancement bitstream. In the case of SNR scalability, this inferred prediction information corresponds to the Coding Unit structure of the base picture, taken as is, before the motion information compression step performed in the base layer. In the case of spatial scalability, the base layer prediction information first undergoes a so-called prediction information up-sampling process. Two distinct Inter-layer predictions of CU/PU/TU partitions are used for spatial ratio 2x and for ratio 1.5x as described in the next section.

# Base Mode tool description

## Base Mode principle

The “Base Mode”  presented in  this test 5.3.3 for TE5 consists in predicting the current CU in the enhancement layer by applying a motion compensation using the motion information (motion vector, reference list, reference index, …) of the collocated base layer CU. Motion vectors are scaled to match the spatial resolution change. In this Base Mode, we are also considering the addition of a residual data. The Base Mode can be summarized by the following formula:

PREEL = MC{ REFEL , SP_ratio*MVBL } + RES,

where

- MC{I, MV} corresponds to the motion compensation operator (using DCT-IF filters) with motion vector MV  and using as reference picture the picture I.

- REFEL is the reference picture for the enhancement layer

- MVBL is the motion vector of the base layer

- SP_ratio is the spatial ratio between the base layer and the enhancement layer and

- And RES is the decoded residual of the corresponding CU in the base layer and is computed as follows:

RES = UPS{RECBL} – MC{UPS{REFBL}, SP_ratio*MVBL }}

where UPS{.} is the up-sampling operator (using DCT-IF filters) and RECBL is the reconstructed base layer CU.

Contrary to the SVC specifications, the temporal residual texture coded and decoded in the base layer is not inherited from the base picture. For the proposed “Base Mode”, the residual RES is based on the GRILP concept [4] and is used for the prediction of the enhancement layer.

## Inter-layer prediction of CU/TU/PU partitions for the dyadic case

Figure 1 depicts the prediction information up-sampling process, performed both by the encoder and the decoder in order to construct the “Base Mode” prediction picture when the spatial ratio between the enhancement and the base layer is equal to 2.

The left side of Figure 1 illustrates a part of the base layer picture. In particular, the Coding Unit representation that has been used to encode the base picture is illustrated, for the two first CTB (Code Tree Block) of the picture. The Coding Unit quad-tree representation of the second CTB is represented, as well as their PU partitions. Moreover, the motion vector associated to each PU is depicted.

On the right side of Figure 1, the result of the prediction up-sampling process can be seen, in case of dyadic spatial scalability. On this figure, the CTB size is the same in the enhancement picture and in the base picture. As can be seen, the up-sampled version of base CTB 1 results in the enhancement CTB 2, 3, 6 and 7. Moreover, the Coding Unit quad-tree has been re-sampled as a function of the scaling ratio that exists between the enhancement picture and the base picture. The Prediction Unit partitioning is of the same type (i.e. PU’s have the same shape) in the enhancement layer and in the base layer. Finally, motion vector coordinates have been re-scaled by 2 as a function of the spatial ratio between the two layers.

In other words, three main steps are involved in the prediction information up-sampling process.

- The Coding Unit quad-tree representation is first up-sampled. To do so, the depth parameter of the base Coding Unit is decreased by 1 in the enhancement layer.

- The Coding Unit partitioning mode is kept the same in the enhancement layer, compared to the base layer. This leads to Prediction Units that have an up-scaled size in the enhancement layer, and have the same shape as their corresponding PU in the base layer.

- The motion vector is re-sampled to the enhancement layer resolution, simply by multiplying their x and y coordinates by the appropriate scaling ratio.

Figure 1: Prediction information up-sampling in dyadic case

Regarding the Coding Unit depth information, Figure 2 illustrates how the HEVC Coding Units are up-sampled from a base layer towards a spatial enhancement layer, in the case of dyadic spatial scalability.

Each Coding Unit has an associated depth level in the quad-tree representation of base CTBs. The up-sampling of Coding Units consists in the following method. For a given Coded Tree Block in the enhancement layer, the Coding Unit that spatially corresponds to that enhancement CTB is searched in the base picture. The enhancement CTB is then given Coding Unit (CU) depths values that are equal to the depth values contained in the found base Coding Units, decreased by 1. These decreased depth values then provide a quad-tree that corresponds to the quad-tree in the base picture, up-sampled by 2 in width and height. In case the base CU depth is equal to 0, then the derived CU depth is also equal to zero (same LCU size in base and enhancement layers).

Figure 2: CU depth information in case of dyadic up-sampling

Concerning the up-sampling of PUs partition type in dyadic case, the Inter-layer derivation is straightforward in the general case, and simply consists in providing the enhancement CU with the same PU partition as the co-located base CU.

### AMP handling

Figure 3 illustrates the method proposed in order to up-sample asymmetric vertical Prediction Unit partitions for the dyadic case. The similar method for asymmetric horizontal partition is described in Error: Reference source not found.The goal of the derivation process applied on PU partition types is to obtain a partitioning into PUs in the enhancement layer that conforms to that of the base layer, in terms of PUs shapes.

The proposed mechanism up-samples base Prediction Units, according to the following policy. For each up-sampled Coding Unit in the enhancement layer, the following applies.

- If the co-located CU in the base layer has a depth strictly higher than 0, then the Prediction Unit type in the enhancement CU is set equal to the Prediction Unit type in the base layer.

- If the base CU has a depth value equal to 0, i.e. the CU has the maximum allowed size, then the following applies.

  - If the co-located base CU has a symmetric Prediction Unit partition, then the enhancement CU is given the 2Nx2N Prediction Unit type. Such case is illustrated on the left of Figure 3. The reason for that is that the base CU, when up-sampled to the enhancement layer resolution, spatially covers four LCU in the enhancement picture.

  - If the co-located base CU has an asymmetric Prediction Unit type, then the enhancement CU is assigned a Prediction Unit type that depends on its spatial position in the enhancement layer, and on the base asymmetric Prediction Unit. This is illustrated on the middle of Figure 3. Indeed, on the example of Figure 3, the base CU has a PU type equal to nLx2N. The spatial up-sampling of the base CU covers 4 LCUs in the enhancement layer. As the goal of the PU up-sampling is to preserve the spatial geometry of the base layer, the enhancement PU assignment is done as follows. Enhancement LCU’s that have an even x index (left side of the diagram) are given a symmetric PU type equal to Nx2N. Enhancement LCUs that have an odd x index (right side of the diagram) are given the 2Nx2N PU type.

Figure 3: example of Asymmetric Motion vertical and horizontal Partition up-sampling for the dyadic case

The result of our proposed method is that the up-sampled prediction information in our method better represents the motion contained in the considered video sequence.

## Inter-layer prediction of CU/TU/PU partitions in case of a spatial ratio 1.5

The goal of Inter-layer prediction information derivation is to keep as much accuracy as possible in the up-scaled Prediction Unit and motion information, in order to generate a Base Mode prediction picture which that is as good as possible.

However, in the case of spatial scalability with ratio 1.5, the block-to-block correspondence between the base picture and the enhancement picture differs from the dyadic case. Therefore, a straight-forward prediction information up-scaling method as that illustrated by Error: Reference source not found does not seem feasible in the case of ratio 1.5.

The method adopted to derive prediction information in case of ratio 1.5 consists in the following, simpler, approach.

- First, each Largest Coding Unit (LCU) in the enhancement picture, the LCU is split into CUs with minimum size (4x4).

- Next, each so-obtained 4x4 is given the Prediction Unit type 2Nx2N.

- Finally, the prediction information of each obtained 4x4 Prediction Unit is computed as a function of prediction information associated to the co-located area in the base picture. The prediction information derived from the base picture include the following:

  - Prediction mode,

  - Merge information,

  - Intra prediction direction (if relevant),

  - Inter direction,

  - Cbf values,

  - Partitioning information,

  - CU size,

  - Motion vector prediction information,

  - Motion vector values (scaled by 1.5).

  - Reference picture indices

Therefore, each CTB of the enhancement picture is organized regardless the way the corresponding LCU in the base picture have been quad-tree represented.

# Results

The testing conditions are those defined by TE5 description document [1]. The version of the SMuC is 0.1.1. The encoding and decoding runtime are reliable.

# Conclusion

This contribution reports the performance of the Base Mode tool initially presented in contribution K0041 for the Call for Proposal of Scalable technologies. The average BDR performance gain of the Base Mode method is about -3,2%(Y), -4,9%(U), -5,2%(V) under the TE5 testing conditions over the SMuC 0.1.1.

# References

- Vadim Seregin, Patrice Onno, Shan Liu, Tammy Lee, Chulkeun Kim, Haitao Yang, Haricharan Laksman, “Description of Tool Experiment C5: Inter-Layer syntax prediction using the HEVC base layer”, Joint Collaborative Team on Video Coding, JCTVC-K1105, October, 2012.

- Sébastien Lasserre, Fabrice Le Léannec, Jonathan Taquet, Naël Ouedraogo, Stéphane Pautet, Christophe Gisquet, Guillaume Laroche, Tangi Poirier, Yann Verdavaine, Edouard Francois, Eric Nassor, “Description of the scalable video coding technology proposal by Canon Research Centre France” Joint Collaborative Team on Video Coding, JCTVC-K0041, October, 2012.

- ITU-T Recommendation H.264, “Advanced Video Coding for Generic Audiovisual Services,” March, 2010.

- Christophe Gisquet, Fabrice Le Léannec, Jonathan Taquet, Edouard François, Guillaume Laroche, Patrice Onno, , “TE3: Results of test 4.6.1 on the Generalized Residual Inter-Layer Prediction (GRILP)”, Joint Collaborative Team on Video Coding, JCTVC-L0100, January, 2013.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).