| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0103 |
| --- | --- |

| Title: | Non-TE5.3: Inter-layer inferred prediction mode and motion compensation restrictions in SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Christophe Gisquet Edouard François Guillaume Laroche Patrice Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33(0)299876800 christophe.gisquet@crf.canon.fr edouard.francois@crf.canon.fr guillaume.laroche@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon Research Centre France |  |  |

_____________________________

# Abstract

In the scope of TE5, section 5.3, Inter-layer inferred prediction mode (InterBL mode), several proposals were evaluated, among which Canon’s proposal, originating for the Call for Proposal response JCTVC-K0041. It has been noted that motion compensation in Core HEVC introduces restrictions (e.g. related to the minimum size of an Inter PU). Considering similar restrictions for the Enhancement layer may impact how the reference layer syntax can be inherited.

This document therefore presents how the syntax derivation for InterBL mode can be performed on an 8x8 basis. In addition it provides results when using bilinear motion compensation filters. It is reported that the Y-BDR performance of the overall method respectively brings -3.3%(RA 2x), -3.5%(RA 1.5x), -3.7%(RA SNR) and -3.4%(LD-P 2x), -3.6%(LD-P 1.5x), -3.8%(LD-P SNR) over SMuC 0.1.1.

# Complexity considerations

## Inter PU size restriction

Different contributions to HEVC such as [1] have introduced restrictions on Inter PU size, which currently are:

- A 8x8 uni-predictive CU can only be partitioned as Nx2N or 2NxN, i.e. 8x4 or 4x8

- A 8x8 bi-predictive CU can only be partitioned

It was noted that, however, upscaling of the motion information for the enhancement layer motion prediction could cause issues, as can be visualized on the figure below.

Figure 1: Example of syntax derivation, taken from [2]

If we imagine b0, b1, b2 and b3 in the reference layer to be bi-predictive 8x8 PUs with strictly different motion, as permitted by the HEVC standard, we notice that the motion mapping may lead to the enhancement layer 8x8 PU u4 to potentially have 4 subblocks each with different motion. This is not permitted by the Core HEVC specification.

During TE5, section 5.3, discussions, and as presented in JCTVC-K0036 [3] and described in section 5.3.1 of TE5, Qualcomm chose to solve this issue by resampling reference layer syntax on a 8x8 basis: the motion buffer position for upper left part of u4 is taken as reference, and its co-located position in the reference layer motion buffer is retrieved. The motion at that position is then used for all of u4.

## Residual inheritance

In the scope of TE5.3 and JCTVC-K0041, we have proposed a base mode that uses Generalized Residual Inter-Layer Prediction (GRILP)-like techniques. GRILP mode consists in generating, given a Prediction Unit and motion vector MV being considered, a corresponding temporal residual PU in the base layer that is added to enhancement layer temporal prediction PU.

The prediction signal is derived as follows (shown for the uni-prediction case, the concept is easily generalized for the bi-prediction case):

PREDEL = MC[REFEL, MVRL] + UP{RECBL} - MC[UP{REFBL}, MVRL]

where PREDEL is the prediction signal of the enhancement layer, REFEL and REFBL are the reference pictures in the Enhancement and Base layers, RECBL is the base layer reconstructed picture corresponding to the current enhancement layer picture, MVRL is the motion vector inherited (and scaled) from the reference layer, UP{.} is the up-sampling operator, MC[I,MV] is the motion compensation step of the current picture using I as reference picture and MV for the motion vector.

The principle of the GRILP method can be summarized by the following Figure 2.

Figure 2: Schematic principle of the GRILP mode

One issue is the complexity of inheriting the BL temporal residual, as it involves additional motion compensation filter complexity. JCTVC-K0041 and K0362 were using the classical DCT-IF ones.

## Implementation efficiency

Canon’s contribution to TE5.3 was using a frame-based solution, and thus, even at the decoder, the whole frame was rebuilt, leading to potential runtime inefficiency. The software tested here does not have such an issue and the decoder runtime increase is mostly due to:

- Doing 8x8 MCs whatever the actual content of the base PU

- GRILP MC complexity

# Proposed solutions

## Presentation

### Position used to inherit the motion vector

We simply suggest using the bottom-right position instead of the top left, as it will probably yield a better vector for motion prediction of the right and bottom PUs (not yet coded). The loss of summarizing the motion in 8x8 block, as also noticed by Qualcomm, is small, so it seems unnecessary to use more complex inheriting schemes.

### Motion compensation filters

We propose to use bilinear interpolation filters for the motion compensation of both the EL and BL, as opposed to the usual HEVC luma’s 8-taps DCT-IF and the chroma’s 4-taps DCT-IF.

## Results

In this test, the up-sampling interpolation filters are those defined in the TE 2, i.e. classical DCT-IF ones.

The results of the interlayer tool with DCT-IF motion compensation filters and a 4x4 syntax upscaling are (for BDR reference, as the runtimes were not reliable):

The results for the updated tool are:

We would like to express our thanks to Huawei for cross-checking those results.

# Conclusion

This contribution proposes a simplification of the TE5, section 5.3.3 solution. It consists in using an 8x8-based BL syntax inheriting and bilinear MC filters in place of a 4x4-based inheriting and DCT-IF MC filters. It results in an average BDR performance of -3.5%(Y), -4.9%(U), -5.2%(V), i.e. to a gain of 0.3% BDR-Y compared to the TE5, section 5.3.3, proposal.

In light of the limited increase of complexity and the important gains, we recommend adopting the tool of TE5, section 5.3.3 with the proposed simplifications into the HEVC based scalable extension.

5.3.3, prwe did not

with the proposed simplifications into the HEVC based scalable extension.

References

- S. Fukushima, M. Ueda, and H. Takehara, “AHG7: Bi-pred restriction for small PUs”, JCTVC-I0297, 9th Meeting: Geneva, CH, 27 April – 7 May 2012

- J. Dong, Y. He, Y. He, G. McCllelan, E.-S. Ryu, X. Xiu, Y. Ye, “Description of scalable video coding technology proposal by InterDigital Communications”, JCTVC-K0034, 11th Meeting: Shangai

- J. Chen, K. Rapaka, X. Li, V. Seregin, L. Guo, M. Karczewicz, G. Van der Auwera, J. Sole, X. Wang, C. J. Tu, Y. Chen, “Description of scalable video coding technology proposal by Qualcomm (configuration 2)”, JCTVC- K0036, 11th Meeting: Shangai

- Heiko Schwarz, Detlev Marpe, and Thomas Wiegand, “Overview of the Scalable Video Coding Extension of the H.264/AVC Standard”, IEEE Trans. On Circuits and Systems for Video Technology, Vol.17, No.9, September 2007

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).