| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0104 |
| --- | --- |

| Title: | Non-TE3: Simplification of Generalized Residual Inter-Layer Prediction (GRILP) in SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Edouard François Jonathan Taquet Christophe Gisquet Guillaume Laroche Patrice Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33(0)299876800 edouard.francois@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution proposes a simplification of the Generalized Residual Inter-Layer Prediction (GRILP), which is an inter prediction mode comprising a second order prediction based on the BL temporal residual. The proposed simplification consists in directly accessing the temporal reference BL data instead of the upsampled temporal reference BL data to generate the second order residual. The solution is evaluated in two different implementations (one from Nokia, one from Qualcomm). In the Nokia implementation, it is reported that the Y-BDR performance of the proposed solution respectively gives -2.1% (RA 2x), -3.2% (RA 2x), -2.4% (LD-P 2x), -3.4% (LD-P 1.5x) over the SMuC 0.1.1. In the Qualcomm implementation, it is reported that the Y-BDR performance of the proposed solution respectively gives -x.x% (RA 2x),         -x.x% (RA 2x), -2.7% (LD-P 2x), -3.7% (LD-P 1.5x) over the SMuC 0.1.1.

# Introduction

This contribution proposes a simplification of the Generalized Residual Inter-Layer Prediction (GRILP).  The GRILP mode is an inter prediction mode proposed in T3.4 by Canon, Nokia and Qualcomm. This mode consists in applying a second order prediction, using the Base Layer temporal residual derived from the scaled EL motion vectors. The prediction signal is derived as follows (shown for the uni-prediction case, the concept is easily generalized for the bi-prediction case):

PREDEL = MC[REFEL, MVEL] + w ( UP{RECBL} - MC[UP{REFBL}, MVEL] )

where PREDEL is the prediction signal of the enhancement layer, REFEL and REFBL are the temporal reference pictures in the Enhancement and Base layers, RECBL is the base layer reconstructed picture corresponding to the current enhancement layer picture, MVEL is the motion vector selected, UP{.} is the up-sampling operator, MC[I,MV] is the motion compensation step of the picture using I as reference picture and MV for the motion vector. w is a weighting factor possibly applied to the second order residual.

The principle of the GRILP mode can be summarized by the following Figure 1.

Figure 1: Schematic principle of the GRILP mode

# Proposed GRILP simplification

The goal of the proposed simplification is to reduce the memory needs by directly accessing the temporal reference BL data instead of applying the motion compensation step to the upsampled temporal reference BL data. The simplification also enables reducing the number of required upsampling or motion compensation operations.

To this end, the Upsampling and Motion Compensation steps applied to the temporal reference BL data are combined into one single operation, noted MCoUP in the following equation:

PREDEL = MC[REFEL, MVEL] + w ( UP{RECBL} - MCoUP{REFBL, MVEL/ratio] )

where MVEL/ratio is the EL motion vector rescaled to the BL resolution.

The modified GRILP mode is schematically represented in the Figure 2.

Figure 2: Schematic principle of the proposed GRILP mode simplification

The accuracy of the rescaled motion vector is defined as follows:

- In dyadic spatial scalability (ratio 2x)

  - 1/8th pixel for luma samples

  - 1/16th pixel for chroma samples

- In spatial scalability with inter-layer ratio of 3/2 (ratio 1.5x)

  - 1/6th pixel for luma samples

  - 1/12th pixel for chroma samples

For luma, 8-taps 6-bit filters are used. For chroma, 4-taps 6-bit filters are used. Filters for phases 0 and ½ are the DCT-IF based filters specified in the Core HEVC specification. For the other phases, a bilinear interpolation from these 2 filters applies. These filters have turned out to perform better than filters fully generated by the DCT-IF approach for all phases.

The corresponding filters, specified on 6-bits, are provided in annex.

# Results

The testing conditions are those defined by TE3 description document [1]. The version of the SMuC is 0.1.1.

The simplification has been implemented in two software versions:

- The GRP implementation proposed by Nokia for the TE3 4.6.3

- The GRP implementation proposed by Qualcomm for the TE3 4.6.2 (test 2)

Full results are provided in the attached xls files.

Results in Nokia’s implementation (from TE3 4.6.3)

The summarized results of the table below show that the proposal improves the TE3 4.6.3 solution by 0.4% for RA 2x, 0.7% for RA 1.5x, 0.1% for LDP 2x and 0.0% for LDP 1.5x cases.

|  | TE3 4.6.3 |  |  |  |  | Proposal |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA-2x | -1.7% | -3.6% | -3.8% | 122% | 103% | -2.1% | -3.3% | -3.3% | 126% |  |
| RA-1.5x | -2.5% | -4.5% | -4.9% | 113% | 110% | -3.2% | -4.2% | -4.5% | 120% |  |
| LDP-2x | -2.3% | -2.9% | -2.8% | 117% | 103% | -2.4% | -2.0% | -1.7% | 122% |  |
| LDP-1.5x | -3.4% | -3.8% | -3.6% | 109% | 100% | -3.4% | -2.9% | -2.5% | 114% |  |

Results in Qualcomm’s implementation (from TE3 4.6.2 – test2)

The summarized results of the table below show that compared to the TE3 4.6.2 test2 solution, a difference of x.x% for RA 2x and of x.x% for RA 1.5x, 0.0% for LDP-2x and 0.2% for LDP-1.5x is observed.

|  | TE3 4.6.2 test2 |  |  |  |  | Proposal |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA-2x | -1.9% | -4.0% | -4.2% | 119% | 105% |  |  |  |  |  |
| RA-1.5x | -2.8% | -5.2% | -5.6% | 111% | 103% |  |  |  |  |  |
| LDP-2x | -2.7% | -3.4% | -3.0% | 124% | 102% | -2.7% | -1.9% | -1.5% | 124% |  |
| LDP-1.5x | -3.9% | -4.3% | -3.5% | 113% | 101% | -3.7% | -2.8% | -1.9% | 114% |  |

# Conclusion

This contribution proposes a simplification of the GRILP mode in order to reduce the required memory need to access the BL reference data. The second order prediction is achieved by accessing directly the temporal reference BL data at the BL resolution, instead of accessing the upsampled temporal reference BL data at the EL resolution.

In the Nokia implementation, the Y-BDR performance of the proposed solution respectively gives -2.1% (RA 2x), -3.2% (RA 2x), -2.4% (LD-P 2x), -3.4% (LD-P 1.5x) over the SMuC 0.1.1.

In the Qualcomm implementation, the Y-BDR performance of the proposed solution respectively gives    -x.x% (RA 2x), -x.x% (RA 2x), -2.7% (LD-P 2x), -3.7% (LD-P 1.5x) over the SMuC 0.1.1.

# References

- X. Li, E. Francois, P. Lai, D. Kwon, A. Saxena, “Description of Tool Experiment B3: Combined Prediction in SHVC”, Joint Collaborative Team on Video Coding, JCTVC-K1103, October, 2012.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex

The interpolation filters used in the proposal are given below. They are based on the Core HEVC DCT-IF filters of phases 0 and 1/2. The other phases are generated from these 2 given filters by bilinear interpolation, constrained to be defined on 6-bits. Default DCT-IF based filters are indicated in bold red font.

Interpolation filters Luma ratio 2x

{  0, 0,   0, 64,  0,   0, 0,  0 },

{  0, 1,  -3, 58, 10,  -3, 1,  0 },

{ -1, 2,  -6, 53, 21,  -6, 2, -1 },

{ -1, 3,  -8, 46, 30,  -8, 3, -1 },

{ -1, 4, -11, 40, 40, -11, 4, -1 },

{ -1, 3,  -8, 30, 46,  -8, 3, -1 },

{ -1, 2,  -6, 21, 53,  -6, 2, -1 },

{  0, 1,  -3, 10, 58,  -3, 1,  0 }

Interpolation filters Chroma ratio 2x

{  0, 64,  0,  0},

{ -1, 61,  5, -1},

{ -1, 57,  9, -1},

{ -2, 54, 14, -2},

{ -2, 50, 18, -2},

{ -3, 47, 23, -3},

{ -3, 43, 27, -3},

{ -4, 40, 32, -4},

{ -4, 36, 36, -4},

{ -4, 32, 40, -4},

{ -3, 27, 43, -3},

{ -3, 23, 47, -3},

{ -2, 18, 50, -2},

{ -2, 14, 54, -2},

{ -1,  9, 57, -1},

{ -1,  5, 61, -1}

Interpolation filters Luma ratio 1.5x

{ 0, 0,   0, 64,  0,   0, 0,  0},

{ 0, 1,  -4, 57, 13,  -4, 1,  0},

{-1, 3,  -7, 48, 26,  -7, 3, -1},

{-1, 4, -11, 40, 40, -11, 4, -1},

{-1, 3,  -7, 26, 48,  -7, 3, -1},

{ 0, 1,  -4, 13, 57,  -4, 1,  0}

Interpolation filters Chroma ratio 1.5x

{  0, 64,  0,  0},

{ -1, 60,  6, -1},

{ -1, 54, 12, -1},

{ -2, 50, 18, -2},

{ -3, 46, 24, -3},

{ -3, 40, 30, -3},

{ -4, 36, 36, -4},

{ -3, 30, 40, -3},

{ -3, 24, 46, -3},

{ -2, 18, 50, -2},

{ -1, 12, 54, -1},

{ -1,  6, 60, -1}