| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0105r1 |
| --- | --- |

| Title: | Non-TE5.1: MPM derivation and coding in SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Edouard François ShaSha Shi Christophe Gisquet Guillaume Laroche Patrice Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33(0)299876800 edouard.francois@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution proposes a modified MPM derivation and Intra Mode coding tree exploiting the intra mode information from the reference Base Layer. It is reported that the Y-BDR performance of the proposed method are of -0.4%(AI 2x), -0.2%(AI 1.5x) over the SMuC 0.1.1 (BL+EL rate) when MDCS is used in the EL, and -0.3%(AI 2x), -0.2%(AI 1.5x) over the SMuC 0.1.1 (BL+EL rate) when MDCS is not used in the EL.

# Introduction

The BL mode of the co-located CU of an EL CU is very often relevant for the EL CU. Consequently the EL intra mode coding process can benefit from this information. In this proposal, two changes of the EL intra mode coding processes are proposed:

- The MPM derivation is modified by always setting the MPM0 to the co-located BL block (BL mode) when this BL block is intra coded;

- The intra mode coding tree is modified to favor more the MPM0.

These 2 modifications are detailed in the following sections.

# MPM derivation

The modified MPM derivation works as follows. If the co-located BL block is not intra, the normal MPM derivation process applies. Otherwise, the MPM0 is set to the BL mode. The 2 other MPMs are then derived from the left and top CU modes (noted as candA and candB) using the following pseudo-code algorithm:

If !(BL CU is available and intra and intraModeBL > 1 and intraModeBL < 34)

apply usual MPM derivation

Otherwise

MPM0 = modeBL

If candA != MPM0		MPM1 = candA

If candB != MPM0 && candB != MPM1	MPM2 = candB

Else 					go to Step1

Else If candB != MPM0	MPM1 = candB and go to Step1

Else				derive MPM1 and MPM2 as the 2 neighbor modes of MPM0

Step1

If MPM1 < MPM0		derive MPM2 as the larger neighbor mode of MPM0

Else				derive MPM2 as the lower neighbor mode of MPM0

# Intra mode coding tree

In order to favor more the BL mode, which is often the MPM0, the modified coding tree is as follows.

# Results

The testing conditions are those defined for TE5.1 ‘Inter-layer intra mode prediction’ [1]. The reference version is the SMuC 0.1.1.

The solution is tested in two configurations: with MDCS on, and with MDCS off.

The summarized results below are provided for AI-2x and AI-1.5x configurations. Full results are provided in the attached xls files.

MDCS On

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.30% | -0.11% | -0.01% |  |  |  |
| Class B | -0.45% | -0.28% | -0.29% | -0.17% | 0.08% | 0.06% |
| Overall (EL+BL) | -0.40% | -0.23% | -0.21% | -0.17% | 0.08% | 0.06% |
| Overall (EL) | -0.71% | -0.33% | -0.28% | -0.42% | 0.29% | 0.24% |
| Enc Time[%] | 103.4% |  |  | 103.0% |  |  |
| Dec Time[%] | 100.5% |  |  | 100.0% |  |  |

MDCS Off

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.24% | 0.02% | 0.10% |  |  |  |
| Class B | -0.28% | -0.05% | 0.01% | -0.15% | 0.11% | 0.11% |
| Overall (EL+BL) | -0.27% | -0.03% | 0.04% | -0.15% | 0.11% | 0.11% |
| Overall (EL) | -0.49% | -0.01% | 0.11% | -0.38% | 0.32% | 0.36% |
| Enc Time[%] | 103.2% |  |  | 102.9% |  |  |
| Dec Time[%] | 100.3% |  |  | 100.2% |  |  |

## Additional results

We also provide below the results when using 2 additional changes that are also used in TE5.1.1 proposal:

- The considered BL CU is the one co-located with the center of the EL block

- If the neighboring EL CU is coded as intra BL, its mode for MPM derivation is inherited from the BL CU when this BL CU is intra

The corresponding results are given below. The gain of the additional tricks is around 0.01% in AI-2x, and 0.03% in AI-1.5x.

MDCS On

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.30% | -0.10% | -0.02% |  |  |  |
| Class B | -0.46% | -0.26% | -0.28% | -0.20% | 0.08% | 0.06% |
| Overall (EL+BL) | -0.41% | -0.21% | -0.20% | -0.20% | 0.08% | 0.06% |
| Overall (EL) | -0.73% | -0.30% | -0.28% | -0.49% | 0.29% | 0.23% |
| Enc Time[%] | 101.2% |  |  | 100.6% |  |  |
| Dec Time[%] | 100.4% |  |  | 99.2% |  |  |

MDCS Off

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.25% | 0.03% | 0.10% |  |  |  |
| Class B | -0.29% | -0.03% | 0.03% | -0.18% | 0.12% | 0.13% |
| Overall (EL+BL) | -0.28% | -0.01% | 0.05% | -0.18% | 0.12% | 0.13% |
| Overall (EL) | -0.51% | 0.01% | 0.13% | -0.45% | 0.36% | 0.39% |
| Enc Time[%] | 100.4% |  |  | 100.6% |  |  |
| Dec Time[%] | 99.8% |  |  | 99.5% |  |  |

## Comparison with TE5.1 results

The following table summarizes the results of the proposal in comparison with the results provided in TE5.1.

|  | parsing | AI HEVC 2x (EL) |  |  | AI HEVC 1.5x (EL) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dependency | Y | U | V | Y | U | V | avg Y |
| 5.1.5.b | Yes | -0.05 | -0.07 | -0.07 | -0.01 | -0.02 | -0.01 | -0.03 |
| 5.1.5.a | Yes | -0.13 | -0.08 | -0.07 | -0.05 | 0.02 | 0.02 | -0.09 |
| 5.1.2.a | Yes | -0.25 | -0.13 | -0.12 | -0.10 | 0.07 | 0.06 | -0.18 |
|  | Yes | -0.27 | -0.13 | -0.11 | -0.11 | 0.08 | 0.06 | -0.19 |
| 5.1.4 | Yes | -0.28 | -0.13 | -0.11 | -0.11 | 0.08 | 0.06 | -0.20 |
| 5.1.5.c | Yes | -0.32 | -0.20 | -0.17 | -0.15 | 0.08 | 0.06 | -0.24 |
| 5.1.1.a | Yes | -0.31 | -0.18 | -0.17 | -0.17 | 0.05 | 0.04 | -0.24 |
| 5.1.3.a | Yes | -0.36 | -0.24 | -0.20 | -0.13 | 0.07 | 0.07 | -0.25 |
| proposal | Yes | -0.40 | -0.23 | -0.21 | -0.17 | 0.08 | 0.06 | -0.29 |
| proposal+MTK tricks | Yes | -0.41 | -0.21 | -0.20 | -0.20 | 0.08 | 0.06 | -0.31 |
| 5.1.2.b | No | -0.13 | 0.06 | 0.12 | -0.09 | 0.09 | 0.10 | -0.11 |
| 5.1.5.d | No | -0.20 | 0.00 | 0.08 | -0.13 | 0.09 | 0.11 | -0.17 |
| 5.1.1.b | No | -0.19 | 0.02 | 0.08 | -0.15 | 0.08 | 0.09 | -0.17 |
| 5.1.3.b | No | -0.24 | -0.04 | 0.04 | -0.12 | 0.09 | 0.11 | -0.18 |
| proposal | No | -0.27 | -0.03 | 0.04 | -0.15 | 0.11 | 0.11 | -0.21 |
| proposal+MTK tricks | No | -0.28 | -0.01 | 0.05 | -0.18 | 0.12 | 0.13 | -0.23 |

# Conclusion

In this contribution, a modified MPM derivation and intra mode coding tree is proposed. The main idea is to favour the BL mode (when the co-located BL CU is intra coded) by setting the MPM0 to the BL mode and using 1 bit to signal if the intra mode is MPM0 or not. The Y-BDR performance are of -0.4%(AI 2x), -0.2%(AI 1.5x) over the SMuC 0.1.1 (BL+EL rate) when MDCS is used in the EL, and -0.3%(AI 2x), -0.2%(AI 1.5x) over the SMuC 0.1.1 (BL+EL rate) when MDCS is not used in the EL.

# References

- V. Seregin, P. Onno, S. Liu, T. Lee, C. Kim, H. Yang, “Description of Tool Experiment C5: Inter-layer syntax prediction using HEVC base layer”, Joint Collaborative Team on Video Coding, JCTVC-K1105, October, 2012.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).