| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0106 |
| --- | --- |

| Title: | Non-TE5.1: simplification of remaining modes coding in SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Edouard François ShaSha Shi Christophe Gisquet Guillaume Laroche Patrice Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33(0)299876800 edouard.francois@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution proposes a modified remaining mode coding for the Enhancement Layer, by reducing the number of remaining modes to a limited set of 4 modes instead of 32 as in the current HEVC design. The remaining mode is coded using 2 bits instead of 5. It is reported that an encoding time reduction of 12-13% is observed without any coding penalty.

# Introduction

For intra-coded EL CUs, it turns out that the intra prediction mode is often equal or very close to the intra prediction mode of the co-located BL CU (noted BL mode), when this BL CU is intra coded. This property can be exploited for a more efficient intra mode coding process.

In this contribution, it is supposed that the BL mode is inserted as one of the 3 MPMs, as done in the SMuC0.11 when the MPM hook (corresponding to the macro ‘SVC_BL_CAND_INTRA’) is enabled. In this hook, the BL mode is often the first MPM, MPM0.

The contribution proposes a simplified process for the remaining mode coding, by reducing the number of remaining modes to a limited set of 4 modes instead of 32 as in the current HEVC design. As a result, the remaining mode can be coded using 2 bits instead of 5. This leads to a noticeable complexity reduction.

# Proposal

In the proposal, the number of possible remaining modes rem[k], k=0..M-1, is limited to M=4. The set of possible remaining modes depends on the MPM0:

- If MPM0 is angular, the set contains the M neighboring modes closest to the MPM0 and which are not equal to any of the MPMs.

  - As in the Core HEVC, the neighboring modes of a mode m are computed as follows:

Lower mode: 2 + ( ( m + 29 ) % 32 )
Larger mode: 2 + ( ( m − 2 + 1 ) % 32 )

- If MPM0 is not angular, the set corresponds to the M first neighboring modes which are not equal to any of the MPMs from the set for rem[k] is {26, 10, 18, 34, 2, 22, 14}.

The process to derive this set symmetrically applies at the encoder and at the decoder.

The number of bits to code the remaining mode is 2. The actual mode is set rem[ rem_intra_luma_pred_mode ].

# Results

The solution has been implemented in the SMuC0.11 with the MPM Hook enabled. The reference version is the SMuC 0.1.1 without any hook. The testing conditions are those defined for TE5.1 ‘Inter-layer intra mode prediction’ [1]. Full results are provided in the attached xls files.

The tables below summarize the results for the AI 2x and 1.5x cases. A significant encoder run time decrease  is observed without any coding penalty, thanks to the reduction of the number of bits to code the remaining mode.

SMuC0.11 with MPM Hook

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.20% | -0.04% | 0.06% |  |  |  |
| Class B | -0.30% | -0.17% | -0.18% | -0.11% | 0.08% | 0.06% |
| Overall (EL+BL) | -0.27% | -0.13% | -0.11% | -0.11% | 0.08% | 0.06% |
| Overall (EL) | -0.48% | -0.17% | -0.13% | -0.26% | 0.23% | 0.20% |
| Enc Time[%] | 102.7% |  |  | 102.9% |  |  |
| Dec Time[%] | 100.7% |  |  | 100.2% |  |  |

SMuC0.11 with MPM Hook And Reduced Number of Remaining Modes

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.25% | -0.05% | 0.07% |  |  |  |
| Class B | -0.29% | -0.15% | -0.14% | -0.11% | 0.11% | 0.11% |
| Overall (EL+BL) | -0.28% | -0.12% | -0.08% | -0.11% | 0.11% | 0.11% |
| Overall (EL) | -0.50% | -0.16% | -0.07% | -0.28% | 0.33% | 0.32% |
| Enc Time[%] | 89.0% |  |  | 90.3% |  |  |
| Dec Time[%] | 100.0% |  |  | 99.9% |  |  |

# Conclusion

In this contribution a simplified process was proposed for the remaining mode coding, by reducing the number of remaining modes to a limited set of 4 modes instead of 32 as in the current HEVC design. As a result, the remaining mode can be coded using 2 bits instead of 5 and the encoding runtime is reduced by  12-13% without any coding penalty.

# References

- V. Seregin, P. Onno, S. Liu, T. Lee, C. Kim, H. Yang, “Description of Tool Experiment C5: Inter-layer syntax prediction using HEVC base layer”, Joint Collaborative Team on Video Coding, JCTVC-K1105, October, 2012.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).