| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L107 |
| --- | --- |

| Title: | On Inter-Layer SAO for Base mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guillaume Laroche Tangi Poirier Christophe Gisquet Edouard François Patrice Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33(0)299876891 guillaume.laroche@crf.canon.fr christophe.gisquet@crf.canon.fr edouard.francois@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon Research Centre France |  |  |

_____________________________

# Abstract

This contribution presents an Inter-Layer SAO applied on the Base Mode prediction. In this proposal, the SAO parameters for the filtering of the base mode predictor is derived from the SAO syntax of the base layer or from parameters transmitted independently for the Luma and Chroma components. The proposed scheme provides 0.4% BDR gain, compared to the base mode proposed in section 5.3.3 of TE5 for Inter configurations.

# Introduction

JCTVC-L102 [1] presents the results of the Inter layer prediction mode described in TE5 section 5.3.3. This Inter-Layer prediction called “Base Mode” consists in using the base layer syntax of the current collocated CU to produce an entire texture data. Moreover, the base mode block predictor is generated by using the Generalised Residual Inter layer Prediction (GRILP) motion compensation method.

In the current version of SMuC, SAO filtering is applied on the base layer and the enhancement layer as a loop filtering. In this contribution, we proposed to increase the coding efficiency of the base mode proposed in TE5 section 5.3.3 [2], by using an Inter-Layer SAO.

# Proposed Method

The proposed inter layer SAO consists in deriving SAO syntax at LCU level as HEVC for the base mode predictor. This syntax can be derived from the base layer or generated according to parameters transmitted in the slice header.

As depicted in Figure 1, the SAO CTB syntax for the enhancement layer is set equal to the SAO CTB syntax of its collocated CTB in the base layer if the SAO type is equal to an Edge Offset type. Otherwise (when the type is equal to “No SAO” or “Band Offset”), the type of the current CTB SAO syntax is set equal to the default Edge prediction. The aim of keeping the base layer syntax for the enhancement layer is to reduce the computational complexity to derive the best parameters for all CTBs.

Figure 1 Interlayer SAO syntax derivation from the base layer to the enhancement layer for dyadic case.

The default SAO parameters are selected independently for luma and chroma components. The type of this default set of parameters is selected among the 4 edges offset types. Yet the set of offsets is always equal to (1, 0, 0, -1). Consequently classes 1 and 2 don’t need to be determined for the pixel classification.

Moreover, the Edge classification and filtering is applied inside CTB to avoid the use of pixels outside the CTB. With this restriction the neighboring base mode blocks don’t need to be reconstructed at decoder side.

The use of this Inter-Layer SAO for the base mode is signaled in the slice header by one flag for each Luma and both Chroma components. If this flag is set to 1 then the edge direction for the default set of parameters is signaled (independently for luma and chroma).

# Results

Table 1 shows the results of the proposed Inter-Layer SAO on the base mode predictor compared to the Base Mode of TE5 section 5.3.3. In average 0.4%, 1.0% and 0.9% BDR gain are obtained on respectively Y, U and V components. The proposed method doesn’t affect the encoding and decoding time.

Table 1 Simulation results of the proposed modification compared to the base mode scheme of TE5 section 5.3.3.

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.5% | -0.9% | -1.0% |  |  |  | -0.4% | -1.1% | -1.4% |
| Class B | -0.2% | -0.4% | -0.2% | -0.2% | -0.4% | -0.2% | -0.2% | -0.3% | -0.1% |
| Overall (EL+BL) | -0.3% | -0.5% | -0.4% | -0.2% | -0.4% | -0.2% | -0.3% | -0.5% | -0.4% |
| Overall (EL) | -0.6% | -1.2% | -1.0% | -0.4% | -1.3% | -0.7% | -0.5% | -1.4% | -1.1% |
| Enc Time[%] | 116.2% |  |  | 116.6% |  |  | 116.7% |  |  |
| Dec Time[%] | 127.8% |  |  | 117.8% |  |  | 131.8% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.6% | -1.8% | -1.7% |  |  |  | -0.5% | -1.8% | -1.8% |
| Class B | -0.4% | -1.2% | -1.1% | -0.4% | -1.4% | -1.5% | -0.5% | -1.5% | -1.3% |
| Overall (EL+BL) | -0.5% | -1.4% | -1.2% | -0.4% | -1.4% | -1.5% | -0.5% | -1.6% | -1.5% |
| Overall (EL) | -0.9% | -2.8% | -2.5% | -0.9% | -3.9% | -3.6% | -0.9% | -3.2% | -2.9% |
| Enc Time[%] | 114.4% |  |  | 114.0% |  |  | 114.0% |  |  |
| Dec Time[%] | 123.5% |  |  | 119.4% |  |  | 130.7% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

Table 2 shows the results of the proposed inter layer SAO on Base Mode prediction compared to the SMuC 0.1.1 anchors. In average the combination gives 3.5% gains and 7.6% on enhancement layer only.

Table 2 Simulation results of the proposed modification compared to SMuC Anchors.

|  | RA HEVC 2x |  |  |  | RA HEVC 1.5x |  |  |  | RA HEVC SNR |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U |  | V | Y |  | U | V | Y |  | U |  | V |  |  |  |
| Class A | -4.1% | -9.2% |  | -8.7% |  |  |  |  | -4.2% |  | -10.8% |  | -11.1% |  |  |  |
| Class B | -3.2% | -5.2% |  | -5.6% | -3.9% |  | -6.3% | -7.0% | -3.5% |  | -4.6% |  | -5.2% |  |  |  |
| Overall (EL+BL) | -3.4% | -6.4% |  | -6.5% | -3.9% |  | -6.3% | -7.0% | -3.7% |  | -6.4% |  | -6.9% |  |  |  |
| Overall (EL) | -6.7% | -12.1% |  | -12.3% | -10.3% |  | -17.0% | -18.4% | -7.8% |  | -13.4% |  | -14.2% |  |  |  |
| Enc Time[%] | 102.0% |  |  |  | 99.8% |  |  |  | 101.3% |  |  |  |  |  |  |  |
| Dec Time[%] | 139.6% |  |  |  | 168.6% |  |  |  | 140.1% |  |  |  |  |  |  |  |
| BL Match | Matched |  |  |  | Matched |  |  |  | Matched |  |  |  |  |  |  |  |
|  | LD-P HEVC 2x |  |  |  | LD-P HEVC 1.5x |  |  |  | LD-P HEVC SNR |  |  |  |  |  |  |  |
|  | Y | U |  | V | Y |  | U | V | Y |  | U |  | V |  |  |  |
| Class A | -3.4% | -7.3% |  | -6.7% |  |  |  |  | -4.2% |  | -8.4% |  | -8.2% |  |  |  |
| Class B | -2.9% | -4.0% |  | -3.9% | -3.4% |  | -5.2% | -5.3% | -3.7% |  | -4.6% |  | -4.8% |  |  |  |
| Overall (EL+BL) | -3.0% | -4.9% |  | -4.7% | -3.4% |  | -5.2% | -5.3% | -3.8% |  | -5.7% |  | -5.8% |  |  |  |
| Overall (EL) | -5.6% | -9.2% |  | -8.9% | -8.2% |  | -13.4% | -13.5% | -7.3% |  | -11.2% |  | -11.2% |  |  |  |
| Enc Time[%] | 102.9% |  |  |  | 100.3% |  |  |  | 102.5% |  |  |  |  |  |  |  |
| Dec Time[%] | 135.6% |  |  |  | 165.6% |  |  |  | 145.5% |  |  |  |  |  |  |  |
| BL Match | Matched |  |  |  | Matched |  |  |  | Matched |  |  |  |  |  |  |  |

# Conclusion

This contribution presents an Inter-Layer SAO method for the base mode prediction in order to improve the coding efficiency of the base mode. The proposed inter layer SAO re-use part of the SAO syntax of the base layer in order to reduce the parameters selection complexity for the enhancement layer. When the collocated CTB of the base layer doesn’t contain SAO parameters or contains band offset parameters, the SAO parameters of current CTB are set equal to a default Edge Offset parameters set. In average, the proposed base mode filtering gives 0.4% on luma components and 0.9% in average for both chroma components. The classical HEVC SAO loop filtering on reconstructed frame is not modified with this proposal.

# References

- Christophe Gisquet, Tangi Poirier, Jonathan Taquet, Fabrice Le Léannec, Edouard François, Guillaume Laroche, Patrice Onno, “TE5: Results of test 5.3.3 on the Inter-Layer Prediction Mode”, Joint Collaborative Team on Video Coding, JCTVC-L102, January 2013.

- Vadim Seregin, Patrice Onno, Shan Liu, Tammy Lee, Chulkeun Kim, Haitao Yang, Haricharan Laksman, “Description of Tool Experiment C5: Inter-Layer syntax prediction using the HEVC base layer”, Joint Collaborative Team on Video Coding, JCTVC-K1105, October, 2012.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).