| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0108 |
| --- | --- |

| Title: | Combination of several interlayer tools |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Christophe Gisquet Edouard François Guillaume Laroche Patrice Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33(0)299876800 christophe.gisquet@crf.canon.fr edouard.francois@crf.canon.fr guillaume.laroche@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon Research Centre France |  |  |

_____________________________

# Abstract

In JCTVC-K0041 and JCTVC-K0362, Canon combined several inter layer tools for the response to the Call for Proposal for SHVC. Some of them were evaluated during TEs. This contribution presents the results of 3 sets of combination for those tools. Set A is reported to have Y-BDR performance of -4.0%(Y),  -5.8%(U), -6.1%(V) over the SMuC 0.1.1 with no runtime increase at the encoder. Set B is reported to have Y-BDR performance of -5.1%(Y), -7.2%(U), -7.5%(V) with about 114% encoding runtime. Finally set C provides the higher performance -6.0%(Y), -7.6%(U), -7.9%(V) while the complexity in terms of encoder runtime is about 200%.

# Introduction

Tool Experiments have focused on evaluating the individual performance of several interlayer tools. However, it is interesting to evaluate their combined effect as they may not be additive.

A list of the tools from K0041[1] evaluated in Tool Experiments are:

- Generalized Residual Inter-Layer Prediction (GRILP) from TE3, test 4.6.1 [2];

- Inter-layer inferred prediction (InterBL, aka base mode) mode from TE5, test 5.3.3 [3];

- AMVP/Merge candidate derivation [4].

## Generalized Residual Inter-Layer Prediction (GRILP)

Generalized Residual Inter-Layer Prediction (GRILP) mode consists in generating, given a Prediction Unit and motion vector MV being considered, a corresponding temporal residual CU in the base layer that is added to enhancement layer temporal prediction PU.

The GRILP prediction mode can be selected on a CU or PU basis, depending on the configuration. It can therefore be activated inside Inter CUs and is signaled by a flag at respectively the CU or PU level. When activated at the CU level, it is used for all PUs of the CU. The GRILP prediction mode proceeds as follows.

The prediction signal is derived as follows (shown for the mono-prediction case, the concept is easily generalized for the bi-prediction case):

PREDEL = MC[REFEL, MVEL] + α.{UP{RECBL} - MC[UP{REFBL}, MVEL]}

where PREDEL is the prediction signal of the enhancement layer, REFEL and REFBL are the reference pictures in the Enhancement and Base layers, RECBL is the base layer reconstructed picture corresponding to the current enhancement layer picture, MVEL is the motion vector selected, UP{.} is the up-sampling operator, MC[I,MV] is the motion compensation step of the current picture using I as reference picture and MV for the motion vectors, and finally, α for the weighting of the interlayer residual.

For this test, and contrary to test 4.6.1, the interpolation filters for motion compensation are not those of the HEVC core design, but instead bilinear filters. The up-sampling interpolation filters are those defined in the TE 2. The possible weights are 0, 1 and 1/2 as seen in other GRILP-based solutions.

The principle of the GRILP mode can be summarized by the following Figure 1.

Figure 1: Schematic principle of the GRILP mode

## Inter-layer inferred prediction (InterBL) mode

The “Base Mode” or “InterBL mode” presented in test 5.3.3 for TE5 consists in predicting the current CU in the enhancement layer by applying a motion compensation using the motion information (motion vector, reference list, reference index, …) of the collocated base layer CU. Motion vectors are scaled to match the spatial resolution change. In this Base Mode, we are also considering the addition of a residual data. The Base Mode can be summarized by the following formula:

PREEL = MC{ REFEL , SP_ratio*MVBL } + RES,

where:

- MC{I, MV} corresponds to the motion compensation operator (using DCT-IF filters) with motion vector MV  and using as reference picture the picture I;

- REFEL is the reference picture for the enhancement layer;

- MVBL is the motion vector of the base layer;

- SP_ratio is the spatial ratio between the base layer and the enhancement layer and;

- And RES is the decoded residual of the corresponding CU in the base layer and is computed as follows:

RES = UPS{RECBL} – MC{UPS{REFBL}, SP_ratio*MVBL }}

where:

- UPS{.} is the up-sampling operator (using DCT-IF filters);

- RECBL is the reconstructed base layer CU.

Contrary to the SVC specifications, the temporal residual texture coded and decoded in the base layer is not inherited from the base picture. For the proposed “Base Mode”, the residual RES is based on the GRILP concept presented in previous section and is used for the prediction of the enhancement layer.

Furthermore, the actual motion information from the reference layer, without compression, is used as a candidate for the merge mode.

## Modification of the Enhancement Layer Merge mode

The derivation process of the Merge, Merge Skip presented in JCTVC-K0041 has been modified compared to the HM8.1, the number of candidates and the order of candidates are modified. Moreover, new candidates are added for the enhancement layer. These adaptations are listed below.

- The number of candidates in the final list is increased from 5 to 6.

- The temporal candidate is on the first position if the temporal collocated block is selected in a forward reference. In that case only the centered position is considered for the enhancement layer. Please note that it affect only the random access case.

- The derivation process of spatial candidates is the same as for the base layer, except that the duplicate check is the same as the MVP hook duplicate check.

- Then the motion vector of the co-located base layer is added to the list of candidates. In that case the bottom right position is considered.

- The base motion vector is followed by 4 offset candidates derived according to derived according to this base motion vector candidate. The generation of these candidates consists in adding one offset (offset value: 4 or -4) alternatively to the horizontal or to the vertical component of the motion vector from list L0 and its inverse value to components of motion vector from list L1.

- Then the combined candidates derivation and the zero candidates derivation are applied if at least one position is available. Yet the number of possible combination of combined candidates have been modify to match 6 candidates in the list.

- Moreover, the MV data compression was removed for the Enhancement layer as it was definitively removed for HM9.

## On Inter-Layer SAO for Base Mode

The proposed inter layer SAO consists in deriving SAO syntax at LCU level as HEVC for the base mode predictor. This syntax can be derived from the base layer or generated according to parameters transmitted in the slice header.

As depicted in Error: Reference source not found, the SAO CTB syntax for the enhancement layer is set equal to the SAO CTB syntax of its collocated CTB in the base layer if the SAO type is equal to an Edge offset type. Otherwise (when the type is equal to “No SAO” or “Band offset”), the type of the current CTB SAO syntax is set equal to the default Edge prediction. The aim to keep base layer syntax for the enhancement layer is to reduce the computational complexity at encoder side to derive the best parameters for all CTB.

Figure 2: Interlayer SAO syntax derivation from the base layer to the enhancement layer for dyadic case

The default SAO parameters are selected independently for luma and chroma components. The type of this default set of parameters is selected among the 4 edges offset types. Yet the set of offsets is always equal to (1, 0, 0, -1). Consequently, classes 1 and 2 don’t need to be determined for the pixel classification.

# Tests

## Description of the tested set of tools

We evaluate 3 sets of tools, First set, A, uses:

- A 8x8-syntax-based base mode using GRILP and bilinear filter;

- The described changes to AMVP and merge.

A second set B with improved efficiency is provided, with the following modifications:

- A 8x8-syntax-based base mode using GRILP and bilinear filter;

- The described changes to AMVP and merge.

- A single PU-level GRILP mode with α=1 and  α=1/2 and performing a very simple motion estimation;

Last set of tools, C illustrates the best performance reachable using all mentioned tools.

- The described changes to AMVP and merge.

- The InterBL tool is used to build a complete image, which gets deblocked using the rescaled syntax from the reference layer;

- Said tool uses the reference layer motion with its fullest granularity (ie 4x4 based);

- The upsampled reference layer image used in InterBL is filtered with a SAO-like processing as described in [5];

- The GRILP mode is now CU-based but uses full motion estimation.

The above tests can be summarized with the following tables:

|  | Set A | Set B | Set C |
| --- | --- | --- | --- |
| Merge hooks | X | X | X |
| Base mode | X | X | X |
| Filtered base mode picture |  |  | X |
| CU-level GRILP |  |  | X |
| PU-level GRILP |  | X |  |
| Secondary GRILP mode |  | X | X |
| Full ME for GRILP |  |  | X |

## Results

### Set A

Average BDR: Y -4.0%	U -5.8%	 V -6.1%

### Set B:

Average BDR: Y -5.1%	U-7.2%,V-7.5%

### Set C:

Average BDR: Y -6.0%	U-7.6%		V-7.9%

# Conclusion

In this contribution, we proposed three combinations of several tools proposed in TE or in contributions. These combinations correspond to three complexity levels at encoder and decoder side. The average gains obtained for all Inter configurations are from 4% to 6% gain for a respective encoder run time from 100% to 230%. Yet, the set of Tools B seems to be an interesting coding efficiency and complexity compromise with 5.1% BDR luma gain and 117% run time encoder and 110% decoder.  So, according to these good coding efficiency/runtime compromises, we recommend to consider the Set B of tools for the definition of SHVC reference software.

# References

- Sébastien Lasserre, Fabrice Le Léannec, Jonathan Taquet, Naël Ouedraogo, Stéphane Pautet, Christophe Gisquet, Guillaume Laroche, Tangi Poirier, Yann Verdavaine, Edouard Francois, Eric Nassor, “Description of the scalable video coding technology proposal by Canon Research Centre France” Joint Collaborative Team on Video Coding, JCTVC-K0041, October, 2012.

- Christophe Gisquet, Fabrice Le Léannec, Jonathan Taquet, Edouard François, Guillaume Laroche, Patrice Onno, “TE3: Results of test 4.6.1 on the Generalized Residual Inter-Layer Prediction (GRILP)”, Joint Collaborative Team on Video Coding, JCTVC-L0100, January 2013

- Christophe Gisquet, Tangi Poirier, Jonathan Taquet, Fabrice Le Léannec, Edouard François, Guillaume Laroche, Patrice Onno, “TE5: Results of test 5.3.3 on the Inter-Layer Prediction Mode”, Joint Collaborative Team on Video Coding, JCTVC-L0107, January 2013

- Guillaume Laroche, Tangi Poirier, Patrice Onno, Christophe Gisquet, Edouard François, “TE5: Results of test 5.2.6 on the Inter-Layer Motion Vector Prediction”, Joint Collaborative Team on Video Coding, JCTVC-L0101, January 2013

- Guillaume Laroche, Tangi Poirier, Christophe Gisquet, Edouard François, Patrice Onno, “On InterLayer SAO for Base Mode”, Joint Collaborative Team on Video Coding, JCTVC-L0107, January 2013

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).