| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0110 |
| --- | --- |

| Title: | TE2 Subtest 1: Inter-layer Texture Prediction Signalling (test 3.1.9) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tammy Lee Elena Alshina | Email: | tammy.lee@samsung.com elena_a.alshina@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution presents the detailed description and the results of inter-layer texture prediction signalling method (test 3.1.9) for TE2 subtest 1. When the CU is coded in Intra_BL mode, the corresponding BL block of reconstructed pixels are scaled to the EL CU resolution and used as the predictor for the 2Nx2N PU of the CU. The resulting residual samples of the CU are coded following non-scalable HEVC's inter residual sample coding process. In addition, Intra_BL Skip mode is introduced as the special mode of Intra_BL. In this case, the CU is implied to have no significant coefficient. It is reportedly shown that the proposed CU based Intra_BL mode provides BD-rate gains by -23.1%, -32.9%, -16.6%, -26.4%, -12.5% and -22.3% for AI-2x, AI-1.5x, RA-2x, RA-1.5x, LDP-2x and LDP-1.5x configurations, respectively. Delta gains relative to SHVC v0.1.1 are -0.3%, -0.3%, -0.3%, -0.3%, -0.3% and -0.4% for AI-2x, AI-1.5x, RA-2x, RA-1.5x, LDP-2x and LDP-1.5x configurations, respectively.

# Introduction

When the CU is coded in Intra_BL mode, no spatial intra directional prediction is performed, and only inter-layer pixel prediction is used. In this mode, the corresponding BL block of reconstructed pixels are scaled to the EL CU resolution and used as the predictor for the 2Nx2N PU of the CU. The resulting residual samples of the CU are coded following non-scalable HEVC's inter residual sample coding process.

When the CU is coded in Intra_BL mode, due to the efficiency of the mode, the residual coefficients often get quantized to zero.  As a result, a special mode of Intra_BL – Intra_BL Skip mode – is introduced. In this case the CU is implied to have no significant coefficient.

The decoding process is provided in the following figure.

Start decoding a CU

Skip mode?

Intra_BL Skip mode?

Intra_BL mode?

Intra/Inter

Process Inter prediction

Process Inter prediction

Process Intra prediction

Process Inter-layer pixel prediction

Process Inter-layer pixel prediction

Residual decoding

Reconstruct CU

End

Yes

Yes

Inter

Yes

No

Intra

No

No

# Simulation Results

Based on test conditions specified in TE2, simulations are performed using all the AI, RA and LD-P configuration for both 2x and 1.5x spatial scalability scenarios as mandatory and the RA and LD-P configuration for SNR scalability scenarios as optional. Detailed results can be found in the associated excel data sheet.

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -27.2% | -27.2% | -28.1% |  |  |  |
| Class B | -21.5% | -21.1% | -21.0% | -32.9% | -32.8% | -33.1% |
| Overall (EL+BL) | -23.1% | -22.9% | -23.0% | -32.9% | -32.8% | -33.1% |
| Overall (EL) | -35.3% | -35.1% | -35.4% | -58.6% | -58.0% | -58.5% |
| Enc Time[%] | 114.7% |  |  | 113.3% |  |  |
| Dec Time[%] | 93.9% |  |  | 91.3% |  |  |
| Enc Mem[%] | 109.6% |  |  | 109.3% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -18.6% | -7.8% | -9.4% |  |  |  |
| Class B | -15.8% | -8.9% | -7.5% | -26.4% | -19.9% | -18.5% |
| Overall (EL+BL) | -16.6% | -8.6% | -8.1% | -26.4% | -19.9% | -18.5% |
| Overall (EL) | -25.6% | -13.3% | -12.7% | -47.8% | -37.3% | -35.0% |
| Enc Time[%] | 107.8% |  |  | 111.3% |  |  |
| Dec Time[%] | 119.9% |  |  | 123.1% |  |  |
| Enc Mem[%] | 112.1% |  |  | 110.0% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -13.7% | -2.0% | -3.2% |  |  |  |
| Class B | -12.0% | -6.6% | -4.9% | -22.3% | -16.5% | -14.9% |
| Overall (EL+BL) | -12.5% | -5.3% | -4.4% | -22.3% | -16.5% | -14.9% |
| Overall (EL) | -20.0% | -8.2% | -7.1% | -41.6% | -31.5% | -28.4% |
| Enc Time[%] | 107.6% |  |  | 111.1% |  |  |
| Dec Time[%] | 126.5% |  |  | 124.5% |  |  |
| Enc Mem[%] | 112.2% |  |  | 110.5% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

The results show the proposed CU based Intra_BL mode provides BD-rate gains by -23.1%, -32.9%, -16.6%, -26.4%, -12.5% and -22.3% for AI-2x, AI-1.5x, RA-2x, RA-1.5x, LDP-2x and LDP-1.5x configurations, respectively. Delta gains relative to SHVC v0.1.1 are -0.3%, -0.3%, -0.3%, -0.3%, -0.3% and -0.4% for AI-2x, AI-1.5x, RA-2x, RA-1.5x, LDP-2x and LDP-1.5x configurations, respectively.

|  | RA HEVC SNR |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -21.6% | -12.0% | -11.6% |
| Class B | -19.6% | -10.4% | -7.4% |
| Overall (EL+BL) | -20.2% | -10.9% | -8.6% |
| Overall (EL) | -31.9% | -18.2% | -15.0% |
| Enc Time[%] | 106.7% |  |  |
| Dec Time[%] | 95.3% |  |  |
| Enc Mem[%] | 99.7% |  |  |
| BL Match | Matched |  |  |
|  | LD-P HEVC SNR |  |  |
|  | Y | U | V |
| Class A | -14.1% | -7.3% | -6.3% |
| Class B | -13.0% | -5.2% | -1.6% |
| Overall (EL+BL) | -13.3% | -5.8% | -2.9% |
| Overall (EL) | -21.4% | -11.1% | -7.0% |
| Enc Time[%] | 105.6% |  |  |
| Dec Time[%] | 96.0% |  |  |
| Enc Mem[%] | 100.3% |  |  |
| BL Match | Matched |  |  |

In RA-SNR and LDP-SNR configurations, it provides -20.2% and -13.3% BD-rate gains, respectively. Delta gains relative to SHVC v0.1.1 are -0.2% and -0.2% in these configurations, respectively.

For more information, the proposed method is tested without Intra_BL Skip mode. In this test, Root Cbf is coded for Intra_BL mode as other CUs of inter prediction.

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -27.2% | -27.0% | -27.9% |  |  |  |
| Class B | -21.4% | -21.0% | -20.9% | -32.8% | -32.8% | -33.0% |
| Overall (EL+BL) | -23.1% | -22.8% | -22.9% | -32.8% | -32.8% | -33.0% |
| Overall (EL) | -35.2% | -34.9% | -35.2% | -58.4% | -58.0% | -58.5% |
| Enc Time[%] | 116.3% |  |  | 114.8% |  |  |
| Dec Time[%] | 94.7% |  |  | 91.4% |  |  |
| Enc Mem[%] | 109.7% |  |  | 108.6% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -18.5% | -7.5% | -9.1% |  |  |  |
| Class B | -15.8% | -8.8% | -7.4% | -26.3% | -19.8% | -18.4% |
| Overall (EL+BL) | -16.6% | -8.4% | -7.9% | -26.3% | -19.8% | -18.4% |
| Overall (EL) | -25.5% | -13.0% | -12.4% | -47.6% | -37.2% | -34.7% |
| Enc Time[%] | 108.4% |  |  | 110.8% |  |  |
| Dec Time[%] | 122.5% |  |  | 117.9% |  |  |
| Enc Mem[%] | 111.9% |  |  | 110.0% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -13.7% | -1.7% | -2.8% |  |  |  |
| Class B | -11.9% | -6.7% | -4.9% | -22.1% | -16.7% | -14.8% |
| Overall (EL+BL) | -12.4% | -5.2% | -4.3% | -22.1% | -16.7% | -14.8% |
| Overall (EL) | -19.8% | -8.2% | -6.9% | -41.1% | -31.9% | -28.5% |
| Enc Time[%] | 107.4% |  |  | 110.3% |  |  |
| Dec Time[%] | 124.5% |  |  | 120.1% |  |  |
| Enc Mem[%] | 112.1% |  |  | 110.4% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

|  | RA HEVC SNR |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -21.5% | -11.6% | -11.2% |
| Class B | -19.6% | -10.2% | -7.4% |
| Overall (EL+BL) | -20.1% | -10.6% | -8.5% |
| Overall (EL) | -31.8% | -17.8% | -14.7% |
| Enc Time[%] | 106.0% |  |  |
| Dec Time[%] | 89.8% |  |  |
| Enc Mem[%] | 99.6% |  |  |
| BL Match | Matched |  |  |
|  | LD-P HEVC SNR |  |  |
|  | Y | U | V |
| Class A | -14.1% | -7.0% | -5.9% |
| Class B | -13.0% | -5.1% | -1.7% |
| Overall (EL+BL) | -13.3% | -5.7% | -2.9% |
| Overall (EL) | -21.3% | -10.8% | -6.8% |
| Enc Time[%] | 104.7% |  |  |
| Dec Time[%] | 90.8% |  |  |
| Enc Mem[%] | 100.2% |  |  |
| BL Match | Matched |  |  |

According to the above results, it turns out that Intra_BL Skip mode provides -0.1% BD-rate gains by introducing one skip flag.

It is noted that the proposed CU based Intra_BL mode reduces decoder complexity approximately by 13%.

# References

[1] J. Boyce, D. Hong, W. Jang, A. Abbas, S. Reddy (Vidyo), K. McCann, J.-H. Park, J. Kim, C. Kim, J.-H. Min, E. Alshina, A. Alshin, I.-K. Kim, T. Lee, B. Choi, Y. Piao, S. Jeong, S. Lee, Y. Cho, J. Y. Choi (Samsung), “Description of low complexity scalable video coding technology proposal by Vidyo and Samsung”, JCTVC-K0045, Shanghai, China, Oct 2012.

[2] L. Guo, Y. He, D. Kown, J. Zan, H. Lakshman, J. Kang (TE Coordinators), “Description of Tool Experiment A2: Inter-layer Texture Prediction Signaling in SHVC”, JCTVC-K1102, Shanghai, China, Oct 2012.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).