| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0111 |
| --- | --- |

| Title: | Lightweight single-loop scalability with SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kemal Ugur Hoda Roodaki-Lavasani Miska M. Hannuksela | Tel: Email: | +358 50 4860857 name.surname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

Experimental results show that constructing a scalable bitstream where scalable coding tools are used only when base-layer pictures are RAP pictures improve the enhancement layer coding performance more than 11% compared to simulcast. It is argued that, this encoding configuration is a lightweight option for many encoders as no multi-loop is necessary and inter-picture related syntax elements need not to be decoded. This contribution proposes indication of such restriction in the scalable bitstream.

# Introduction

The current SHVC draft test model utilizes multi-loop coding approach where base layer samples need to be reconstructed fully to decode enhancement layer picture. This requires an SHVC decoder to perform multiple motion compensation processes, which results in a higher complexity. If multiple layers are used, then the complexity is increased even further.

It is observed that having a scalable bitstream, where inter layer prediction is used only when base layer picture is a RAP picture already improves the coding efficiency over simulcast. Hence, this configuration could be a lightweight option for many encoders as no multi-loop is necessary and inter-picture related syntax elements need not to be decoded.

We are proposing to indicate this restriction as a sequence level flag, called inter_layer_pred_from_bl_rap_only_flag. The proposed flags indicate encoder restrictions and it could be argued that those could be placed in an SEI message. However, indicating it as a flag in VPS has the following benefits:

- An EL decoder can safely discard all inter BL pictures without decoding.

- An SHVC decoder can support and announce a higher level if inter_layer_pred_from_bl_rap_only_flag is 1 and a lower level otherwise.

- In a multi-party conferencing scenario, a middle box could decide not sending base layer inter pictures when serving the enhancement layer.

# Syntax  and Semantics

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| // layer dependency |  |
| num_direct_ref_layers[ i ] | u(6) |
| for( j = 0; j < num_direct_ref_layers[ i ]; j++ ) { |  |
| ref_layer_id[ i ][ j ] | u(6) |
| } |  |
| } |  |
| inter_layer_pred_from_bl_rap_only_flag | u(1) |
| } |  |

inter_layer_pred_from_bl_rap_only_flag equal to 1 indicates that any non-RAP picture with layer_id equal to 0 is not used for inter-layer prediction. inter_layer_pred_from_bl_rap_only_flag equal to 0 indicates that any non-RAP picture with layer_id equal to 0 may or may not be used inter-layer prediction.

# Experimental Results

We tested the performance of using this flag in the current SHVC 0.1.1 software with the current common conditions and compared it with respect to simulcast. Motion and intra mode related syntax prediction hooks were disabled. Inter-layer prediction was enabled only when base layer picture is a RAP picture.

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -7.4% | -5.5% | -5.6% |  |  |  | -8.5% | -7.4% | -7.4% |
| Class B | -5.3% | -3.4% | -3.2% | -8.5% | -6.4% | -6.3% | -7.2% | -5.4% | -4.8% |
| Overall (EL+BL) | -5.9% | -4.0% | -3.9% | -8.5% | -6.4% | -6.3% | -7.5% | -6.0% | -5.5% |
| Overall (EL) | -8.9% | -6.1% | -5.9% | -14.5% | -11.2% | -11.1% | -11.5% | -9.3% | -8.8% |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).