| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 12th Meeting: Geneva, CH, 14-23 January, 2013 | Document: JCTVC-L0112 |
| --- | --- |

| Title: | AHG7: Reference sample filtering in intra prediction for chroma extended format |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junghye Min Tammy Lee Chanyul Kim | Email: | jh643.min@samsung.com tammy.lee@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution proposes a reference sample filtering method in intra prediction for chroma extended formats. In the common software with range extension support, same reference sample filtering process from luma component is applied for chroma. Because chroma and luma components provides different information in images, different filtering process for chroma component is needed to improve coding efficiency. In this proposal, 2 filtering decision methods for chroma components are presented. The fist method provides -0.1%,-0.1%, -0.2%, performance gains for each color component respectively. The second method provides better gains which are -0.1%,-0.2%,-0.2%. These two methods provide coding efficiency without increase of encoding/decoding time as shown in the test results.

# Introduction

In the HM-9.0.1, reference sample filtering in the intra prediction (Fig.1) is used only for luma component.

Figure.1 reference sample filtering for intra prediction

In the common software for range extension support, same filtering process from luma is applied for chroma component for 4:4:4 formats to improve performance of chroma intra prediction. For 4:2:2 chroma formats, sample filtering is not applied. Based on the transform unit size and intra prediction mode, whether filtered samples are used is determined as in following process.

diff = min (abs( predMode - HOR_IDX), abs(predMode - VER_IDX));

sizeIndex=getSizeIndex(TuWidth)

filterFlag = diff > m_aucIntraFilter[sizeIndex]? 1:0;

m_aucIntraFilter [] =

{   10, //4x4

7, //8x8

1, //16x16

0, //32x32

},

Filtering decision rules for luma component from above process can be summarized as following table where yellow color represents use of filtered reference samples.

Table 1. Filtering decision rule for luma component

# Proposed method

Because chroma component represents color information while luma represents the brightness in images, different filtering decision process should be used for chroma intra prediction for better coding efficiency. Chroma component is known to have noises with unnatural patterns which can affect the coding efficiency of intra prediction. To remove unnatural noise patterns from chroma components, filtering is applied for more cases comparing to luma filtering decision rule. The strong filtering which involves second filtering is also exploited for better coding efficiency.  While no filtering is applied for 4:2:2: format in the common test s/w, reference sample filtering is also applied for 4:2:2 format. Two filtering decision methods are as follows.

Method 1)

Filtering is applied for more cases in 4x4 and 8x8 TU sizes. Minimal changes in the s/w are required which replace numbers in the table m_aucIntraFilter with {6,1,0,0}. Filtering decision results from changes can be presented as in following table.

Table2.  Filtering decision rule for chroma component in the method 1

Method 2)

The strong filter with second filtering is exploited for large TU sizes to remove noises in reference samples. Numbers in the table m_aucIntraFilter were changed to {6,2,1,0} in this case. Additional process for second filtering was added in the s/w. In the following table, green color represents the use of strong filter.

Table3.  Filtering decision rule for chroma component in the method 2

# Results

Proposed methods were implemented on top of HM-range-extensions s/w under common test condition described in JCTVC-K1006 [1].  Table 4 describes 3 different test cases. For all cases, chroma reference sample filtering was also applied for 4:2:2 format.

Table 4. Description of filtering method for test cases

| Chroma formats | Common s/w | Test1 | Test2 | Test3 |
| --- | --- | --- | --- | --- |
| 4:4:4 | Same as luma | Same as luma | Use method 1 | Use method 2 |
| 4:2:2 | Not applied | Same as luma | Use method 1 | Use method 2 |

The first test shows the performance gains when same filtering process from luma is applied for 4:2:2 format. Test 2 and 3 show the performance of method 1 and 2 respectively. Without increase of encoding/decoding time, the method 1 provides -0.1%. -0.1% -0.2% performance gains and the method 2 provides -0.1%,-0.2%,-0.2% gains in AI test configuration. In RA test condition, -0.1% gains are observed in luma component for both tests (6.Full test results). Test results show that strong filter provides additions gains. While proposed methods are applied only for chroma, performance gains are obtained also in luma component.

Table I. Test results when chroma filtering in applied for 422 format.

| test1 | All Intra HE Main-tier |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| RGB 4:4:4 | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 | 0.0% | 0.0% | 0.0% |
| YCbCr 4:2:2 | 0.0% | -0.1% | -0.1% |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

Table I. Test results of method 1

| test2 | All Intra HE Main-tier |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| RGB 4:4:4 | -0.1% | -0.1% | -0.2% |
| YCbCr 4:4:4 | -0.1% | -0.1% | -0.1% |
| YCbCr 4:2:2 | -0.1% | -0.2% | -0.2% |
| Overall | -0.1% | -0.1% | -0.2% |
|  | -0.1% | -0.1% | -0.2% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

Table IV. Test results of method 2

| test3 | All Intra HE Main-tier |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| RGB 4:4:4 | -0.2% | -0.2% | -0.3% |
| YCbCr 4:4:4 | -0.1% | -0.2% | -0.2% |
| YCbCr 4:2:2 | -0.1% | -0.1% | -0.2% |
| Overall | -0.1% | -0.2% | -0.2% |
|  | -0.1% | -0.2% | -0.2% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

| test2 | All Intra HE Main-tier |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| RGB 4:4:4 | -0.1% | -0.1% | -0.2% |
| YCbCr 4:4:4 | -0.1% | -0.1% | -0.1% |
| YCbCr 4:2:2 | -0.1% | -0.2% | -0.2% |
| Overall | -0.1% | -0.1% | -0.2% |
|  | -0.1% | -0.1% | -0.2% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

# Conclusions

This proposal provides reference sample filtering process in chroma intra prediction for extended chroma model. Proposed methods provide additional gains in all color components without increase in encoding and decoding time. We recommend adopting the proposed sampling filtering method for the common  s/w with range extension support.

- JCT-VC, “Common test conditions and software reference configurations for HEVC range extensions”, JCTVC-K1006, 11th JCT-VC Meeting, Shanghai, China, 10-19 Oct., 2012

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Full test results

| test1 | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:2:2 | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:2:2 | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:2:2 | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

| test2 | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% | -0.1% |
| YCbCr 4:4:4 | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| YCbCr 4:2:2 | -0.1% | -0.2% | -0.2% | -0.1% | -0.1% | -0.2% |
| Overall | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% | -0.2% |
|  | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% | -0.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| YCbCr 4:4:4 | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:2:2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| Overall | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | -0.1% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:2:2 | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

| test3 | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | -0.2% | -0.2% | -0.3% | -0.1% | -0.1% | -0.2% |
| YCbCr 4:4:4 | -0.1% | -0.2% | -0.2% | -0.1% | -0.1% | -0.2% |
| YCbCr 4:2:2 | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% | -0.2% |
| Overall | -0.1% | -0.2% | -0.2% | -0.1% | -0.1% | -0.2% |
|  | -0.1% | -0.2% | -0.2% | -0.1% | -0.1% | -0.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| YCbCr 4:4:4 | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:2:2 | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
|  | -0.1% | 0.2% | 0.0% | -0.1% | -0.2% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| YCbCr 4:2:2 | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |