| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0115 |
| --- | --- |

| Title: | TE5 Subtest 3: Inter-layer inferred prediction mode (test 5.3.4) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tammy Lee Elena Alshina | Email: | tammy.lee@samsung.com elena_a.alshina@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution presents the detailed description and the results of inter-layer inferred prediction mode (test 5.3.4) for TE5 subtest 3. When the CU is coded in Inter_BL mode, all motion information for each 8x8 block of the CU is derived from the corresponding BL block’s motion information. If the corresponding BL motion information of an 8x8 block is not available, the 8x8 block’s motion is set by the previous 8x8 block’s motion in z-scan order or it is set to zero motion if it is the first 8x8 block inside the CU. It is reportedly shown that the proposed CU based Inter_BL mode provides BD-rate gains by -1.2%, -1.6%, -0.5% and -0.8% for RA-2x, RA-1.5x, LDP-2x and LDP-1.5x configurations, respectively.

# Introduction

When the CU is coded in Inter_BL mode, all motion information for each 8x8 block of the CU is derived from the corresponding BL block’s motion information.  For example, if the CU is a 32x32 block, then each of the sixteen 8x8 blocks of the CU gets its own motion information derived from the corresponding BL. When determining the corresponding BL motion information, the BL PU (or a block in case of mixed-codec scalability) that includes the corresponding sample (after proper scaling) of the left-top corner of each 8x8 block is selected.  Hence, each 8x8 block of the CU may get motion compensated blocks from different locations. If the corresponding BL motion information of an 8x8 block is not available, the 8x8 block’s motion is set by the previous 8x8 block’s motion in z-scan order or it is set to zero motion if it is the first 8x8 block inside the CU. Therefore, in this mode, only BL prediction mode and motion information are used. Since this mode sets the same motion information with adjacent blocks if a particular 8x8 block’s motion information cannot be determined, it increases the probability to have the same motion information with neighbors inside the CU coded as Inter_BL mode even though 8x8 block’s motion information is derived individually from the base-layer. In actual implementation, this enables relatively larger size of motion compensation unit than 8x8 to reduce memory bandwidth.

# Simulation Results

Based on test conditions specified in TE5, simulations are performed using all the RA and LD-P configuration for both 2x and 1.5x spatial scalability scenarios as mandatory and the RA and LD-P configuration for SNR scalability scenarios as optional. Detailed results can be found in the associated excel data sheet.

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.9% | -1.8% | -1.7% |  |  |  |
| Class B | -1.3% | -2.1% | -2.1% | -1.6% | -2.5% | -2.7% |
| Overall (EL+BL) | -1.2% | -2.0% | -2.0% | -1.6% | -2.5% | -2.7% |
| Overall (EL) | -2.2% | -3.7% | -3.8% | -4.3% | -6.7% | -7.0% |
| Enc Time[%] | 101.4% |  |  | 100.1% |  |  |
| Dec Time[%] | 100.7% |  |  | 100.4% |  |  |
| Enc Mem[%] | 100.0% |  |  | 100.0% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.4% | -0.6% | -0.6% |  |  |  |
| Class B | -0.6% | -0.9% | -0.9% | -0.8% | -1.3% | -1.4% |
| Overall (EL+BL) | -0.5% | -0.8% | -0.8% | -0.8% | -1.3% | -1.4% |
| Overall (EL) | -0.9% | -1.5% | -1.4% | -1.8% | -3.0% | -3.0% |
| Enc Time[%] | 102.4% |  |  | 101.5% |  |  |
| Dec Time[%] | 99.7% |  |  | 99.6% |  |  |
| Enc Mem[%] | 100.0% |  |  | 100.0% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

The results show the proposed CU based Intra_BL mode provides BD-rate gains by -1.2%, -1.6%, -0.5% and -0.8% for RA-2x, RA-1.5x, LDP-2x and LDP-1.5x configurations, respectively.

|  | RA HEVC SNR |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -1.9% | -4.0% | -4.2% |
| Class B | -1.9% | -3.1% | -3.2% |
| Overall (EL+BL) | -1.9% | -3.3% | -3.5% |
| Overall (EL) | -4.0% | -6.7% | -7.0% |
| Enc Time[%] | 100.0% |  |  |
| Dec Time[%] | 103.5% |  |  |
| Enc Mem[%] | 100.0% |  |  |
| BL Match | Matched |  |  |
|  | LD-P HEVC SNR |  |  |
|  | Y | U | V |
| Class A | -1.3% | -2.2% | -2.1% |
| Class B | -1.2% | -1.7% | -1.8% |
| Overall (EL+BL) | -1.2% | -1.9% | -1.9% |
| Overall (EL) | -2.3% | -3.4% | -3.5% |
| Enc Time[%] | 101.1% |  |  |
| Dec Time[%] | 102.8% |  |  |
| Enc Mem[%] | 100.0% |  |  |
| BL Match | Matched |  |  |

In RA-SNR and LDP-SNR configurations, it provides -1.9% and -1.2% BD-rate gains, respectively.

In the above testing, the deblocking filter performs filtering for each 8x8 block boundaries if a CU is chosen as the proposed Inter_BL mode because PU size can be regarded as 8x8 generally even though the partition size of Inter_BL mode is set to SIZE_2Nx2N for simplicity in syntax design.

However, the proposed method is optionally tested in the following without this deblocking filtering over 8x8 block boundaries.

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.9% | -1.8% | -1.7% |  |  |  |
| Class B | -1.3% | -2.1% | -2.1% | -1.6% | -2.5% | -2.6% |
| Overall (EL+BL) | -1.2% | -2.0% | -2.0% | -1.6% | -2.5% | -2.6% |
| Overall (EL) | -2.2% | -3.8% | -3.8% | -4.2% | -6.7% | -7.0% |
| Enc Time[%] | 102.2% |  |  | 100.8% |  |  |
| Dec Time[%] | 99.8% |  |  | 100.3% |  |  |
| Enc Mem[%] | 100.0% |  |  | 100.1% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.4% | -0.7% | -0.6% |  |  |  |
| Class B | -0.6% | -0.9% | -0.9% | -0.7% | -1.2% | -1.3% |
| Overall (EL+BL) | -0.5% | -0.9% | -0.8% | -0.7% | -1.2% | -1.3% |
| Overall (EL) | -0.9% | -1.5% | -1.4% | -1.7% | -2.8% | -2.9% |
| Enc Time[%] | 103.2% |  |  | 102.3% |  |  |
| Dec Time[%] | 101.0% |  |  | 100.4% |  |  |
| Enc Mem[%] | 100.0% |  |  | 100.0% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

|  | RA HEVC SNR |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -1.9% | -4.0% | -4.2% |
| Class B | -1.9% | -3.1% | -3.2% |
| Overall (EL+BL) | -1.9% | -3.3% | -3.5% |
| Overall (EL) | -4.0% | -6.7% | -7.0% |
| Enc Time[%] | 99.8% |  |  |
| Dec Time[%] | 100.2% |  |  |
| Enc Mem[%] | 100.0% |  |  |
| BL Match | Matched |  |  |
|  | LD-P HEVC SNR |  |  |
|  | Y | U | V |
| Class A | -1.3% | -2.2% | -2.1% |
| Class B | -1.2% | -1.7% | -1.8% |
| Overall (EL+BL) | -1.2% | -1.9% | -1.9% |
| Overall (EL) | -2.3% | -3.4% | -3.4% |
| Enc Time[%] | 101.0% |  |  |
| Dec Time[%] | 98.8% |  |  |
| Enc Mem[%] | 100.1% |  |  |
| BL Match | Matched |  |  |

As it is shown in the above, the deblocking filter modification affects almost no performance difference. Since motion compensation is performed on 8x8 block basis, it is recommended to modify to set edge on 8x8 block boundaries for deblocking filtering.

# References

[1] J. Boyce, D. Hong, W. Jang, A. Abbas, S. Reddy (Vidyo), K. McCann, J.-H. Park, J. Kim, C. Kim, J.-H. Min, E. Alshina, A. Alshin, I.-K. Kim, T. Lee, B. Choi, Y. Piao, S. Jeong, S. Lee, Y. Cho, J. Y. Choi (Samsung), “Description of low complexity scalable video coding technology proposal by Vidyo and Samsung”, JCTVC-K0045, Shanghai, China, Oct 2012.

[2] V. Seregin, P. Onno, S. Liu, T. Lee, C. Kim, H. Yang (TE coordinators), “Description of Tool Experiment C5: Inter-layer syntax prediction using HEVC base layer”, JCTVC-K1105, Shanghai, China, Oct 2012.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).