| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0117 |
| --- | --- |

| Title: | AHG7: Residual DPCM for HEVC lossless coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sunil Lee Il-Koo Kim Chanyul Kim | Email: | sunil.lee@samsung.com ilkoo.kim@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In this contribution, a residual DPCM method for HEVC lossless coding is proposed. The proposed method is invoked when lossless coding is enabled for the CU including the corresponding TU, the prediction mode is intra, and the intra prediction mode is either vertical or horizontal mode. The sample-based DPCM is applied to the residual samples in either vertical or horizontal direction, and the differentially coded residual samples are signalled. At the decoder side, the original residual samples are directly reconstructed from the parsed samples without waiting for the reconstruction of the previous residual sample, so that the throughput problem of the sample-based prediction method can be avoided. The proposed method is exactly the same as the intra residual transform-bypass decoding process specified in 8.5.15 of H.264/AVC specification. When the proposed method is implemented in HEVC range extension software, the average coding gain is 8.0%, 1.5%, and 1.0% in All Intra, Random Access, and Low Delay B configurations, respectively, under the common test condition of HEVC range extension. When the proposed method is implemented in HM9.1 and compared to the HM9.1 lossless coding, the average coding gain (w/o Class F) is 5.2% in AI-Main, 6.1% in AI-HE10, 1.3% in RA-Main, 1.8% in RA-HE10, 1.1% in LB-Main, 2.0% in LB-HE10, 1.2% in LP-Main, and 2.1% in LP-HE10, respectively. For class F sequences only, 9.2% in AI-Main, 11.4% in AI-HE10, 5.7% in RA-Main, 7.6 % in RA-HE10, 4.8% in LB-Main, 6.4% in LB-HE10, 4.9% in LP-Main, and 6.4% in LP-HE10, respectively.

# Introduction

Lossless coding has many applications where it is essential to maintain the integrity of an original image or video, e.g. medical image compression. In the current design of HEVC, lossless coding is simply implemented by bypassing transform, quantization, and all the in-loop filters [1]. Even though the current design is simple and straightforward, its coding performance is not satisfactory. For example, in medical image compression which is one of the most promising applications of the lossless coding, HEVC lossless coding is known to be significantly outperformed by the other lossless image coding standards such as JPEG-LS [2]. In this contribution, a residual DPCM method for HEVC lossless coding is proposed. In the proposed method, the DPCM is applied to the residual samples when the lossless coding is enabled for the CU including the corresponding TU, the prediction mode is intra, and the intra prediction mode is either vertical or horizontal. Even though this is a sample-based differential coding method, there is no throughput problem, since the inverse DPCM process can be performed directly from the parsed residual samples. In other words, the current residual sample can be reconstructed without waiting for the reconstruction of the previous (above in vertical mode and left in horizontal mode) residual sample. The proposed method is exactly the same as the intra residual transform-bypass encoding/decoding process of H.264/AVC standard [3]. The intra prediction process and all the syntax elements remain unchanged, but only the residual DPCM is newly introduced to the decoding process. The details of the algorithm description and test results will be provided in Section 2 and Section 3, respectively.

# Algorithm description

The proposed residual DPCM process is invoked when cu_transquant_bypass_flag is equal to 1 (lossless coding is enabled in the CU including the corresponding TU), the prediction mode is intra, and the applicable intra prediction mode is equal to either vertical or horizontal mode. The process for the chroma components is applied in the same way as for the luma component.

Figure 1. The residual DPCM when intra prediction mode is (a) vertical and (b) horizontal mode.

Let the MxN array R with elements ri,j be either an array including the residual samples of luma or chroma component obtained as a result of intra prediction. In the proposed method, the residual DPCM is applied to the residual samples, so that the modified MxN array R with elements ri,j is obtained as follows when the intra prediction mode is vertical mode

ri,j=ri,j,i=0,0≤j≤(N−1)ri,j−r(i−1),j,1≤i≤(M−1),0≤j≤(N−1),

or when the intra prediction mode is horizontal mode

ri,j=ri,j,0≤i≤(M−1),j=0ri,j−ri,(j−1),0≤i≤(M−1),1≤j≤(N−1).

When the proposed method is invoked, the modified residual samples of R are signalled to the decoder, instead of the original residual samples of R.

At the decoder side, the original residual samples can be reconstructed after the modified residual samples are parsed as follows when the intra prediction mode is vertical mode

ri,j=k=0irk,j,0≤i≤(M−1),0≤j≤(N−1),

or when the intra prediction mode is horizontal mode

ri,j=k=0jri,k,0≤i≤(M−1),0≤j≤(N−1).

Note that the current residual sample can be directly reconstructed from the sum of the modified residual samples without reconstructing the previous (left in horizontal mode and above in vertical mode) residual sample. Thus the throughput problem of the conventional sample-based differential coding method can be avoided in the proposed method. As explained in Section 1, the proposed method is exactly the same as the intra residual transform-bypass decoding process specified in Section 8.5.15 of H.264/AVC specification [3].

# Test results

## Test results under common test condition of HEVC range extensions

The proposed method is implemented in HM range extension software, and its performance is evaluated under the common test condition of HEVC range extension [4]. Only two options, TransquantBypassEnableFlag and CUTransquantBypassFlagForce in the configuration file, are set equal to 1 to enable the lossless coding. As summarized in Table 2, the average coding gain of the proposed method is 8.0%, 1.5%, and 1.0% in All Intra, Random Access, and Low Delay B configurations, respectively.

Table 1. Performance of the proposed method under common test condition of HEVC range extension

| Config. Class | All Intra HE | Random Access HE | Low Delay B HE |
| --- | --- | --- | --- |
|  | Bitrate saving | Bitrate saving | Bitrate saving |
| RGB 4:4:4 | -9.9% | -1.8% | -1.2% |
| YCbCr 4:4:4 | -7.5% | -1.5% | -1.0% |
| YCbCr 4:2:2 | -6.2% | -1.2% | -0.7% |
| Overall | -8.0% | -1.5% | -1.0% |
| Enc Time[%] | 98% | 100% | 100% |
| Dec Time[%] | 89% | 97% | 97% |

## Test results under common test condition of HEVC

The proposed method is also implemented in HM9.1 software, and its performance is evaluated under the common test condition of HEVC [5]. Only three options, LosslessCuEnabled, TransquantBypassEnableFlag and CUTransquantBypassFlagValue in the configuration file, are set equal to 1 to enable the lossless coding. As summarized in Table 2, the proposed method provides an average coding gain (w/o Class F) of 5.2% in AI-Main, 6.1% in AI-HE10, 1.3% in RA-Main, 1.8% in RA-HE10, 1.1% in LB-Main, 2.0% in LB-HE10, 1.2% in LP-Main, and 2.1% in LP-HE10, respectively. For class F sequences only, 9.2% in AI-Main, 11.4% in AI-HE10, 5.7% in RA-Main, 7.6 % in RA-HE10, 4.8% in LB-Main, 6.4% in LB-HE10, 4.9% in LP-Main and 6.4% in LP-HE10, respectively.

Table 2. Performance of the proposed method under common test condition of HEVC

|  | All Intra Main |  |  |  | All Intra HE10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  |  | Bit-rate saving | compression ratio |  |  | Bit-rate saving |
|  | Reference | Tested |  |  | Reference | Tested |  |  |
| Class A | 2.1 | 2.3 |  | -6.7% | 1.4 | 1.5 |  | -6.7% |
| Class B | 2.1 | 2.1 |  | -3.2% | 1.3 | 1.4 |  | -4.0% |
| Class C | 2.0 | 2.0 |  | -3.9% | 1.3 | 1.4 |  | -4.6% |
| Class D | 1.8 | 2.0 |  | -5.9% | 1.2 | 1.3 |  | -6.8% |
| Class E | 2.7 | 2.9 |  | -7.6% | 1.7 | 1.9 |  | -10.0% |
| Overall (w/o F) | 2.1 | 2.2 |  | -5.2% | 3.1 | 3.6 |  | -6.1% |
| Overall (w/ F) | 2.5 | 2.7 |  | -5.9% | 1.4 | 1.5 |  | -7.0% |
| Class F | 4.6 | 5.2 |  | -9.2% | 3.1 | 3.6 |  | -11.4% |
| Enc Time[%] | 99% |  |  |  | 99% |  |  |  |
| Dec Time[%] | 96% |  |  |  | 93% |  |  |  |
|  | Random Access Main |  |  |  | Random Access HE10 |  |  |  |
|  | compression ratio |  |  | Bit-rate saving | compression ratio |  |  | Bit-rate saving |
|  | Reference | Tested |  |  | Reference | Tested |  |  |
| Class A | 2.4 | 2.5 |  | -2.4% | 1.5 | 1.5 |  | -2.6% |
| Class B | 2.4 | 2.4 |  | -0.6% | 1.5 | 1.5 |  | -1.4% |
| Class C | 2.5 | 2.5 |  | -1.0% | 1.5 | 1.6 |  | -1.5% |
| Class D | 2.6 | 2.7 |  | -1.3% | 1.6 | 1.6 |  | -1.9% |
| Overall (w/o F) | 2.5 | 2.5 |  | -1.3% | 1.5 | 1.5 |  | -1.8% |
| Overall (w/ F) | 7.5 | 8.0 |  | -2.1% | 4.9 | 5.2 |  | -2.9% |
| Class F | 29.1 | 31.1 |  | -5.7% | 19.3 | 21.0 |  | -7.6% |
| Enc Time[%] | 101% |  |  |  | 102% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 98% |  |  |  |
|  | Low delay B Main |  |  |  | Low delay B HE10 |  |  |  |
|  | compression ratio |  |  | Bit-rate saving | compression ratio |  |  | Bit-rate saving |
|  | Reference | Tested |  |  | Reference | Tested |  |  |
| Class B | 2.4 | 2.4 |  | -0.5% | 1.5 | 1.5 |  | -1.2% |
| Class C | 2.5 | 2.5 |  | -0.8% | 1.5 | 1.6 |  | -1.4% |
| Class D | 2.6 | 2.7 |  | -1.0% | 1.6 | 1.6 |  | -1.6% |
| Class E | 3.2 | 3.3 |  | -2.4% | 1.9 | 2.0 |  | -4.7% |
| Overall (w/o F) | 2.6 | 2.7 |  | -1.1% | 1.6 | 1.6 |  | -2.0% |
| Overall (w/ F) | 11.5 | 12.0 |  | -1.8% | 7.5 | 7.9 |  | -2.9% |
| Class F | 47.1 | 49.4 |  | -4.8% | 31.2 | 33.0 |  | -6.4% |
| Enc Time[%] | 100% |  |  |  | 101% |  |  |  |
| Dec Time[%] | 99% |  |  |  | 97% |  |  |  |
|  | Low delay P Main |  |  |  | Low delay P HE10 |  |  |  |
|  | compression ratio |  |  | Bit-rate saving | compression ratio |  |  | Bit-rate saving |
|  | Reference | Tested |  |  | Reference | Tested |  |  |
| Class B | 2.3 | 2.3 |  | -0.7% | 1.4 | 1.5 |  | -1.4% |
| Class C | 2.4 | 2.5 |  | -0.9% | 1.5 | 1.6 |  | -1.4% |
| Class D | 2.6 | 2.6 |  | -1.1% | 1.6 | 1.6 |  | -1.6% |
| Class E | 3.1 | 3.2 |  | -3.0% | 1.9 | 2.0 |  | -4.7% |
| Overall (w/o F) | 2.6 | 2.6 |  | -1.2% | 1.6 | 1.6 |  | -2.1% |
| Overall (w/ F) | 11.4 | 11.9 |  | -2.0% | 7.5 | 7.9 |  | -2.9% |
| Class F | 46.7 | 48.9 |  | -4.9% | 31.0 | 32.9 |  | -6.4% |
| Enc Time[%] | 101% |  |  |  | 102% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 98% |  |  |  |

# Conclusion

In this contribution, a residual DPCM method for HEVC lossless coding is proposed. Under the common test conditions of both HEVC and HEVC range extension, the proposed method provides the significant coding gain while the intra prediction process and the syntax elements remain unchanged. Furthermore, the proposed method is not a newly introduced coding tool, but exactly same as the intra residual transform-bypass decoding process of H.264/AVC standard. It is recommended to adopt this method into the test model for HEVC range extension for lossless coding.

# Proposed text changes

It is suggested to modify “8.6.2 Scaling and transformation process” of the text as follows.

### Scaling and transformation process

Inputs to this process are:

–	a luma location ( xT, yT ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

–	a variable trafoDepth specifying the hierarchy depth of the current block relative to the coding block,

–	a variable cIdx specifying the colour component of the current block,

–	a variable nT specifying the size of the current transform block.

Output of this process is:

–	the (nT)x(nT) array of residual samples r with elements r[ x ][ y ].

The quantization parameter qP is derived as follows.

–	If cIdx is equal to 0,

qP = QP′Y			(8-1)

–	Otherwise, if cIdx is equal to 1,

qP = QP′Cb		(8-2)

–	Otherwise (cIdx is equal to 2),

qP = QP′Cr		(8-3)

The (nT)x(nT) array of residual samples r is derived as specified as follows:

- If cu_transquant_bypass_flag is equal to 1, the following ordered steps apply:

- The (nT)x(nT) array r is set equal to the (nT)x(nT) array of transform coefficients TransCoeffLevel[ xT ][ yT ][ cIdx ].

- If predModIntra is equal to either 26 (Intra_Vertical) or 10 (Intra_Horizontal), the intra residual transform-bypass decoding process as specified in subclause 8.6.6 is invoked with the size of the transform block nT, the intra prediction mode predModeIntra, and the (nT)x(nT) array r as the inputs, and the output is a modified version of the (nT)x(nT) array r.

- Otherwise, the following ordered steps apply:

- The scaling process for transform coefficients as specified in subclause Error: Reference source not found is invoked with the transform block location ( xT, yT ), the size of the transform block nT, the colour component variable cIdx and the quantization parameter qP as the inputs and the output is a (nT)x(nT) array of scaled transform coefficients d.

- The (nT)x(nT) array of residual samples r is derived as follows.

- If transform_skip_flag[ xT ][ yT ][ cIdx ] is equal to 1, the residual sample array values r[ x ][ y ] with x = 0..nT − 1, y = 0..nT − 1 are derived as:

r[ x ][ y ] = ( d[ x ][ y ] << 7 )		(8-4)

- Otherwise (transform_skip_flag[ xT ][ yT ][ cIdx ] is equal to 0), the transformation process for scaled transform coefficients as specified in subclause Error: Reference source not found is invoked with the transform block location ( xT, yT ), the size of the transform block nT, the colour component variable cIdx, and the (nT)x(nT) array of scaled transform coefficients d as the inputs and the output is a (nT)x(nT) array of residual samples r.

- The variable bdShift is derived as:

bdShift = ( cIdx = = 0 ) ? 20 − BitDepthY : 20 − BitDepthC	(8-5)

- The residual sample values r[ x ][ y ] with x = 0..nT − 1, y = 0..nT − 1 are modified as follows.

r[ x ][ y ] = ( r[ x ][ y ] + (1 << ( bdShift − 1) ) ) >> bdShift	(8-6)

It is also suggested to add the following section into the text.

8.6.6 Intra residual transform-bypass decoding process

This process is invoked when cu_transquant_bypass_flag is equal to 1, pred_mode_flag is equal to 1, and the applicable intra prediction mode is equal to either vertical or horizontal mode. The process for the Cb and Cr components is applied in the same way as for the luma component.

Inputs to this process are:

- a variable nT

- a variable predModeIntra

- an (nT)x(nT) array r with elements r[ x ][ y ] which is either an array relating to a residual transform-bypass block of the luma component or an array relating to a residual transform-bypass block of the Cb and Cr component.

Output of this process is a modified version of the (nT)x(nT) array r with elements r[ x ][ y ] containing the result of the intra residual transform-bypass decoding process.

Let f be a temporary (nT)x(nT) array with element f[ x ][ y ], which are derived by;

f[ x ][ y ] = r[ x ][ y ]   with x=0..nT-1 and y=0..nT-1

Depending on predModeIntra, the following applies:

- If predModeIntra is equal to 26 (Intra_Vertical), the modified array r is derived by:

r[x][y]=k=0yf[x][k]  with x=0..nT-1 and y=0..nT-1

Otherwise (predModeIntra is equal to 10 (Intra_Horizontal)), the modified array r is derived by:

r[x][y]=k=0xf[k][y]  with x=0..nT-1 and y=0..nT-1

# References

- B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm, and T. Wiegand, “High efficiency video coding (HEVC) text specification draft 9 (SoDIS)” Document of Joint Collaborative Team on Video Coding, JCTVC-K1003, Oct. 2012.

- Sunil Lee, Min-Su Cheon, Yumi Sohn, Il-Koo Kim, and Jeonghoon Park, “HEVC lossless coding for medical image compression” Document of Joint Collaborative Team on Video Coding, JCTVC-J0232, July 2012.

- “ITU-T Recommendation H.264 and ISO/IEC 14496-10 AVC,” Joint Video Team of ISO/IEC JCT1/SC29/WG11 & ITU-T SG16/Q.6, Jan. 2012.

- D. Flynn, “Common test conditions and software reference configurations for HEVC range extensions,” Document of Joint Collaborative Team on Video Coding, JCTVC-K1006, Oct. 2012.

- F. Bossen, “Common HM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-K1100, Oct. 2012.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).