| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0118 |
| --- | --- |

| Title: | AHG7: Residual quadtree for HEVC lossless coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tammy Lee Sunil Lee | Email: | tammy.lee@samsung.com sunil.lee@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution presents the proposed residual quadtree configuration and encoder modification for lossless coding and the unnecessary syntax elements are classified for lossless coding. Since the smallest residual quadtree nodes are beneficial for intra prediction and residual quadtree nodes’ size has less meaningful in inter prediction when residual is coded as lossless, it is proposed to use the highest RQT depth tree and the minimum TUs as RQT leaf nodes in intra prediction and the lowest RQT depth tree in inter prediction. Compared to the revision 3055 of the common software (based upon HM-9.0.1) with range extension support setting, the proposed residual quadtree setting provides bitrate saving by -2.9%, -0.4% and -0.2% for AI, RA and LD configurations, respectively.

# Introduction

In the current HM, the lossless coding is enabled by bypassing transform, quantization and in-loop filters (de-blocking filter, SAO and ALF) and can be turned on/off at CU-level by using lossless coding on/off flag at CU level. Since transform and quantization are passed when lossless coding is turned on, residual quadtree works as different meaning other than transform unit.

In intra prediction, reference samples are selected around TU of residual quadtree leaf nodes to make intra prediction signals. Since there is no transform and quantization, it is clear that the smallest TU is always beneficial to make accurate prediction signal by using closet reference samples.

On the other hand, complex residual quadtree is not meaningful in inter prediction because it doesn’t make different coefficients no matter how residual quadtree splits.

However, in both cases, residual quadtree works to make different Cbf map to signal Cbf values which are highly probable to have non-zero coefficients in lossless coding.

In the following tests, the above assumption is verified. And also, it is considered to avoid unnecessary syntax elements and infer those values for the CUs coded as lossless for the following sections.

# Simulation Results

Currently, there is no recommended test configuration for lossless coding. In this proposal, we tested lossless coding by setting the following configuration changes from the recommended test configuration based on the revision 3055 of the common software (based upon HM-9.0.1) with range extension support setting:

TransquantBypassEnableFlag : 1

CUTransquantBypassFlagForce: 1

And as input QP, -24 is given to Traffic RGB 4:4:4 case and -12 for all other cases.

In the common test configuration, the depth values of RQT for both inter and intra predictions are set to 3. To support the maximum RQT depth, SW is changed to fix confirmation of input parameters to follow the defined semantics on the specification.

max_transform_hierarchy_depth_inter specifies the maximum hierarchy depth for transform units of coding units coded in inter prediction mode. The value of max_transform_hierarchy_depth_inter shall be in the range of 0 to Log2CtbSizeY − Log2MinTrafoSize, inclusive.

max_transform_hierarchy_depth_intra specifies the maximum hierarchy depth for transform blocks of coding blocks coded in intra prediction mode. The value of max_transform_hierarchy_depth_intra shall be in the range of 0 to Log2CtbSizeY − Log2MinTrafoSize, inclusive.

Based on the above test conditions, simulations are performed using AI, RA and LDB configuration.

When the depths of RQT of intra prediction and inter prediction are set to 5 and 1 respectively, performance is almost unchanged while encoder complexity of RA & LDB configurations is reduced by 27~30%. The reason why performance in intra configuration is unchanged even though better prediction can be provided with the higher RQT depth of intra prediction is that the current SW applies fast decision and the leaf nodes of the mimimum possible TU size may not be reached due to this. The following results are obtained by forcing to use only leaf nodes of the minimum TU size (i.e. 4x4 in case of RQT depth 5) in intra prediction by code modification in encoder side under the same setting with the above (i.e. the depths of RQT of intra prediction and inter prediction are set to 5 and 1 respectively). Detailed results can be found in the associated excel data sheet.

|  | All Intra HE Lossless |
| --- | --- |
|  | Bitrate saving |
| RGB 4:4:4 | -4.0% |
| YCbCr 4:4:4 | -2.9% |
| YCbCr 4:2:2 | -1.8% |
| Overall | -2.9% |
| Enc Time[%] | 141% |
| Dec Time[%] | 109% |

|  | Random Access HE Lossless |
| --- | --- |
|  | Bitrate saving |
| RGB 4:4:4 | -0.6% |
| YCbCr 4:4:4 | -0.4% |
| YCbCr 4:2:2 | -0.3% |
| Overall | -0.4% |
| Enc Time[%] | 74% |
| Dec Time[%] | 103% |
|  | Low delay B HE Lossless |
|  | Bitrate saving |
| RGB 4:4:4 | -0.2% |
| YCbCr 4:4:4 | -0.2% |
| YCbCr 4:2:2 | -0.2% |
| Overall | -0.2% |
| Enc Time[%] | 77% |
| Dec Time[%] | 103% |

By changing only test configuration and encoder side modification, BD-rate gains can be achieved in intra prediction case and encoder complexity can be greatly reduced in inter prediction case.

Furthermore, we can consider that RQT structure information is useless in lossless coding because it is obvious that the smaller size of RQT leaf nodes is the better for intra prediction and RQT structure has less meaning for inter prediction where RQT structure for inter prediction works as Cbf map whose values are highly probable to have non-zero. Therefore, it is also proposed to skip signaling of split_transform_flag by inferring its value to have the highest RQT depth tree for intra prediction and the lowest RQT depth tree for inter prediction. The following is the proposed specification change:

- Coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enable_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| … |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && !(PartMode = = PART_2Nx2N && merge_flag[x0][y0]) ) |  |
| rqt_root_cbf | ae(v) |
| if( rqt_root_cbf ) { |  |
| MaxTrafoDepth = ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : (cu_transquant_bypass_flag ? 0 : max_transform_hierarchy_depth_inter )) |  |
| transform_tree( x0, y0, x0, y0, log2CbSize, 0, 0 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |

- Transform tree syntax

| transform_tree( x0, y0, xBase, yBase, log2TrafoSize, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| if( log2TrafoSize <= Log2MaxTrafoSize && log2TrafoSize > Log2MinTrafoSize && trafoDepth < MaxTrafoDepth && !(IntraSplitFlag && trafoDepth = = 0) && !cu_transquant_bypass_flag ) |  |
| split_transform_flag[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| if( log2TrafoSize > 2 ) { |  |
| if( trafoDepth = = 0 ¦ ¦ cbf_cb[ xBase ][ yBase ][ trafoDepth − 1 ] ) |  |
| cbf_cb[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| if( trafoDepth = = 0 ¦ ¦ cbf_cr[ xBase ][ yBase ][ trafoDepth − 1 ] ) |  |
| cbf_cr[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| } |  |
| if( split_transform_flag[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| x1 = x0 + ( ( 1 << log2TrafoSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2TrafoSize ) >> 1 ) |  |
| transform_tree( x0, y0, x0, y0, log2TrafoSize − 1, trafoDepth + 1, 0 ) |  |
| transform_tree( x1, y0, x0, y0, log2TrafoSize − 1, trafoDepth + 1, 1 ) |  |
| transform_tree( x0, y1, x0, y0, log2TrafoSize − 1, trafoDepth + 1, 2 ) |  |
| transform_tree( x1, y1, x0, y0, log2TrafoSize − 1, trafoDepth + 1, 3 ) |  |
| } else { |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ¦ ¦ trafoDepth != 0 ¦ ¦ cbf_cb[ x0 ][ y0 ][ trafoDepth ] ¦ ¦ cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) |  |
| cbf_luma[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| transform_unit (x0, y0, xBase, yBase, log2TrafoSize, trafoDepth, blkIdx) |  |
| } |  |
| } |  |

- Transform tree semantics

split_transform_flag[ x0 ][ y0 ][ trafoDepth ] specifies whether a block is split into four blocks with half horizontal and half vertical size for the purpose of transform coding. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered block relative to the top-left luma sample of the picture. The array index trafoDepth specifies the current subdivision level of a coding block into blocks for the purpose of transform coding. trafoDepth is equal to 0 for blocks that correspond to coding blocks.

The variable interSplitFlag is derived as follows.

- If max_transform_hierarchy_depth_inter is equal to 0 and CuPredMode[ x0 ][ y0 ] is equal to MODE_INTER and PartMode is not equal to PART_2Nx2N and trafoDepth is equal to 0, interSplitFlag is set to 1.

- Otherwise, interSplitFlag is set to 0.

When split_transform_flag[ x0 ][ y0 ][ trafoDepth ] is not present, it is inferred as follows:

- If one or more of the following conditions are true, the value of split_transform_flag[ x0 ][ y0 ][ trafoDepth ] is inferred to be equal to 1.

- log2TrafoSize is greater than Log2MaxTrafoSize

- IntraSplitFlag is equal to 1 and trafoDepth is equal to 0

- interSplitFlag is equal to 1

- log2TrafoSize is greater than Log2MinTrafoSize, trafoDepth is less than MaxTrafoDepth and cu_transquant_bypass_flag is equal to 1

- Otherwise, the value of split_transform_flag[ x0 ][ y0 ][ trafoDepth ] is inferred to be equal to 0.

It is also noted that quantization is skipped in lossless coding. Therefore, delta QP values are not needed to be coded. The following proposed specification change skips delta QP value coding for CUs which have the same size with the quantization group and being signaled as transform quantization bypassed.

- Coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enable_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if ( cu_transquant_bypass_flag && !IsCuQpDeltaCoded && log2CbSize == Log2MinCuQpDeltaSize) { |  |
| IsCuQpDeltaCoded = 1 |  |
| CuQpDelta = 0 |  |
| } |  |
| … |  |
| } |  |

# Conclusion

In this proposal, different RQT structure is tested by changing test configuration and encoder modification to enable the maximum RQT depth and the smallest leaf nodes of minimum TU size for intra prediction and the minimum RQT depth for inter prediction in lossless coding. This proposed test configuration and encoder modification help to understand how much gain the current specification can achieve by using the nearest reference samples in intra prediction where residual coding is bypassed. Also, the minimum RQT depth for inter prediction in lossless coding can reduce encoder complexity without significant coding loss.

It is also proposed to bypass unnecessary symbols coding such as RQT split flags and delta QP for lossless coding assuming that the above proposed test configuration and encoder modification are always enabled for transquant bypass CUs. Although coding gain by skipping such symbols is not noticeable due to the fact that the most bit overhead in lossless coding comes from coefficient coding, it can help to increase the CABAC throughput.

# References

[1] D. Flynn, “Common test conditions and software reference configurations for HEVC range extensions”, JCTVC-K1006, Shanghai, China, Oct 2012.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).