| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0121 |
| --- | --- |

| Title: | TE5: Cross-check of TE5.2.2 inter-layer motion prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Cross Check |  |  |
| Author(s) or Contact(s): | Döne Bugdayci, Kemal Ugur | Email: | ext-done.bugdayci@nokia.com kemal.ugur@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

This contribution reports the cross-check results of TE5.2.2 Inter-layer motion prediction. Six configurations are simulated, three of which are included in the TE response as AMVP, Merge and AMVP + Merge modifications. The additional three cases named AMVP Complete, Merge Complete and AMVP + Merge Complete are also tested.

# Introduction

TE 5.2.2 proposes to use the base layer motion vector as a candidate in AMVP and Merge lists. Uncompressed base layer motion vector collocated to the center location of the enhancement layer block is scaled and added to the first location in merge list and after TMVP in AMVP list. The software is examined and found to match the description in TE. Additional results are related to replacing the base layer motion vector with the existing motion vector predictor candidate in the list after the list generation is completed as in the single layer case.

# Simulation Results

## Complexity Results

Accurate timing information for additional results is not available.

Table 1 Timing information for AMVP, Merge and AMVP + Merge cases with respect to the anchor

|  | AMVP |  |  | Merge |  |  | AMVP + Merge |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | RA 2x | RA 1.5x | RA SNR | RA 2x | RA 1.5x | RA SNR | RA 2x | RA 1.5x | RA SNR |
| Enc Time[%] | 98.0% | 97.2% | 98.3% | 94.5% | 92.9% | 96.3% | 94.6% | 91.5% | 94.6% |
| Dec Time[%] | 99.6% | 99.2% | 100.0% | 99.2% | 98.7% | 100.6% | 99.6% | 106.8% | 101.4% |
|  | LD-P 2x | LD-P 1.5x | LD-P SNR | LD-P 2x | LD-P 1.5x | LD-P SNR | LD-P 2x | LD-P 1.5x | LD-P SNR |
| Enc Time[%] | 98.7% | 97.8% | 98.5% | 95.7% | 94.1% | 96.4% | 96.2% | 95.1% | 98.2% |
| Dec Time[%] | 97.0% | 99.7% | 104.0% | 96.6% | 98.9% | 101.3% | 99.8% | 99.6% | 96.5% |

## BD-RD Results

BD-rate results for RA and LP configurations including additional results.

Table 2 RA BD-rate (BL+EL) results for AMVP

|  | 2x |  |  | 1.5x |  |  | SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Traffic | -1.2% | -1.2% | -1.3% | - | - | - | -1.4% | -1.1% | -1.6% |
| PeopleOnStreet | -2.0% | -4.3% | -4.2% | - | - | - | -1.7% | -4.8% | -4.9% |
| Kimono | -1.7% | -2.6% | -3.1% | -1.7% | -2.3% | -2.7% | -1.6% | -2.3% | -2.6% |
| ParkScene | -1.0% | -1.2% | -1.2% | -1.2% | -1.4% | -1.4% | -1.2% | -1.3% | -1.5% |
| Cactus | -1.4% | -1.7% | -1.8% | -1.5% | -1.4% | -1.9% | -1.5% | -1.6% | -2.1% |
| BasketballDrive | -1.9% | -3.1% | -3.4% | -1.8% | -2.4% | -2.9% | -2.0% | -3.3% | -3.3% |
| BQTerrace | -1.0% | -1.4% | -1.2% | -1.2% | -1.4% | -1.4% | -1.4% | -2.5% | -2.7% |
| Class A average | -1.6% | -2.7% | -2.7% | - | - | - | -1.6% | -3.0% | -3.2% |
| Class B average | -1.4% | -2.0% | -2.2% | -1.5% | -1.8% | -2.1% | -1.5% | -2.2% | -2.4% |
| Average | -1.5% | -2.2% | -2.3% | -1.5% | -1.8% | -2.1% | -1.5% | -2.4% | -2.7% |

Table 3 LP BD-rate (BL+EL) results for AMVP

|  | 2x |  |  | 1.5x |  |  | SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Traffic | -1.0% | -1.0% | -1.2% | - | - | - | -1.2% | -1.1% | -1.0% |
| PeopleOnStreet | -1.5% | -3.0% | -2.7% | - | - | - | -1.3% | -3.2% | -3.0% |
| Kimono | -0.9% | -1.4% | -1.5% | -0.9% | -1.3% | -1.4% | -0.9% | -1.1% | -1.2% |
| ParkScene | -0.8% | -1.0% | -1.2% | -0.8% | -0.9% | -1.1% | -0.9% | -0.9% | -0.9% |
| Cactus | -0.7% | -0.7% | -0.8% | -0.6% | -0.9% | -1.1% | -0.8% | -0.9% | -1.0% |
| BasketballDrive | -1.1% | -1.8% | -1.9% | -0.9% | -1.1% | -1.4% | -1.0% | -1.7% | -1.6% |
| BQTerrace | -1.0% | -1.4% | -1.5% | -0.9% | -1.2% | -1.1% | -1.3% | -2.0% | -1.9% |
| Class A average | -1.3% | -2.0% | -1.9% | - | - | - | -1.2% | -2.1% | -2.0% |
| Class B average | -0.9% | -1.3% | -1.4% | -0.8% | -1.1% | -1.2% | -1.0% | -1.3% | -1.3% |
| Average | -1.0% | -1.5% | -1.5% | -0.8% | -1.1% | -1.2% | -1.1% | -1.5% | -1.5% |

Table 4 RA BD-rate (BL+EL) results for Merge

|  | 2x |  |  | 1.5x |  |  | SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Traffic | -2.1% | -2.6% | -2.7% | - | - | - | -2.7% | -2.9% | -3.7% |
| PeopleOnStreet | -3.3% | -8.3% | -7.7% | - | - | - | -2.9% | -10.1% | -9.9% |
| Kimono | -3.1% | -6.1% | -6.8% | -3.1% | -5.7% | -6.1% | -2.4% | -4.5% | -5.0% |
| ParkScene | -1.4% | -1.9% | -1.9% | -1.7% | -2.7% | -2.7% | -1.8% | -2.6% | -2.8% |
| Cactus | -2.3% | -3.2% | -3.4% | -2.7% | -3.3% | -4.0% | -2.5% | -3.4% | -4.1% |
| BasketballDrive | -2.8% | -5.4% | -5.6% | -3.0% | -4.6% | -5.4% | -3.0% | -5.8% | -5.5% |
| BQTerrace | -0.9% | -1.7% | -1.3% | -1.4% | -2.5% | -2.0% | -2.2% | -4.1% | -4.1% |
| Class A average | -2.7% | -5.4% | -5.2% | - | - | - | -2.8% | -6.5% | -6.8% |
| Class B average | -2.1% | -3.7% | -3.8% | -2.4% | -3.8% | -4.1% | -2.4% | -4.1% | -4.3% |
| Average | -2.3% | -4.2% | -4.2% | -2.4% | -3.8% | -4.1% | -2.5% | -4.8% | -5.0% |

Table 5 LP BD-rate (BL+EL) results for Merge

|  |  |  |  | 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Y | U | V |  |  |  |
| Traffic | -1.3% | -1.8% | -1.9% | - | - | - | -2.8% | -2.9% | -3.1% |
| PeopleOnStreet | -1.9% | -4.9% | -4.5% | - | - | - | -2.1% | -5.6% | -5.4% |
| Kimono | -1.6% | -3.0% | -3.1% | -1.5% | -2.5% | -2.7% | -1.5% | -2.0% | -2.3% |
| ParkScene | -0.9% | -1.6% | -1.6% | -1.0% | -1.6% | -2.0% | -1.7% | -1.8% | -2.2% |
| Cactus | -1.2% | -1.6% | -1.8% | -1.4% | -1.7% | -2.2% | -1.7% | -2.0% | -2.3% |
| BasketballDrive | -1.4% | -2.5% | -2.7% | -1.3% | -2.0% | -2.2% | -1.7% | -2.8% | -2.7% |
| BQTerrace | -1.0% | -2.3% | -2.0% | -1.2% | -2.2% | -2.0% | -2.7% | -4.5% | -4.9% |
| Class A average | -1.6% | -3.4% | -3.2% | - | - | - | -2.5% | -4.2% | -4.2% |
| Class B average | -1.2% | -2.2% | -2.2% | -1.3% | -2.0% | -2.2% | -1.9% | -2.6% | -2.9% |
| Average | -1.3% | -2.5% | -2.5% | -1.3% | -2.0% | -2.2% | -2.0% | -3.1% | -3.3% |

Table 6 RA BD-rate (BL+EL) results for AMVP + Merge

|  | 2x |  |  | 1.5x |  |  | SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Traffic | -2.2% | -2.7% | -2.8% | - | - | - | -2.8% | -3.0% | -3.8% |
| PeopleOnStreet | -3.5% | -8.5% | -8.0% | - | - | - | -3.1% | -10.3% | -10.1% |
| Kimono | -3.3% | -6.2% | -6.8% | -3.2% | -5.8% | -6.2% | -2.5% | -4.5% | -5.1% |
| ParkScene | -1.5% | -2.0% | -2.0% | -1.8% | -2.9% | -2.8% | -1.9% | -2.7% | -2.9% |
| Cactus | -2.6% | -3.5% | -3.6% | -2.9% | -3.4% | -4.2% | -2.7% | -3.5% | -4.2% |
| BasketballDrive | -3.0% | -5.6% | -5.9% | -3.1% | -4.8% | -5.6% | -3.1% | -6.0% | -5.8% |
| BQTerrace | -0.9% | -1.9% | -1.4% | -1.5% | -2.5% | -2.1% | -2.3% | -4.3% | -4.4% |
| Class A average | -2.9% | -5.6% | -5.4% | - | - | - | -2.9% | -6.6% | -6.9% |
| Class B average | -2.3% | -3.8% | -4.0% | -2.5% | -3.9% | -4.2% | -2.5% | -4.2% | -4.5% |
| Average | -2.4% | -4.3% | -4.4% | -2.5% | -3.9% | -4.2% | -2.6% | -4.9% | -5.2% |

Table 7 LP BD-rate (BL+EL) results for AMVP + Merge

|  | 2x |  |  | 1.5x |  |  | SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Traffic | -1.5% | -1.9% | -2.1% | - | - | - | -2.9% | -2.9% | -3.2% |
| PeopleOnStreet | -2.1% | -5.1% | -4.7% | - | - | - | -2.2% | -5.8% | -5.6% |
| Kimono | -1.7% | -3.2% | -3.2% | -1.6% | -2.5% | -2.9% | -1.6% | -2.0% | -2.3% |
| ParkScene | -1.0% | -1.7% | -1.6% | -1.1% | -1.8% | -2.2% | -1.8% | -1.9% | -2.1% |
| Cactus | -1.2% | -1.7% | -1.8% | -1.4% | -1.9% | -2.1% | -1.8% | -2.2% | -2.4% |
| BasketballDrive | -1.5% | -2.7% | -2.9% | -1.3% | -2.2% | -2.3% | -1.7% | -2.9% | -2.7% |
| BQTerrace | -1.1% | -2.6% | -2.2% | -1.3% | -2.4% | -2.2% | -2.7% | -4.5% | -4.8% |
| Class A average | -1.8% | -3.5% | -3.4% | - | - | - | -2.6% | -4.3% | -4.4% |
| Class B average | -1.3% | -2.4% | -2.4% | -1.4% | -2.2% | -2.3% | -1.9% | -2.7% | -2.9% |
| Average | -1.4% | -2.7% | -2.6% | -1.4% | -2.2% | -2.3% | -2.1% | -3.2% | -3.3% |

Table 8 RA BD-rate (BL+EL) results for AMVP Complete

|  | 2x |  |  | 1.5x |  |  | SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Traffic | -1.2% | -1.1% | -1.3% | - | - | - | -1.4% | -1.1% | -1.7% |
| PeopleOnStreet | -2.0% | -4.3% | -4.2% | - | - | - | -1.7% | -4.9% | -4.8% |
| Kimono | -1.7% | -2.6% | -3.2% | -1.7% | -2.3% | -2.7% | -1.6% | -2.3% | -2.6% |
| ParkScene | -1.0% | -1.1% | -1.2% | -1.2% | -1.4% | -1.4% | -1.2% | -1.3% | -1.5% |
| Cactus | -1.4% | -1.7% | -1.9% | -1.5% | -1.4% | -1.9% | -1.5% | -1.7% | -2.1% |
| BasketballDrive | -1.9% | -3.1% | -3.4% | -1.8% | -2.5% | -3.0% | -2.0% | -3.4% | -3.3% |
| BQTerrace | -1.0% | -1.4% | -1.1% | -1.2% | -1.5% | -1.3% | -1.3% | -2.5% | -2.7% |
| Class A average | -1.6% | -2.7% | -2.7% | - | - | - | -1.5% | -3.0% | -3.2% |
| Class B average | -1.4% | -2.0% | -2.1% | -1.5% | -1.8% | -2.1% | -1.5% | -2.2% | -2.4% |
| Average | -1.5% | -2.2% | -2.3% | -1.5% | -1.8% | -2.1% | -1.5% | -2.4% | -2.7% |

Table 9 LP BD-rate (BL+EL) results for AMVP Complete

|  | 2x |  |  | 1.5x |  |  | SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Traffic | -1.0% | -1.0% | -0.9% | - | - | - | -1.1% | -1.0% | -1.1% |
| PeopleOnStreet | -1.5% | -2.9% | -2.7% | - | - | - | -1.3% | -3.1% | -3.0% |
| Kimono | -0.9% | -1.3% | -1.6% | -0.9% | -1.2% | -1.4% | -0.9% | -1.1% | -1.3% |
| ParkScene | -0.8% | -1.0% | -1.1% | -0.8% | -0.9% | -1.1% | -0.9% | -0.8% | -0.8% |
| Cactus | -0.7% | -0.7% | -0.7% | -0.6% | -0.8% | -0.9% | -0.8% | -0.8% | -1.0% |
| BasketballDrive | -1.1% | -1.8% | -1.9% | -0.9% | -1.1% | -1.3% | -1.0% | -1.7% | -1.6% |
| BQTerrace | -1.0% | -1.1% | -1.4% | -0.9% | -1.0% | -1.1% | -1.3% | -2.0% | -2.2% |
| Class A average | -1.3% | -2.0% | -1.8% | - | - | - | -1.2% | -2.0% | -2.0% |
| Class B average | -0.9% | -1.2% | -1.3% | -0.8% | -1.0% | -1.2% | -1.0% | -1.3% | -1.4% |
| Average | -1.0% | -1.4% | -1.5% | -0.8% | -1.0% | -1.2% | -1.1% | -1.5% | -1.6% |

Table 10 RA BD-rate (BL+EL) results for Merge Complete

|  | 2x |  |  | 1.5x |  |  | SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Traffic | -1.8% | -2.5% | -2.6% | - | - | - | -2.4% | -2.9% | -3.6% |
| PeopleOnStreet | -3.0% | -7.9% | -7.4% | - | - | - | -2.6% | -9.8% | -9.6% |
| Kimono | -2.9% | -6.0% | -6.6% | -2.9% | -5.7% | -6.1% | -2.2% | -4.3% | -4.9% |
| ParkScene | -1.2% | -1.8% | -1.8% | -1.5% | -2.7% | -2.7% | -1.6% | -2.5% | -2.8% |
| Cactus | -2.0% | -3.0% | -3.2% | -2.5% | -3.2% | -4.0% | -2.2% | -3.3% | -3.9% |
| BasketballDrive | -2.6% | -5.1% | -5.3% | -2.8% | -4.5% | -5.2% | -2.7% | -5.5% | -5.3% |
| BQTerrace | -0.6% | -1.7% | -1.1% | -1.2% | -2.3% | -2.0% | -1.9% | -4.1% | -4.2% |
| Class A average | -2.4% | -5.2% | -5.0% | - | - | - | -2.5% | -6.3% | -6.6% |
| Class B average | -1.8% | -3.5% | -3.6% | -2.2% | -3.7% | -4.0% | -2.1% | -3.9% | -4.2% |
| Average | -2.0% | -4.0% | -4.0% | -2.2% | -3.7% | -4.0% | -2.2% | -4.6% | -4.9% |

Table 11 LP BD-rate (BL+EL) results for Merge Complete

|  | 2x |  |  | 1.5x |  |  | SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Traffic | -0.9% | -1.6% | -1.7% | - | - | - | -2.4% | -2.7% | -2.9% |
| PeopleOnStreet | -1.7% | -4.6% | -4.2% | - | - | - | -1.9% | -5.5% | -5.3% |
| Kimono | -1.4% | -2.8% | -3.0% | -1.3% | -2.5% | -2.7% | -1.3% | -2.0% | -2.2% |
| ParkScene | -0.6% | -1.6% | -1.7% | -0.8% | -1.6% | -1.9% | -1.4% | -1.8% | -2.0% |
| Cactus | -0.8% | -1.3% | -1.4% | -1.1% | -1.6% | -1.8% | -1.4% | -1.9% | -2.1% |
| BasketballDrive | -1.2% | -2.4% | -2.7% | -1.1% | -2.0% | -2.1% | -1.5% | -2.6% | -2.5% |
| BQTerrace | -0.7% | -2.2% | -1.9% | -1.0% | -2.1% | -2.4% | -2.4% | -4.2% | -4.6% |
| Class A average | -1.3% | -3.1% | -3.0% | - | - | - | -2.2% | -4.1% | -4.1% |
| Class B average | -0.9% | -2.1% | -2.1% | -1.1% | -1.9% | -2.2% | -1.6% | -2.5% | -2.7% |
| Average | -1.0% | -2.4% | -2.4% | -1.1% | -1.9% | -2.2% | -1.8% | -3.0% | -3.1% |

Table 12 RA BD-rate (BL+EL) results for AMVP + Merge Complete

|  | 2x |  |  | 1.5x |  |  | SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Traffic | -1.8% | -2.6% | -2.7% | - | - | - | -2.4% | -2.9% | -3.7% |
| PeopleOnStreet | -3.1% | -8.2% | -7.6% | - | - | - | -2.7% | -10.0% | -9.8% |
| Kimono | -3.0% | -6.0% | -6.7% | -3.0% | -5.7% | -6.2% | -2.2% | -4.4% | -5.0% |
| ParkScene | -1.2% | -1.9% | -1.9% | -1.6% | -2.8% | -2.8% | -1.6% | -2.6% | -2.9% |
| Cactus | -2.3% | -3.3% | -3.4% | -2.7% | -3.4% | -4.1% | -2.4% | -3.4% | -4.1% |
| BasketballDrive | -2.8% | -5.5% | -5.7% | -2.9% | -4.7% | -5.5% | -2.9% | -5.8% | -5.6% |
| BQTerrace | -0.7% | -1.8% | -1.2% | -1.2% | -2.4% | -2.0% | -1.9% | -4.2% | -4.3% |
| Class A average | -2.4% | -5.4% | -5.2% | - | - | - | -2.6% | -6.5% | -6.8% |
| Class B average | -2.0% | -3.7% | -3.8% | -2.3% | -3.8% | -4.1% | -2.2% | -4.1% | -4.4% |
| Average | -2.1% | -4.2% | -4.2% | -2.3% | -3.8% | -4.1% | -2.3% | -4.7% | -5.1% |

Table 13 LP BD-rate (BL+EL) results for AMVP + Merge Complete

|  | 2x |  |  | 1.5x |  |  | SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Traffic | -1.1% | -1.8% | -1.9% | - | - | - | -2.5% | -2.8% | -3.0% |
| PeopleOnStreet | -1.9% | -4.8% | -4.5% | - | - | - | -2.0% | -5.6% | -5.4% |
| Kimono | -1.5% | -3.0% | -3.0% | -1.4% | -2.4% | -2.6% | -1.4% | -2.0% | -2.3% |
| ParkScene | -0.7% | -1.6% | -1.7% | -0.9% | -1.7% | -2.0% | -1.4% | -1.8% | -2.0% |
| Cactus | -0.8% | -1.5% | -1.5% | -1.1% | -1.6% | -2.0% | -1.4% | -1.7% | -2.2% |
| BasketballDrive | -1.4% | -2.6% | -2.7% | -1.2% | -2.0% | -2.2% | -1.6% | -2.7% | -2.7% |
| BQTerrace | -0.8% | -2.2% | -2.3% | -1.1% | -2.1% | -2.2% | -2.4% | -4.3% | -4.7% |
| Class A average | -1.5% | -3.3% | -3.2% | - | - | - | -2.3% | -4.2% | -4.2% |
| Class B average | -1.0% | -2.2% | -2.3% | -1.1% | -2.0% | -2.2% | -1.6% | -2.5% | -2.8% |
| Average | -1.2% | -2.5% | -2.5% | -1.1% | -2.0% | -2.2% | -1.8% | -3.0% | -3.2% |

# Patent rights declaration(s)

Nokia Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).