| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0125 |
| --- | --- |

| Title: | TE5: Cross-check of TE5.1.1 inter-layer intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Cross Check |  |  |
| Author(s) or Contact(s): | Döne Bugdayci, Kemal Ugur Visiokatu 3, Tampere, Finland | Email: | ext-done.bugdayci@nokia.com kemal.ugur@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

This contribution reports the cross-check results of TE5.1.1 Inter-layer intra prediction. Two configurations are simulated, one with parsing dependency and one without parsing dependency on base layer by enabling/disabling parsing dependency part of the software provided.

# Introduction

TE5.1.1 proposes to modify the luma and chroma intra prediction mode coding of enhancement layer. The software is examined and following changes are found: DC and Planar modes are removed. For luma coding, MPM list construction is modified to have only one most probable mode and collocated base layer intra prediction mode is taken as the first candidate in the list construction if it’s not equal to DC or Planar modes. Center location is used when getting the base layer intra prediction mode. The remaining 32 modes are coded with 5 bits of Fixed-Length binarization. For chroma coding, allowed chroma modes are reduced to 3 due to the removal of DC and Planar modes. There are also changes related to setting the intra prediction mode of IntraBL blocks to DC mode. As a conclusion the software is found to match the TE response.

Two sets of results are generated:

- Intra prediction mode coding proposal with parsing dependency

- Intra prediction mode coding proposal without parsing dependency

# Simulation Results

Complexity and BD-rate results are provided. More details on the results can be found in the attached documents.

## Complexity Results

Table 1 Timing results (in %) with respect to the anchor

| Parsing Dependency | Encoding Time |  | Decoding Time |  |
| --- | --- | --- | --- | --- |
|  | 2x | 1.5x | 2x | 1.5x |
| Yes | 96.5% | 97.1% | 98.2% | 100.2% |
| No | 95.9% | 97.4% | 98.3% | 100.1% |

## BD-RD Results

Table 2 All Intra configuration BD rate gains (in %) with parsing dependency

|  | 2x |  |  |  |  |  | 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | EL+BL |  |  | EL |  |  | EL+BL |  |  | EL |  |  |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Traffic | -0.2 | 0.0 | 0.1 | -0.4 | 0.0 | 0.2 | - | - | - | - | - | - |
| PeopleOnStreet | -0.3 | -0.2 | -0.2 | -0.6 | -0.3 | -0.3 | - | - | - | - | - | - |
| Kimono | -0.1 | 0.0 | 0.0 | -0.3 | 0.0 | -0.1 | -0.1 | 0.0 | 0.1 | -0.2 | 0.2 | 0.3 |
| ParkScene | -0.1 | -0.1 | -0.2 | -0.1 | -0.2 | -0.3 | 0.0 | 0.0 | 0.0 | -0.1 | 0.1 | 0.0 |
| Cactus | -0.4 | -0.3 | -0.3 | -0.7 | -0.5 | -0.6 | -0.2 | -0.1 | -0.1 | -0.5 | -0.1 | -0.3 |
| BasketballDrive | -0.5 | -0.3 | -0.2 | -1.0 | -0.5 | -0.4 | -0.4 | 0.3 | 0.2 | -0.9 | 0.8 | 0.6 |
| BQTerrace | -0.5 | -0.4 | -0.3 | -0.8 | -0.6 | -0.4 | -0.2 | 0.0 | 0.0 | -0.5 | 0.0 | 0.0 |
| Class A average | -0.3 | -0.1 | 0.0 | -0.5 | -0.1 | 0.0 | - | - | - | - | - | - |
| Class B average | -0.3 | -0.2 | -0.2 | -0.6 | -0.4 | -0.4 | -0.2 | 0.1 | 0.0 | -0.4 | 0.2 | 0.1 |
| Average | -0.3 | -0.2 | -0.2 | -0.6 | -0.3 | -0.3 | -0.2 | 0.1 | 0.0 | -0.4 | 0.2 | 0.1 |

Table 3 All Intra configuration BD rate gains (in %) without parsing dependency

|  | 2x |  |  |  |  |  | 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | EL+BL |  |  | EL |  |  | EL+BL |  |  | EL |  |  |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Traffic | -0.2 | 0.1 | 0.2 | -0.4 | 0.3 | 0.4 | - | - | - | - | - | - |
| PeopleOnStreet | -0.2 | 0.0 | 0.0 | -0.5 | -0.1 | 0.0 | - | - | - | - | - | - |
| Kimono | -0.1 | 0.0 | 0.0 | -0.3 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | 0.0 | 0.1 |
| ParkScene | -0.1 | -0.1 | -0.1 | -0.1 | -0.1 | -0.2 | 0.0 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 |
| Cactus | -0.1 | 0.2 | 0.3 | -0.2 | 0.3 | 0.4 | -0.1 | 0.1 | 0.1 | -0.4 | 0.2 | 0.3 |
| BasketballDrive | -0.2 | -0.1 | 0.0 | -0.5 | -0.3 | 0.0 | -0.3 | 0.2 | 0.2 | -0.8 | 0.6 | 0.6 |
| BQTerrace | -0.4 | 0.0 | 0.3 | -0.6 | 0.0 | 0.4 | -0.2 | 0.0 | 0.1 | -0.5 | 0.1 | 0.2 |
| Class A average | -0.2 | 0.0 | 0.1 | -0.4 | 0.1 | 0.2 | - | - | - | - | - | - |
| Class B average | -0.2 | 0.0 | 0.1 | -0.3 | 0.0 | 0.1 | -0.2 | 0.1 | 0.1 | -0.4 | 0.2 | 0.2 |
| Average | -0.2 | 0.0 | 0.1 | -0.4 | 0.0 | 0.1 | -0.2 | 0.1 | 0.1 | -0.4 | 0.2 | 0.2 |

# Patent rights declaration(s)

Nokia Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).