| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0127 |
| --- | --- |

| Title: | AHG7: LCU-based non-local means filter for chroma components in HEVC range extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Masaaki Matsumura Seishi Takamura Hiroshi Fujii Atsushi Shimizu 1-1, Hikari-no-oka, Yokosuka-shi, Kanagawa 239-0847, JAPAN | Tel: Email: | +81-46-859-3521 matsumura.masaaki@lab.ntt.co.jp +81-46-859-2371 takamura.seishi@lab.ntt.co.jp +81-46-859-8452 fujii.hiroshi@lab.ntt.co.jp +81-46-859-2448 shimizu.atsushi@lab.ntt.co.jp |
| Source: | NTT (Nippon Telegraph and Telephone) Corporation |  |  |

_____________________________

# Abstract

In this contribution, syntax, semantics and decoder description proposed in JCTVC-Lxxxx is described.

# Semantics and syntax

In this section, syntax and semantics for proposed NLM filter are described. The modified syntax and additional syntax are specified in Table 1 to 4. In Table 1 to 3, the difference between syntax for HM range extensions and proposal set highlighted in yellow.

non_local_means_filter_enable_flag equal to 1 specifies the non-local means filter process is applied to the reconstructed picture after the sample adaptive offset process.

| Table 1: Sequence parameter set RBSP syntax |  |
| --- | --- |
| seq_parameter_set_rbsp( ) { | Descriptor |
| ... |  |
| sample_adaptive_offset_enabled_flag | u(1) |
| non_local_means_filter_enable_flag | u(1) |
| } |  |

slice_nlm_filter_enable_flag[cIdx] equal to 1 specifies that NLM filter is enabled for the colour component cIdx in the current slice. slice_nlm_filter_enable_flag[cIdx] equal to 0 specifies that NLM filter is disabled for the colour component cIdx in the current slice.

slice_nlm_single_filter_flag equal to 1 specifies that the filter pattern index of all coding tree blocks is assigned as slice_nlm_filter_pattern_index described below.

slice_nlm_filter_pattern_index[cIdx] specifies the filter pattern index for the colour component cIdx. The value of slice_nlm_filter_pattern_index[cIdx] shall be in the range of 0 to 4, inclusive.

slice_nlm_filter_pattern_index4_flag equal to 1 specifies that slice_nlm_filter_pattern_index[cIdx] is set to 4.

slice_nlm_filter_pattern_add2_flag equal to 1 specifies that the index of 2 is added to slice_nlm_filter_pattern_index[cIdx].

slice_nlm_filter_pattern_add1_flag equal to 1 specifies that the index of 1 is added to slice_nlm_filter_pattern_index[cIdx].

| Table 2: General slice segment header syntax |  |
| --- | --- |
| slice_segment_header( ) { | Descriptor |
| ... |  |
| if( !dependent_slice_segment_flag ) { |  |
| ... |  |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| slice_sao_luma_flag | u(1) |
| slice_sao_chroma_flag | u(1) |
| } |  |
| if(non_local_means_filter_enable_flag){ |  |
| for( i = 0; i < 3; i++ ) |  |
| slice_nlm_enable_flag[ i ] | u(1) |
| if(slice_nlm_filter_enable_flag[ 0 ] ¦¦ slice_nlm_filter_enable_flag[ 1 ] ¦¦ slice_nlm_filter_enable_flag[ 2 ]){ |  |
| slice_nlm_single_filter_flag | u(1) |
| if(slice_nlm_single_filter_flag) |  |
| for( i = 0; i < 3; i++ ) |  |
| if(slice_nlm_filter_enable_flag[ i ]){ |  |
| slice_nlm_filter_pattern_index[i] = 0 |  |
| slice_nlm_filter_pattern_index4_flag | u(1) |
| if(slice_nlm_filter_pattern_index4_flag) |  |
| slice_nlm_filter_pattern_index[i] = 4 |  |
| else{ |  |
| slice_nlm_filter_pattern_add2_flag | u(1) |
| if(slice_nlm_filter_pattern_add2_flag) |  |
| slice_nlm_filter_pattern_index[i] += 2 |  |
| slice_nlm_filter_pattern_add1_flag | u(1) |
| if(slice_nlm_filter_pattern_add1_flag) |  |
| slice_nlm_filter_pattern_index[i] += 1 |  |
| } |  |
| } |  |
| } |  |
| } |  |
| ... |  |
| } |  |
| ... |  |
| } |  |

ctb_nlm_filter_enable_flag[xCtbIdx][yCtbIdx] equal to 1 specifies that NLM filter is enabled for the coding tree block. ctb_nlm_filter_enable_flag[xCtbIdx][yCtbIdx] equal to 0 specifies that NLM filter is disabled for the coding tree block. Here, xCtbIdx and yCtbIdx specify current locations of the coding tree block in coding tree block order.

| Table 3: Coding tree unit syntax |  |
| --- | --- |
| coding_tree_unit( ) { | Descriptor |
| xCtb = ( CtbAddrInRS % PicWidthInCtbsY ) << Log2CtbSizeYx |  |
| yCtb = ( CtbAddrInRS / PicWidthInCtbsY ) << Log2CtbSizeYy |  |
| CtbAddrInSliceSeg = CtbAddrInRS − slice_segment_address |  |
| if( slice_sao_luma_flag ¦ ¦ slice_sao_chroma_flag ) |  |
| sao( xCtb >> Log2CtbSizeYx, yCtb >> Log2CtbSizeYy ) |  |
| if(non_local_means_filter_enable_flag){ |  |
| xCtbIdx = xCtb >> Log2CtbSizeYx |  |
| yCtbIdx = yCtb >> Log2CtbSizeYy |  |
| ctb_nlm_filter_enable_flag[xCtbIdx][yCtbIdx] | u(1) |
| if(ctb_nlm_filter_enable_flag[xCtbIdx][yCtbIdx] && !slice_nlm_single_filter_flag) |  |
| nlm(xCtbIdx, yCtbIdx, CtbAddrInSliceSeg) |  |
| } |  |
| coding_quadtree( xCtb, yCtb, Log2CtbSizeY, 0 ) |  |
| } |  |

ctb_nlm_merge_with_left_flag equal to 1 specifies that the value of ctb_nlm_filter_pattern_index[cIdx][rx][xy] described below is derived from the corresponding value of the left coding tree block (ctb_nlm_filter_pattern_index[cIdx][rx−1][xy]); equal to 0 specifies that the value is not derived from the corresponding value of the left coding tree block.

ctb_nlm_merge_with_up_flag equal to 1 specifies that the value of ctb_nlm_filter_pattern_index[cIdx][rx][xy] described below is derived from the corresponding value of the above coding tree block (ctb_nlm_filter_pattern_index[cIdx][rx][xy−1]); equal to 0 specifies that the value is not derived from the corresponding value of the above coding tree block.

ctb_nlm_filter_pattern_index[cIdx][rx][xy] specifies the filter pattern index for the colour component cIdx. The value of ctb_nlm_filter_pattern_index[cIdx][rx][ry] shall be in the range of 0 to 4, inclusive. Here, rx and ry specify current locations of the coding tree block in coding tree block order.

ctb_nlm_filter_pattern_index4_flag equal to 1 specifies that ctb_nlm_filter_pattern_index[cIdx][rx][xy] is set to 4.

ctb_nlm_filter_pattern_add2_flag equal to 1 specifies that the index of 2 is added to ctb_nlm_filter_pattern_index[cIdx][rx][xy].

ctb_nlm_filter_pattern_add1_flag equal to 1 specifies that the index of 1 is added to ctb_nlm_filter_pattern_index[cIdx][rx][xy].

| Table 4: non-local means filter syntax |  |
| --- | --- |
| nlm (rx, ry, CtbAddrInSliceSeg) { | Descriptor |
| if(slice_nlm_single_filter_flag) |  |
| for( i = 0; i < 3; i++ ) |  |
| ctb_nlm_ filter_pattern_index[i][rx][ry] = slice_nlm_ filter_pattern_index[i] |  |
| else{ |  |
| ctb_nlm_filter_merge_with_left_flag = 0 |  |
| ctb_nlm_filter_merge_with_up_flag = 0 |  |
| if( rx > 0 && ry > 0){ |  |
| leftCtbInSliceSeg = CtbAddrInSliceSeg > 0 |  |
| leftCtbInTile = TileId[CtbAddrInTS] == TileId[CtbAddrRStoTS[CtbAddrInRS − 1]] |  |
| upCtbInSliceSeg = (CtbAddrInRS − PicWidthInCtbsY) >= slice_segment_address |  |
| upCtbInTile = TileId[CtbAddrInTS] == TileId[CtbAddrRStoTS[CtbAddrInRS − PicWidthInCtbsY]] |  |
| AvailableLeft = (leftCtbInSliceSeg && leftCtbInTile) ? 1 : 0 |  |
| AvailableUp = (upCtbInSlice && upCtbInTile) ? 1 : 0 |  |
| if(AvailableLeft && ctb_nlm_filter_enable_flag[rx − 1][ry]) |  |
| ctb_nlm_filter_merge_with_left_flag | u(1) |
| if(AvailableUp && ctb_nlm_filter_enable_flag[rx][ry − 1] && !ctb_nlm_filter_merge_with_left_flag) |  |
| ctb_nlm_filter_merge_with_up_flag | u(1) |
| } |  |
| else if(rx > 0){ |  |
| leftCtbInSliceSeg = CtbAddrInSliceSeg > 0 |  |
| leftCtbInTile = TileId[CtbAddrInTS] == TileId[CtbAddrRStoTS[CtbAddrInRS − 1]] |  |
| AvailableLeft = (leftCtbInSliceSeg && leftCtbInTile) ? 1 : 0 |  |
| if(AvailableLeft && ctb_nlm_filter_enable_flag[rx − 1][ry]) |  |
| ctb_nlm_filter_merge_with_left_flag | u(1) |
| } |  |
| else if(ry > 0){ |  |
| upCtbInSliceSeg = (CtbAddrInRS − PicWidthInCtbsY) >= slice_segment_address |  |
| upCtbInTile = TileId[CtbAddrInTS] == TileId[CtbAddrRStoTS[CtbAddrInRS − PicWidthInCtbsY]] |  |
| AvailableUp = (upCtbInSlice && upCtbInTile) ? 1 : 0 |  |
| if(AvailableUp && ctb_nlm_filter_enable_flag[rx][ry − 1]) |  |
| ctb_nlm_filter_merge_with_up_flag | u(1) |
| } |  |
| if(ctb_nlm_filter_merge_with_left_flag) |  |
| for( i = 0; i < 3; i++ ) |  |
| ctb_nlm_ filter_pattern_index[i][rx][ry] = ctb_nlm_ filter_pattern_index[i][rx − 1][ry] |  |
| if(ctb_nlm_filter_merge_with_up_flag) |  |
| for( i = 0; i < 3; i++ ) |  |
| ctb_nlm_ filter_pattern_index[i][rx][ry] = ctb_nlm_ filter_pattern_index[i][rx][ry − 1] |  |
| if(!ctb_nlm_filter_merge_with_left_flag && !ctb_nlm_filter_merge_with_up_flag) |  |
| for( i = 0; i < 3; i++ ) |  |
| if(slice_nlm_enable_flag[ i ]){ |  |
| ctb_nlm_ filter_pattern_index[i][rx][ry] = 0 |  |
| ctb_nlm_filter_pattern_index4_flag | u(1) |
| if(ctb _nlm_filter_pattern_index4_flag) |  |
| ctb_nlm_ filter_pattern_index[i][rx][ry] = 4 |  |
| else{ |  |
| ctb_nlm_filter_pattern_add2_flag | u(1) |
| if(ctb_nlm_filter_pattern_add2_flag) |  |
| ctb_nlm_ filter_pattern_index[i][rx][ry] += 2 |  |
| ctb_nlm_filter_pattern_add1_flag | u(1) |
| if(ctb_nlm_filter_pattern_add2_flag) |  |
| ctb_nlm_ filter_pattern_index[i][rx][ry] += 1 |  |
| } |  |
| } |  |
| } |  |
| } |  |

# Decoder description

In this section, the decoding process of proposed NLM filter is described. Where, variables used in this section are described below:

- xbitShift/ybitShift denote horizontal/vertical scaling factor between luma and chroma,

- szxCtbL denotes horizontal size of coding tree block for luma sample,

- szyCtbL denotes vertical size of coding tree block for luma sample,

- szxCtbC denoting horizontal size of coding tree block for chroma sample is set to szxCtbL >> xbitShift,

- szyCtbC denoting vertical size of coding tree block for chroma sample is set to szyCtbL >> ybitShift,

- szWidthL denoting horizontal picture size for luma sample is set equal to pic_width_in_luma_samples,

- szHeightL denoting vertical picture size for luma sample is set equal to pic_height_in_luma_samples,

- szWidthC denoting horizontal picture size for chroma sample is set to szWidthL >> xbitShift,

- szHeightC denoting vertical picture size for chroma sample is set to szHeightL >> ybitShift,

- bitDepth denotes internal bit depth,

- maxVal denoting maximum value of the sample is set to (1 << bitDepth) − 1.

Inputs of this process are the reconstructed picture buffers prior to non-local means filter recPictureL, recPictureCb and recPictureCr.

Outputs of this process are the modified reconstructed picture buffers after non-local means filter nlmPictureL, nlmPictureCb and nlmPictureCr.

For the derivation of nlmPictureL, nlmPictureCb and nlmPictureCr, following ordered steps apply:

- All samples of nlmPictureL, nlmPictureCb and nlmPictureCr are set equal to recPictureL, recPictureCb and recPictureCr, respectively.

- The setting process for boundaries as specified in subclause 2.1 is invoked with boundary conditions of LCUs, slice information and tile information as the inputs and nlmBoundaryL, nlmBoundaryCb and nlmBoundaryCr as the outputs.

- For every coding tree blocks with the location (xCtb, yCtb) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture., the following applies:

  - The coding tree block of padded picture buffers recPicturePadded is set equal to recPicture.

  - Generation process for half-shrunk padded reference picture as specified in subcluase The generation process for padded reference picture as specified in subclause 2.2 is invoked with the location (xC, yC) set equal to (xCtb, yCtb), and recPicture as the inputs and recPicturePaddedHalf as the output.

  - When ctb_nlm_filter_enable_flag[xCtb >> Log2CtbSizeYx][yCtb >> Log2CtbSizeYy] is equal to 1, following ordered steps apply:

- If slice_ctb_filter_enable_flag[0] is equal to 1, following steps apply:

- The padding process for the coding tree block as specified in subclause 2.3 is invoked with location (xC, yC) set equal to (xCtb, yCtb), nSx/nSy set equal to szxCtbL/szyCtbL, szWidth/szHeight set equal to szWidthL/szHeightL, recSample set equal to recPicturePaddedL, and recSampleHalf set equal to recPicturePaddedHalfL as the inputs and recSample (recPicturePaddedL) and recSampleHalf (recPicturePaddedHalfL) as the outputs.

- The filtering process as specified in subclause 2.4 is invoked with the location (xC, yC) set equal to (xCtb, yCtb), the end location (xE, yE) set equal to (Min(xC + szxCtbL, szWidthL), Min(xC + szyCtbL, szHeightL)), recSample set equal to recPicturePaddedL, recSampleHalf set equal to recPicturePaddedHalfL, filterPatternIdx set equal to ctb_nlm_filter_pattern_index[0][xCtb >> Log2CtbSizeYx][yCtb >> Log2CtbSizeYy], and boundary condition nlmBoundary set equal to nlmBoundaryL as the inputs and filterd picture buffer nlmSample (nlmPictureL) as the output.

- If slice_ctb_filter_enable_flag[1] is equal to 1, following steps apply:

- The padding process for the coding tree block as specified in subclause 2.3 is invoked with location (xC, yC) set equal to (xCtb >> xbitShift, yCtb >> ybitShift), nSx/nSy set equal to szxCtbC/szyCtbC, sampleWidth/sampleHeight set equal to szWidthC/szHeightC, recSample set equal to recPicturePaddedCb, and recSampleHalf set equal to recPicturePaddedHalfCb as the inputs and recSample (recPicturePaddedCb) and recSampleHalf (recPicturePaddedHalfCb) as the outputs.

- The filtering process as specified in subclause 2.4 is invoked with the location (xC, yC) set equal to (xCtb >> xbitShift, yCtb >> ybitShift), the end location (xE, yE) set equal to (Min(xC + szxCtbC, szWidthC), Min(xC + szyCtbC, szHeightC)), recSample set equal to recPicturePaddedCb, recSampleHalf set equal to recPicturePaddedHalfCb, filterPatternIdx set equal to ctb_nlm_filter_pattern_index[1][xCtb >> Log2CtbSizeYx][yCtb >> Log2CtbSizeYy], and boundary condition nlmBoundary set equal to nlmBoundaryCb as the inputs and filterd picture buffer nlmSample (nlmPictureCb) as the output.

- If slice_ctb_filter_enable_flag[2] is equal to 1, following steps apply:

- The padding process for the coding tree block as specified in subclause 2.3 is invoked with location (xC, yC) set equal to (xCtb >> xbitShift, yCtb >> ybitShift), nSx/nSy set equal to szxCtbC/szyCtbC, sampleWidth/sampleHeight set equal to szWidthC/szHeightC, recSample set equal to recPicturePaddedCr, and recSampleHalf set equal to recPicturePaddedHalfCr as the inputs and recSample (recPicturePaddedCr) and recSampleHalf (recPicturePaddedHalfCr) as the outputs.

- The filtering process as specified in subclause 2.4 is invoked with the location (xC, yC) set equal to (xCtb >> xbitShift, yCtb >> ybitShift), the end location (xE, yE) set equal to (Min(xC + szxCtbC, szWidthC), Min(xC + szyCtbC, szHeightC)), recSample set equal to recPicturePaddedCr, recSampleHalf set equal to recPicturePaddedHalfCr, filterPatternIdx set equal to ctb_nlm_filter_pattern_index[2][xCtb >> Log2CtbSizeYx][yCtb >> Log2CtbSizeYy], and boundary condition nlmBoundary set equal to nlmBoundaryCr as the inputs and filterd picture buffer nlmSample (nlmPictureCr) as the output.

[Ed. (WJ): recPicturePadded and recPicturePaddedHalf are now a picture-size array, but actually CU size + appropriate margin is enough]

## Setting process for boundaries

Inputs to this process are:

- boundary conditions of LCUs,

- slice information,

- tile information.

Output of this process is boundary map consisting of nlmBoundaryL, nlmBoundaryCb, and nlmBoundaryCr.

For the derivation of the boundary map, to each colour component, following ordered steps apply:

- All the samples of nlmBoundary are set to 0.

- Samples of nlmBoundary belonging to bottom 3 to 4-line or right 3 to 4-line from each LCU boundary is set to 1.

- Samples of nlmBoundary belonging to top 1 to 2-line or left 1 to 2-line from each LCU boundary is set to 2.

- Samples of nlmBoundary belonging to bottom 1 to 2-line from each LCU boundary is set to 3.

- Samples of nlmBoundary belonging to right 1 to 2-line from each LCU boundary are set to 4.

- Samples of nlmBoundary belonging to bottom-right 2x2 block of each LCU are set to 7.

- When the LCU adjoins slice/tile boundary at the top, following ordered steps apply:

- Samples of nlmBoundary belonging to top 3 to 4-line or right 3 to 4-line from each slice/tile boundary is set to 1.

- Samples of nlmBoundary belonging to top 1 to 2-line or right 3 to 4-line from each slice/tile boundary is set to 5.

- Samples of nlmBoundary belonging to top-left 2x2 block of each LCU are set to 9.

- Samples of nlmBoundary belonging to top-right 2x2 block of each LCU are set to 8.

- When the LCU adjoins slice/tile boundary at the left, following ordered steps apply:

- Samples of nlmBoundary belonging to top 3 to 4-line or right 3 to 4-line from each slice/tile boundary is set to 1.

- Samples of nlmBoundary belonging to top 1 to 2-line or right 3 to 4-line from each slice/tile boundary is set to 6.

- Samples of nlmBoundary belonging to top-left 2x2 block of each LCU are set to 9.

- Samples of nlmBoundary belonging to bottom-right 2x2 block of each LCU are set to 10.

Figure 1: Boundary setting for nlmBoundary.

## Generation process for half-shrunk padded reference picture

Inputs to this process are:

- a location (xC, yC) specifying the coding tree block location,

- a padded picture buffer recSample.

Output of this process is a half-shrunk padded reference picture buffer recPicturePaddedHalf.

For all colour component cIdx = 0..2, the half-shrunk padded reference picture buffer recPicturePaddedHalf[ x2 ][ y2 ] with x2 = bx2..ex2 and y2 = by2..ey2 are derived by:

xB = xC >> (cIdx == 0 ? 0 : xbitShift),							(2-1)
yB = yC >> (cIdx == 0 ? 0 : ybitShift),							(2-2)
nSx = cIdx == 0 ? szxCtbL : szxCtbC,							(2-3)
nSy = cIdx == 0 ? szyCtbL : szyCtbC,							(2-4)
szWidth  = cIdx == 0 ? szWidthL : szWidthC,						(2-5)
szHeight = cIdx == 0 ? szHeightL : szHeightC,						(2-6)

bx2 = xC >> 1, 											(2-7)
by2 = yC >> 1, 											(2-8)
ex2 = (xC + nSx < szWidth  ? ((xC + nSx) >> 1) : szWidth  >> 1) − 1, 			(2-9)
ey2 = (yC + nSy < szHeight? ((yC + nSy) >> 1) : szHeight >> 1) − 1, 			(2-10)

x = x2 << 1,												(2-11)
y = y2 << 1, 											(2-12)
recPicturePaddedHalf[x2][y2] = recSample[x][y    ] + recSample[x+1][y   ]

recSample[x][y+1] + recSample[x+1][y+1].			(2-13)

## Padding process for the coding tree block

Inputs to this process are:

- a location (xC, yC) specifying the coding tree block location,

- two horizontal/vertical coding tree block sizes nSx/nSy for the colour component,

- two horizontal/vertical variables szWidth/szHeight specifying the width/height for the colour component,

- a padded picture buffer recSample for the colour component,

- a half-shrunk padded picture buffer recSampleHalf for the colour component.

Outputs of this process are:

- a padded picture buffers recSample for the colour component,

- a half-shrunk reference picture buffer recSampleHalf for the colour component.

Padded reference pictures recSample and recSampleHalf are specified in the follows:

xE = (xC + nSx < szWidth ) ? xC + nSx − 1 : szWidth  − 1,				(2-14)
yE = (yC + nSy < szHeight) ? yC + nSy − 1 : szHeight − 1, 				(2-15)

szWidthH  = szWidth  >> 1,			 					(2-16)
szHeight H = szHeight >> 1,			 					(2-17)
xCH = xC >> 1, 									(2-18)
yCH = yC >> 1, 									(2-19)
xEH = xE >> 1, 										(2-20)
yEH = yE >> 1. 										(2-21)

- When xC is equal to 0 or the LCU adjoins slice/tile boundary at the left, for all samples of the vertical location y = yC−4..yE, recSample[xC−1..xC−3][y] are set equal to recSample[xC][y]; and for all samples of the vertical location y = yCH..yEH, recSampleHalf[xCH−1][y] is set equal to recSampleHalf[xCH][y]. These processes are illustrated in Figure 2.

- When xE is equal to szWidth−1 or the LCU adjoins slice/tile boundary at the right, for all samples of the vertical location y = yC−4..yE, recSample[szWidth..szWidth+3][y] are set equal to recSample[szWidth−1][y]; and for all samples of the vertical location yH = yCH..yEH, recSampleHalf[xEH+1][yH] is set equal to recSampleHalf[xEH][yH]. These processes are illustrated in Figure 3.

- When yC is equal to 0 or the LCU adjoins slice/tile boundary at the top, for all samples of the vertical location x = xC−4..xE+4, recSample[x][yC−1..yC−2] are set equal to recSample[x][yC]; and for all samples of the vertical location xH = xCH..xEH, recSampleHalf[xH][yCH−1] is set equal to recSampleHalf[xH][ yCH]. These processes are illustrated in Figure 4.

- When yE is equal to szHeight−1 or the LCU adjoins slice/tile boundary at the bottom, for all samples of the vertical location x = xC−4..xE+4, recSample[x][xE..xE+1] are set equal to recSample[x][xE−1]; and for all samples of the vertical location xH = xCH..xEH, recSampleHalf[xH][yEH] is set equal to recSampleHalf[xH][yEH−1]. These processes are illustrated in Figure 5.

- Generally, for all samples of the vertical location y = yCH..yEH, recSampleHalf[xEH+1][y] are set equal to recSampleHalf[xEH][yH]. Then, for all samples of the vertical location x = xCH..xEH, recSampleHalf[x][yEH+1] are set equal to recSampleHalf[x][yEH]. These processes are illustrated in Figure 6.

Figure 2: The setting of padded area for recSample and recSmapleHalf adjoining picture/slice/tile boundary at the left.

Figure 3: The setting of padded area for recSample and recSmapleHalf adjoining picture/slice/tile boundary at the right.

Figure 4: The setting of padded area for recSample and recSmapleHalf adjoining picture/slice/tile boundary at the top.

Figure 5: The setting of padded area for recSample and recSmapleHalf adjoining picture/slice/tile boundary at the bottom.

Figure 6: The setting of padded area for recSampleHalf corresponding to LCU location.

## Filtering process

Inputs to this process are:

- a location (xC, yC) specifying the top-left coding tree block location,

- an end location (xE, yE) specifying the bottom-right coding tree block location,

- a padded picture buffer recSample for the colour component,

- a padded half-shrunk reference picture buffer recSampleHalf for the colour component,

- a variable filterPatternIdx specifying the filter pattern of the coding tree block,

- a boundary information nlmBoundary for the colour component.

Output of this process is filtered picture buffer nlmSample for the colour component.

The filtered picture buffer nlmSample is derived in the following ordered steps:

- All the samples of the location (x, y) nlmSample[x][y] with x = xL..xE−1 and y = yL..yE−1 are set equal to recSample[x][y].

- The filter block is split into 2x2 sub-blocks; for each 2x2 sub-block with the location (xS, yS) specifying the top-left the sample, the following steps apply:

- The derivation process for the index of activity and gradient as specified in 2.5 is invoked with recSampleHalf set equal to recSampleHalf, boundaryIdx set equal to nlmBoundary[xS][yS] and the location (xS, yS) as the inputs and the activityX and directionIdx as the outputs.

- If activityX is greater than or equal to 4, the derivation process for index of filter strength as specified in 2.6 is invoked with QP specifying quantization parameter, activityX and filterPatternIdx as the inputs and the filter strength filterStrength as the output.

- If activityX is greater than or equal to 64 and nlmBoundary[xS][yS] is equal to 0, for each location of the sample (xP, yP) in 2x2 sub-block, the following steps apply:

- The derivation process for SSD table of cross-shaped template as specified in subclause 2.7.1 and the filter strength set equal to filterStrength as the input and ssdTable as the output.

- Initially, sumWeights is assigned to 255, and sumSamples is assigned to 255 * recSample[xP][yP].

- For each element of tap (the relative location of element of tap (xP2, yP2) and the location (xP, yP) corresponding to directionIdx are illustrated in Figure 7), sumWeights and sumSamples are derived as follows:

tableIdx0 = recSample[xP2−1][yP2−1] − recSample[xP−1][yP−1] + maxVal),	(2-22)

tableIdx1 = recSample[xP2+1][yP2−1] − recSample[xP+1][yP−1] + maxVal), 	(2-23)

tableIdx2 = recSample[xP2   ][yP2  ] − recSample[xP   ][yP  ] + maxVal),	(2-24)

tableIdx3 = recSample[xP2−1][yP2+1] − recSample[xP−1][yP+1] + maxVal), 	(2-25)

tableIdx4 = recSample[xP2+1][yP2+1] − recSample[xP+1][yP+1] + maxVal), 	(2-26)

weight = expTable[Min((ssdTable[tableIdx0] + ssdTable[tableIdx1]

+ ssdTable[tableIdx2] + ssdTable[tableIdx3]

+ ssdTable[tableIdx4]) >> 3, 1023 )], 			(2-27)

sumWeights += weight,								(2-28)
sumSamples += weight * recSample[xP2][yP2]. 				(2-29)

- nlmSample[xP][yP] is derived by nlmSample[xP][yP] = sumSamples / sumWeights.

- Otherwise, if activityX is greater than or equal to 4, for each location of the sample (xP, yP) in 2x2 sub-block, the following steps apply:

- The derivation process for Exp table of pixel template as specified in subclause 2.7.2 and the filter strength set equal to filterStrength as the input and ssdTable as the output.

- Initially, sumWeights is assigned to 255, and sumSamples is assigned to 255 * recSample[xP][yP].

- For each element of tap (the relative location of element of tap (xP2, yP2) and the location (xP, yP) corresponding to directionIdx are illustrated in Figure 7), sumWeights and sumSamples are derived as follows.

tableIdx = recSample[xP2][yP2] − recSample[xS][yS] + maxVal,			(2-30)

weight = expTablePel[expIdx],							(2-31)
sumWeights += weight,								(2-32)
sumSamples += weight * recSample[xP2][yP2]. 				(2-33)

- nlmSample[xP][yP] is derived by nlmSample[xP][yP] = sumSamples / sumWeights.

NOTE –expTable[i] ( i = 0..1023 ) is assigned by

expTable[ i ]= 0;i=1023Floor( 255*exp( −i÷198.0 )+0.5 );otherwise.				(2-34)

Figure 7: Relative locations (xP2, yP2) and the location (xP, yP) corresponding to directionIdx.

## Derivation process for activity and direction

Inputs to this process are:

- a location (xS, yS) specifying the 2x2 sub-block location,

- a boundary condition boundaryIdx,

- a padded half-shrunk reference picture buffer recSampleHalf.

Outputs of this process are:

- a variable activityX,

- a variable direction index directionIdx.

activityX is assigned to:

x2 = xS >> 1, 											(2-35)
y2 = yS >> 1, 											(2-36)
gradX = recSampleHalf[cIdx][x2+1][y2] − recSampleHalf[cIdx][x2−1][y2], 		(2-37)
gradY = recSampleHalf[cIdx][x2][y2+1] − recSampleHalf[cIdx][x2][y2−1], 		(2-38)
activityX = (Abs(gradX) + Abs(gradY)) >> (bitDepth − 8).					(2-39)

For derivation of the directionIdx, the following steps apply:

- If boundaryIdx is greater than 1, directionIdx is assigned to:

directionIdx= 101112131415161718;;;;;;;;;boundaryIdx==2boundaryIdx==3boundaryIdx==4boundaryIdx==5boundaryIdx==6boundaryIdx==7boundaryIdx==8boundaryIdx==9boundaryIdx==10.						(2-40)

- Otherwise, directionIdx is assigned to:

- dir = (gradX << 10) / gradY	,								(2-41)
directionIdx= 56789012345;;;;;;;;;;;gradY==0 or dir<−5120dir<−1707dir<−1024dir<−416dir<−205dir<205dir<416dir<1024dir<1707dir<5120otherwise	.					(2-42)

## Derivation process for index of filter strength

Inputs to this process are:

- a variable QP specifying quantization parameter,

- a variable activityX,

- a variable filterPatternIdx.

Outputs of this process are filterStrengthIdx.

For derivation of the filterStrengthIdx, the following steps apply:

- Two variables Scale and Offset are set to 1.0 and 0.5, respectively.

- If filterPatternIdx is equal to 0, Scale is set to 0.8.

- Otherwise, if filterPatternIdx is equal to 1, Scale is set to 2.0.

- Otherwise, if filterPatternIdx is equal to 2, Offset is set to 4.5.

- Otherwise, if filterPatternIdx is equal to 3, Offset is set to 8.5.

- filterStrengthIdx is assigned to:

val1 = 2.0e0.0381QP+0.0002QP3,								(2-43)
val2 = MinacitvityX, 2552551.011.4651e−0.0502QP,						(2-44)
filterStrengthIdx = Floor(Max(0, Min(Scale*val1*val2+Offset, 31)).			(2-45)

NOTE– All processes described in this subclaus are pre-computed and changed into LUT tables.

## Derivation process for table

All processes described in this subclaus are pre-computed and changed into LUT tables.

### Derivation process for SSD table of cross-shaped template

Input to this process is a variable filterStrength specifying the filter strength of the LCU,

Output of this process is ssdTable specifying the SSD lookup table.

For each element of ssdTable[n] with n = 0..(1 << (bitDepth+1)) − 1 is set to:

div = 5.0 * ((filterStrength + 1) / 32.0)2 * (bitDepth − 8)2,					(2-46)
ssdTable[n] = Round(Min(8.0 *(n − maxVal)2 ÷ div, 65535)).				(2-47)

### Derivation process for Exp table of pixel template

Input to this process is a variable filterStrength specifying the filter strength of the LCU.

Output of this process is expTablePel specifying the SSD lookup table.

For each element of expTablePel[n] with n = 0..(1 << (bitDepth + 1)) − 1 is set to:

div = ((filterStrength + 1) / 32.0)2 * (bitDepth − 8)2,						(2-48)
expTablePel[n] = expTable[Round(Min((n − maxVal)2 ÷ div), 1023))].			(2-49)

# Patent rights declaration(s)

NTT Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).