| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0128 |
| --- | --- |

| Title: | AHG7: Modification of intra angular prediction blending |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shohei Matsuo Seishi Takamura Hiroshi Fujii Atsushi Shimizu Y-517A, 1-1 Hikari-no-oka, Yokosuka-shi, Kanagawa, Japan, 239-0847 | Tel: Email: | +81-46-859-2070 matsuo.shohei@lab.ntt.co.jp +81-46-859-2371 takamura.seishi@lab.ntt.co.jp +81-46-859-8452 fujii.hiroshi@lab.ntt.co.jp +81-46-859-2448 shimizu.atsushi@lab.ntt.co.jp |
| Source: | NTT (Nippon Telegraph and Telephone) Corporation |  |  |

_____________________________

# Abstract

This document introduces an intra angular prediction method using the reference pixels that is closer to the predicted pixels than the original reference pixels. In the proposed method, the predicted signals of the angular prediction mode are generated by blending the predicted signals of the selected angular prediction mode and that of the other mode which has the opposite prediction direction. The original proposal was introduced in JCTVC-K0155. In this contribution, the blending method was modified. The overall average coding gains against the HM9.0 RExt anchor were 0.31%, 0.21%, 0.19% (Y, Cb, Cr in All intra Main-tier), 0.25%, 0.16% and 0.16% (Y, Cb, Cr in All intra High-tier). The maximum coding gains were 0.5%, 0.5% and 0.5% (R, G and B) for the sequence “EBURainFruits” in the RGB 4:4:4 case of the Main-tier. The average run-times of encoding and decoding were 103.8% and 102.8%, respectively.

# Introduction

In the intra prediction of HEVC, there are 35 intra prediction modes: planar (mode 0), DC (mode 1) and 33 angular modes (mode 2 to 34). The definition of the intra modes is the same among all block sizes. Finer prediction direction makes flexible predicted signals, which results in high coding efficiency. When the intra angular mode is selected, the value of weighted reference pixels is copied to the predicted pixels along the prediction direction as described in the equation (8-50) and (8-58) of the draft international standard (SoDIS) [1].

predSamples[ x ][ y ] = ( ( 32 – iFact )*ref[ x+iIdx+1 ] + iFact*ref[ x+iIdx+2] + 16 ) >> 5

predSamples[ x ][ y ] = ( ( 32 – iFact )*ref[ y+iIdx+1 ] + iFact*ref[ y+iIdx+2] + 16 ) >> 5

The array predSamples[x][y] and refMain[x, y] denote the predicted samples and the reference pixels, respectively. The parameter iIdx and iFact denote the index and the multiplication factor determined by the intra prediction mode. In this contribution, to make the predicted signals which reduce the prediction error energy, the proposal exploits other reference pixels located in the closer position to the predicted pixel. The proposal is based on the idea that the prediction error can be reduced when the distance between the predicted pixel and the reference pixel is short.

# Modified intra angular prediction blending

## Basic idea

HEVC employs 33 kinds of intra angular prediction. When mode 2 to 9 and mode 27 to 34 are selected, there are some decoded reference pixels that are located on the prediction direction. As shown in Fig. 1 that shows the case that the block size is 8x8 and the number of the intra mode is 34, the vertical line of the decoded reference pixels as well as the horizontal line can be used for the generation of the predicted signals. In the case of Fig. 1, the predicted pixels located at the bottom left corner are closer to the vertical line of the decoded reference pixels than the original horizontal line. Therefore, the coding efficiency can be improved by using the vertical line as well as the horizontal line to make the predicted signals. In the proposal, all decoded reference pixels on the other side are used for the intra angular prediction.

Figure 1: Conventional intra angular prediction (left) and proposed intra angular prediction (right) (Example of 8x8 and mode 34).

In the previous contribution JCTVC-K0155 [2], the weight coefficients were prepared for all of the angular prediction modes and all of the block sizes. It increased the memory consumption since a lot of matrices of the weight coefficients were needed. In this contribution, instead of preparing the weight coefficients, a weight function is defined. All weight coefficients are generated by using the weight function. Fig. 2 illustrates an example of the weight function f(d) (0<d<1). Along the prediction direction, the defined weight function generates the blending ratio of each predicted pixel. Considering the unification, the same weight function is applied for all of the angular prediction modes and all of the block sizes.

Figure 2: Example of weight function (f(d)=exp(–d2)).

Figure 3 shows an example of the generation of the weight coefficients. The position d=0 defines the position of the reference pixel. The position d=1 defines the position of the reference pixel on the other side. Therefore, the parameter d denotes the distance ratio of the predicted pixels between the original reference pixel A and the other reference pixel B.

For example, in the case of Fig. 3, the four pixels surrounded by the green line are calculated as follows:

predSamples[3][0] = (f(0.2)*predSamples1[3][0]+(1–f(0.2))*predSample2[3][0])

predSamples[2][1] = (f(0.4)*predSamples1[2][1]+(1–f(0.4))*predSample2[2][1])

predSamples[1][2] = (f(0.6)*predSamples1[1][2]+(1–f(0.6))*predSample2[1][2])

predSamples[3][0] = (f(0.8)*predSamples1[0][3]+(1–f(0.8))*predSample2[0][3])

where the array predSamples[x][y] is the blended predicted signals. The array predSamples1[x][y] and predSamples2[x][y] denote the predicted signals generated by the selected intra angular prediction mode and the predicted signals by the intra angular prediction that has the opposite prediction direction, respectively.

Figure 3: Example of generation of the weight coefficients

## Encoding algorithm

The detailed encoding algorithm is summarized as follows. The encoding process is the same as the previous contribution [2].

S1: Set  that indicates the class of the proposed intra angular modes.

S2: Input the value of the intra prediction mode. If the input value is included in , go to S3. Otherwise, the corresponding conventional intra prediction process is performed as defined in Working Draft 9 [1]. After the process is done, go to S4.

S3: Proposed intra angular prediction is done by using the reference pixels on the other side as well as the original reference pixels. After the proposed intra angular prediction is done, go to S4.

S4: Move on to the next prediction block. Repeat the loop (S1 to S4) until all the intra prediction process is completed.

## Parameter syntax related to the proposal

In the current implementation, the proposed encoder and decoder share the class of the intra angular modes () and the weight function f(d). Therefore, there is no additional syntax related to the proposed intra angular prediction.

## Decoder description

When the intraPredMode is included in , the predicted signals are modified by the following equation:

predSamples[ x ][ y ] = (w[ x ][ y ]*predSamples1[ x ][y ] + (1024–w[ x ][ y ])*predSamples2[ x ][ y ] + 512) >> 10

where predSamples[x][y] is the blended predicted signals. The array predSamples1[x][y] and predSamples2[x][y] denote the predicted signals generated by the selected intra angular prediction mode and the predicted signals by the intra angular prediction that has the opposite prediction direction, respectively. The array w[x][y] denotes the weight coefficients. In the following explanation, the original reference pixel is denoted as A[xA][yA] and the other reference pixel is denoted as B[xB][yB]. In the case of f(d)=e–d*d, the array w[x][y] is calculated as follows:

w[ x ][ y ] = exp −Sqrt((xA−x)2+(yA−y)2)Sqrt((xA−xB)2+(yA−yB)2)2

(i) if predModeIntra is equal to or more than 27, the following equations are applied.

xA = (y+1) * intraPredAngle + x * 32

yA = –32

xB = –32

yB = (y+1) * intraPredAngleR + x * 32

where intraPredAngle is defined in Table8-4 of Working Draft 9 [1] and intraPredAngleR is defined in Table 1.

(ii) if predModeIntra is equal to or less than 9, the following equations are applied.

xA = –32

yA = (x+1) * intraPredAngle + y * 32

xB = (x+1) * intraPredAngleR + y * 32

yB = –32

where intraPredAngle is defined in Table8-4 of Working Draft 9 [1] and intraPredAngleR is defined in Table 1.

(iii) otherwise, the blending process is omitted and predSamples1[x][y] is output as predSamples[x][y].

Table 1: Specification of intraPredAngularR

| predModeIntra | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| intraPredAngleR | 32 | 39 | 49 | 60 | 79 | 114 | 205 | 512 |
| predModeIntra | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| intraPredAngleR | 512 | 205 | 114 | 79 | 60 | 49 | 39 | 32 |

# Experimental results

The proposal was incorporated in HM9.0 RExt software. It was applied to all components. The class of the proposed intra angular modes  includes mode 2 to 9 and mode 27 to 34. Weight coefficients are generated using the function of f(d)=e-d*d. Experimental conditions are subject to the common test conditions for HEVC range extensions [3]. From Table 2, the average coding gains were about 0.31%, 0.21% and 0.19% for Y, Cb and Cr of Main-tier. In the case of the High-tier configuration, they were about 0.25%, 0.16% and 0.16% for Y, Cb and Cr. The proposal seemed to be effective for the RGB 4:4:4. The increment of the encoding and decoding run-times were 3.8% and 2.8% for Main-tier, respectively.

Table 2: Experimental results (anchor: HM9.0 RExt).

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | -0.32% | -0.28% | -0.31% | -0.24% | -0.20% | -0.22% |
| YCbCr 4:4:4 | -0.38% | -0.27% | -0.25% | -0.32% | -0.20% | -0.18% |
| YCbCr 4:2:2 | -0.24% | -0.07% | -0.01% | -0.20% | -0.09% | -0.06% |
| Overall | -0.31% | -0.21% | -0.19% | -0.25% | -0.16% | -0.16% |
|  | -0.31% | -0.21% | -0.20% | -0.25% | -0.16% | -0.16% |
| Enc Time[%] | 103.8% |  |  | 103.2% |  |  |
| Dec Time[%] | 102.8% |  |  | 102.4% |  |  |

# Conclusion

In this contribution, modification of intra prediction blending was proposed. The proposal uses the reference pixels on the other side as well as the original reference pixels. Average coding gains against the anchor of HM9.0 RExt software were 0.31%, 0.21%, 0.19% (Y, Cb, Cr in Main-tier), 0.25%, 0.16% and 0.16% (Y, Cb, Cr in High-tier). The maximum coding gains were about 0.5%, 0.5% and 0.5% (R, G and B) for the sequence “EBURainFruits” in RGB 4:4:4 case of the Main-tier. The average run-times of encoding and decoding were 103.8% and 102.8%, respectively. It is recommended to incorporate the proposal into the intra angular prediction of HEVC ver.2 (range extensions).

# References

[1] B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan and T. Wiegand, “High Efficiency Video Coding (HEVC) text specification draft 9,” JCTVC-K1003_v13, 11th meeting, Shanghai, CN, 10-19 October, 2012.

[2] S. Matsuo, S. Takamura, H. Fujii and A. Shimizu, “Intra angular prediction blending,” JCTVC-K0155, 11th meeting, Shanghai, CN, 10-19 October, 2012.

[3] D. Flynn, “Common test conditions and software reference configurations for HEVC range extensions,” JCTVC-K1006, 11th JCT-VC meeting, Shanghai, CN, 10-19 October, 2012.

# Patent rights declaration(s)

NTT Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).