| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0129_r1 |
| --- | --- |

| Title: | AHG7: Modification filter for intra angular prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shohei Matsuo Seishi Takamura Hiroshi Fujii Atsushi Shimizu Y-517A, 1-1 Hikari-no-oka, Yokosuka-shi, Kanagawa, Japan, 239-0847 | Tel: Email: | +81-46-859-2070 matsuo.shohei@lab.ntt.co.jp +81-46-859-2371 takamura.seishi@lab.ntt.co.jp +81-46-859-8452 fujii.hiroshi@lab.ntt.co.jp +81-46-859-2448 shimizu.atsushi@lab.ntt.co.jp |
| Source: | NTT (Nippon Telegraph and Telephone) Corporation |  |  |

_____________________________

# Abstract

This document introduces a modification of intra angular prediction. In the intra DC mode, the modification filter is used to modify the predicted signals adjacent to the reference pixels. In the proposed method, the filter is modified and applied to the angular prediction mode. The overall average coding gains against the HM9.0 RExt anchor were 0.23%, 0.13%, 0.08% (Y, Cb, Cr in All intra Main-tier), 0.17%, 0.09% and 0.04% (Y, Cb, Cr in All intra High-tier). The maximum coding gains were 0.7%, 0.4% and 0.3% (R, G and B) for the sequence “Traffic” in the RGB 4:4:4 case of the Main-tier. The average run-times of encoding and decoding were 100.0% and 101.1%, respectively.

# Introduction

In the intra prediction of HEVC, there are 35 intra prediction modes: planar (mode 0), DC (mode 1) and 33 angular modes (mode 2 to 34). The definition of the intra modes is the same among all block sizes. In the DC mode, after the predicted signals are calculated, the modification filter is used as shown in section 8.4.4.2.5 of Working Draft 9 [1].

predSamples[ 0 ][ 0 ] = ( p[ −1 ][ 0 ] + 2*dcVal + p[ 0 ][ −1 ] + 2 ) >> 2

predSamples[ x ][ 0 ] = ( p[ x ][ −1 ] + 3*dcVal + 2 ) >> 2, with x = 1..nT−1

predSamples[ 0 ][ y ] = ( p[ −1 ][ y ] + 3*dcVal + 2 ) >> 2, with y = 1..nT−1

predSamples[ x ][ y ] = dcVal, with x, y = 1..nT−1

where the array predSamples[x][y], the array p[x][y] and the parameter nT denote the predicted samples, the neighboring reference samples and the transform block size. In this contribution, the concept of the modification filer is extended to the intra angular prediction.

# Modification Filter for intra angular prediction

## Basic idea

In the intra DC mode of the current HEVC decoder specification [1] and the current range extensions, as shown in Fig. 1 (a), the modification filter described in section 1 is applied to the predicted signals adjacent to the reference pixels in the case of luminance signals. However, this kind of filter is not applied to the angular mode as shown in Fig. 1 (b) in the current scheme. When the intra prediction mode 2 to 9 and 27 to 34 are selected, there are some decoded reference pixels that are located on the prediction direction. As shown in Fig. 1 (b) and (c) showing that the block size is 8x8 and the number of the intra mode is 34, the vertical line of the decoded reference pixels as well as the horizontal line can be used for the generation of the predicted signals. Considering that the prediction distance and the directional characteristics of the intra angular modes, as shown in Fig. 1 (c), the proposed modification filter is applied to the predicted signals adjacent to the reference pixels in the case of the intra mode 2 to 9 and 27 to 34.

Figure 1: (a) Modification filter for intra DC mode, (b) conventional intra angular prediction, and (c) proposed intra angular prediction.

## Encoding algorithm

The detailed encoding algorithm is summarized as follows.

S1: Set  that indicates the intra prediction modes 2 to 9 and 27 to 34.

S2: Input the value of the intra prediction mode. If the input value is included in , go to S3. Otherwise, the corresponding conventional intra prediction process is performed as defined in Working Draft 9 [1]. After the process is done, go to S4.

S3: Proposed filter is performed to the predicted signals adjacent to the reference pixels. After that, go to S4.

S4: Move on to the next prediction block. Repeat the loop (S1 to S4) until all the intra prediction process is completed.

## Parameter syntax related to the proposal

In the current implementation, the proposed encoder and decoder share the class of the intra angular modes (). Therefore, there is no additional syntax related to the proposed intra angular prediction.

## Decoder description

The predicted signals of the intra angular prediction modes are modified by the following equations:

(i) if predModeIntra is equal to or more than 27, the following equations are applied.

predSamplesMod[ 0 ][ y ] = ( p[ −1 ][ y + 1 ] + 3*predSamples[ 0 ][ y ] + 2 ) >> 2, with x = 0 and y = 0..nT−1

predSamplesMod[ x ][ y ] = predSamples[ x ][ y ], with x = 1..nT−1 and y = 0..nT−1

where predSamplesMod[x][y], predSamples[x][y], and p[x][y] denote the modified predicted signals, the original predicted signals generated by the selected intra angular prediction mode, and the reference signals.

(ii) if predModeIntra is equal to or less than 9, the following equations are applied.

predSamplesMod[ x ][ 0 ] = ( p[ x + 1 ][ −1 ] + 3*predSamples[ x ][ 0 ] + 2 ) >> 2, with x = 0..nT−1 and y = 0

predSamplesMod[ x ][ y ] = predSamples[ x ][ y ], with x = 0..nT−1 and y = 1..nT−1

where predSamplesMod[x][y], predSamples[x][y], and p[x][y] denote the modified predicted signals, the original predicted signals generated by the selected intra angular prediction mode, and the reference signals.

(iii) otherwise, the modification filter is not applied.

# Experimental results

The proposal was incorporated in HM9.0 RExt software. It was applied to all components. Experimental conditions are subject to the common test conditions of HEVC range extensions [2]. The experimental results of all intra coding are shown in Table 1. The average coding gains were about 0.23%, 0.13% and 0.08% for Y, Cb and Cr of Main-tier. In the case of the High-tier configuration, they were about 0.17%, 0.09% and 0.04% for Y, Cb and Cr. The proposal seemed to be effective for the RGB 4:4:4. The increment of the encoding and decoding run-times were 0.0% and 1.1% for Main-tier, respectively. The results of inter-coding cases are shown in Table 2. It was confirmed that the proposal was effective for the inter-coding for HEVC range extensions.

The proposal was also incorporated in HM9.0.1. The experimental results of all intra coding are shown in Tables 3. The experimental conditions are subject to the common test conditions [3]. The average coding gains were about 0.07%, 0.12% and 0.14% for HEVC Main profile. The increment of the encoding run-time was about 0.6% compared to the HM9.0.1 anchor. The decoding run-time was decreased by about 0.8% compared to the HM9.0.1 anchor.

Table 1: Experimental results of all intra coding (anchor: HM9.0 RExt).

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | -0.30% | -0.22% | -0.18% | -0.19% | -0.13% | -0.09% |
| YCbCr 4:4:4 | -0.17% | -0.10% | -0.01% | -0.13% | -0.06% | 0.01% |
| YCbCr 4:2:2 | -0.23% | -0.06% | -0.03% | -0.19% | -0.06% | -0.04% |
| Overall | -0.23% | -0.13% | -0.08% | -0.17% | -0.09% | -0.04% |
|  | -0.23% | -0.13% | -0.08% | -0.17% | -0.08% | -0.04% |
| Enc Time[%] | 100.0% |  |  | 99.9% |  |  |
| Dec Time[%] | 101.1% |  |  | 101.0% |  |  |

Table 2: Experimental results of inter coding cases (anchor: HM9.0 RExt)

|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | -0.19% | -0.07% | -0.13% | -0.09% | -0.03% | -0.06% |
| YCbCr 4:4:4 | -0.10% | -0.13% | -0.09% | -0.07% | -0.11% | -0.05% |
| YCbCr 4:2:2 | -0.12% | -0.05% | 0.04% | -0.09% | -0.04% | -0.01% |
| Overall | -0.14% | -0.08% | -0.06% | -0.08% | -0.06% | -0.04% |
|  | -0.14% | -0.22% | -0.04% | -0.09% | 0.23% | -0.05% |
| Enc Time[%] | 100.0% |  |  | 100.0% |  |  |
| Dec Time[%] | 101.4% |  |  | 101.0% |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | -0.03% | -0.02% | -0.05% | -0.02% | 0.00% | -0.04% |
| YCbCr 4:4:4 | -0.04% | -0.09% | -0.02% | -0.03% | -0.07% | 0.02% |
| YCbCr 4:2:2 | -0.07% | -0.09% | 0.18% | -0.05% | -0.04% | -0.02% |
| Overall | -0.04% | -0.07% | 0.03% | -0.03% | -0.03% | -0.01% |
|  | -0.05% | 0.12% | 0.04% | -0.03% | -0.07% | -0.02% |
| Enc Time[%] | 100.1% |  |  | 100.1% |  |  |
| Dec Time[%] | 100.6% |  |  | 100.7% |  |  |

nchor: HM9.0 RExttal results of inter coding cases ()eh -coding cases.000000000000000000000000000000000000000000000000000000000

Table 3: Experimental results of all intra coding (anchor: HM9.0.1).

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.16% | -0.24% | -0.22% | -0.18% | -0.29% | -0.27% |
| Class B | -0.04% | -0.11% | -0.13% | -0.05% | -0.10% | -0.11% |
| Class C | -0.04% | -0.08% | -0.11% | -0.04% | -0.09% | -0.09% |
| Class D | -0.04% | -0.04% | -0.10% | -0.05% | -0.12% | -0.04% |
| Class E | -0.07% | -0.11% | -0.13% | -0.09% | -0.16% | -0.08% |
| Overall | -0.07% | -0.12% | -0.14% | -0.08% | -0.15% | -0.12% |
|  | -0.07% | -0.12% | -0.13% | -0.08% | -0.14% | -0.12% |
| Class F | 0.16% | 0.04% | 0.02% | 0.13% | -0.01% | 0.05% |
| Enc Time[%] | 100.6% |  |  | 100.6% |  |  |
| Dec Time[%] | 99.2% |  |  | 99.3% |  |  |

# Conclusion

In this contribution, a modification filter for intra angular prediction was proposed. Average coding gains against the anchor of HM9.0 RExt software were 0.23%, 0.13%, 0.08% (Y, Cb, Cr in Main-tier), 0.17%, 0.09% and 0.04% (Y, Cb, Cr in High-tier). The maximum coding gains were about 0.7%, 0.4% and 0.3% (R, G and B) for the sequence “Traffic” in RGB 4:4:4 case of the Main-tier. The average run-times of encoding and decoding were 100.0% and 101.1%, respectively. Average coding gains against the anchor of HM9.0.1 software were 0.07%, 0.12%, 0.14% (Y, Cb, Cr in Main profile). The average run-times of encoding and decoding were 100.6% and 99.2%, respectively. It is recommended to incorporate the proposal into the intra angular prediction of HEVC ver.2 (range extensions) and ver. 1.

# References

[1] B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan and T. Wiegand, “High Efficiency Video Coding (HEVC) text specification draft 9,” JCTVC-K1003_v13, 11th meeting, Shanghai, CN, 10-19 October, 2012.

[2] D. Flynn, “Common test conditions and software reference configurations for HEVC range extensions,” JCTVC-K1006, 11th JCT-VC meeting, Shanghai, CN, 10-19 October, 2012.

[3] F. Bossen, “Common test conditions and software reference configurations,” JCTVC-K1100, 11th JCT-VC meeting, Shanghai, CN, 10-19 October, 2012.

# Patent rights declaration(s)

NTT Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).