| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0131_r2 |
| --- | --- |

| Title: | Multi-standard extension design |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Yongjin Cho Min Woo Park Jaewon Yoon Junghoon Park | Tel: Email: | b.d.choi@samsung.com |
| Source: | Samsung Electronics Co. Ltd. |  |  |

_____________________________

# Abstract

Signalling the profile and level information for an AVC base layer is proposed. In the current HEVC specification, video parameter set is used for session negotiation and contents selection. However, any profile and level information of the base layer is not signalled in video parameter set, when the base layer is compatible with AVC decoders. In the same design philosophy of HEVC only extension, signaling the profile and level information of non-HEVC base layer in VPS extension is desirable.

# Profile and level information for AVC base layer

Signalling the profile and level information for an AVC base layer is proposed. In the current HEVC specification, video parameter set is used for session negotiation and contents selection. However, any profile and level information of the base layer is not signalled in video parameter set, when the base layer is compatible with AVC decoders. In the same design philosophy of HEVC only extension, signaling the profile and level information of non-HEVC base layer in VPS extension is desirable. Syntaxes signalling profile and level information for AVC base layers can be same as in Rec. ITU-T H.264 | ISO/IEC 14496-10.

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| avc_base_codec_flag | u(1) |
| scalability_mask | u(16) |
| for( i = 0; i<NumScalabilityTypes; i++ ) { |  |
| dimension_id_len_minus1[ i ] | u(3) |
| } |  |
| vps_nuh_layer_id_present_flag | u(1) |
| // layer specific information |  |
| for( i = 1; i<= vps_max_layers_minus1; i++ ) { |  |
| // mapping of layer ID to scalability dimension IDs |  |
| if( vps_nuh_layer_id_present_flag ) |  |
| layer_id_in_nuh[ i ] | u(6) |
| for( j = 0; j <= num_dimensions_minus1; j++ ) |  |
| dimension_id[ i ][ j ] | u(v) |
| } |  |
| if(avc_base_codec_flag) |  |
| avc_layer_profile_level_idc | u(24) |
| for( i = 1; i<= vps_max_layers_minus1 ; i++ ) |  |
| profile_tier_level( 1, vps_max_sub_layers_minus1 ) |  |
| for( i = 1; i<= vps_max_layers_minus1; i++ ) { |  |
| // layer dependency |  |
| num_direct_ref_layers[ i ] | u(6) |
| for( j = 0; j <num_direct_ref_layers[ i ]; j++ ) |  |
| ref_layer_id[ i ][ j ] | u(6) |
| } |  |
| } |  |

avc_layer_profile_level_idc indicates the conformance point of the representation of the base layer conforming to Rec. ITU-T H.264 | ISO/IEC 14496-10. avc_layer_profile_level_idc is the exact copy of the three bytes comprised of profile_idc, constraint_set0_flag, constraint_set1_flag, constraint_set2_flag, constraint_set3_flag, constraint_set4_flag, constraint_set5_flag, reserved_zero_2bits and level_idc in the Rec. ITU-T H.264 | ISO/IEC 14496-10, as if these syntax elements were used to specify the profile and level conformance of the representation of the base layer conforming to Rec. ITU-T H.264 | ISO/IEC 14496-10.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).