| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0133 |
| --- | --- |

| Title: | TE 4: Results of test on 4.5.1Inter layer filtering of boundary between EL and BL |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chul Keun Kim Byeongmoon Jeon #19 Yangjae-daero 11gil, Seocho-gu Seoul 137-130 Korea | Tel: Email: | 82-10-8893-4332 chulkeun.kim@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

In this contribution, Inter layer filtering is proposed for removing blocking artifacts between the prediction block from the base layer (BL) and the neighboring reconstructed region in the enhancement layer (EL). Inter layer filtering on/off decision method is basically the same as that of de-blocking filter for luma in HEVC. Simulation results reportedly show 0.1%, 0.0%, 0.1%, 0.0%, 0.1%, 0.1%, 0.0% 0.1% BD-rate savings on average for AI-2x, AI-1.5x, RA-2x, RA-1.5x, , RA-SNR, LDP-2x, LDP-1.5x and LDP-SNR, respectively, compared with SMuC-0.1.1 anchors.

# Introduction

When a coding unit (CU) in the EL is coded as inter-layer texture prediction, the corresponding texture in the BL is used as texture predictors for the CU in the enhancement layer. Blocking artifacts occur between prediction block from the BL and the neighboring reconstructed region in the EL. In JCTVC-K0033 [1], Boundary smoothing for inter-layer texture prediction is proposed and these techniques were decided to be investigated on top of SMuC-0.1.1 as a part of Tool Experiment 4 (TE4).

# Proposed method

In this contribution, Inter layer filtering is proposed for removing blocking artifacts between the prediction block from the BL and the neighboring reconstructed region in the EL. Horizontal and vertical directions use a common filter for boundary smoothing. As shown in Figure 1, when boundary smoothing is on, the smoothing is as follows.

Inter layer filtering on/off decision method is basically the same as that of de-blocking filter for luma in HEVC, except ß_offset_div2. ß_offset_div2 is set equal to 1. Figure 1 depicts the boundary smoothing on/off decision process.

Figure 1.  Boundary smoothing on/off decision method

# Experimental results

The proposed method was implemented on the SHVC 0.1.1 and the performance was measured under both of intra and inter configuration. Error: Reference source not found summarizes the experimental results.

Table 1. Experimental results

# References

- C. Kim, J. Park, J. Kim, Hendry, S. Park, Y. Jeon, J. Lim, N. Park, B. Jeon, S. Liu, X. Zhang, M. Guo, Z. Chen, T.-D. Chuang, S.-T. Hsiang, C.-Y. Chen, C.-Y. Tsai, C.-M. Fu, C.W. Hsu, Y.-W. Huang, S. Lei, “Description of scalable video coding technology proposal by LG Electronics and MediaTek (differential coding mode on),” Document of Joint Collaborative Team on Video Coding, JCTVC-K0033, October 2012.

# Patent rights declaration(s)

(LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).