| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 |  |  | Document: JCTVC-L0134 |
| --- | --- | --- | --- |
| Title: | Non-TE 2: Constrained intra prediction at enhancement layer |  |  |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chul Keun Kim Jungsun Kim Byeongmoon Jeon #19 Yangjae-daero 11gil, Seocho-gu Seoul 137-130 Korea | Tel: Email: | 82-10-8893-4332 chulkeun.kim@lge.com |
| Source: | LG Electronics |  |  |

# Abstract

This contribution presents constrained intra prediction (CIP) at enhancement layer (EL). In the SMuC-0.1.1, when constrained intra prediction (CIP) is enabled, Intra BL cannot be used in intra prediction. In the proposed technique, when CIP is enabled, if slice type of base layer is Intra, an intra BL CU at the enhancement layer should be used in intra prediction. Simulation results reportedly show 1.5%, 0.4%, 0.4%, 0.1%, 0.0%, 0.1%, 0.0% 0.0% BD-rate savings on average for AI-2x, AI-1.5x, RA-2x, RA-1.5x, , RA-SNR, LDP-2x, LDP-1.5x and LDP-SNR, respectively, compared with SMuC-0.1.1 anchors(CIP On).

# Introduction

Currently, when constrained intra prediction (CIP) is enabled, only intra coded neighbouring PUs can be used in intra prediction. That is, neighbouring PUs coded by an intra BL mode cannot be used in intra prediction. Hence, coding efficiency decreases when CIP is enabled.

# Proposed method

This proposal suggests that when CIP is enabled, if slice type of base layer is Intra, an intra BL CU at the enhancement layer should be used in intra prediction.

Figure 1. Proposed CIP process

# Experimental results

The proposed method was implemented on the SHVC 0.1.1 and the performance was measured under both of intra and inter configuration. Table 1 shows the experimental results of proposed method. Simulation results reportedly show 1.5%, 0.4%, 0.4%, 0.1%, 0.0%, 0.1%, 0.0% 0.0% BD-rate savings on average for AI-2x, AI-1.5x, RA-2x, RA-1.5x, , RA-SNR, LDP-2x, LDP-1.5x and LDP-SNR, respectively, compared with SMuC-0.1.1 anchors(CIP On).   Detailed results are described in the accompanied excel file.

Table 1.  Results of the proposed method (anchor: SMuC-0.1.1, CIP On)

# Patent rights declaration(s)

(LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).