| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0135 |
| --- | --- |

| Title: | TE3-4.2.3: Intra prediction based on difference picture |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Joonyoung Park Byeongmoon Jeon #19 Yangjae-daero 11gil, Seocho-gu Seoul 137-130 Korea | Email: | jy.park@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution presents intra coding in difference domain (D-domain) at enhancement layer (EL). In the proposed technique, D-domain is basically obtained by subtracting base layer (BL) samples from EL samples. Intra prediction is performed in D-domain. After obtaining reconstruction samples in D-domain, they are added to collocated samples at BL for the final reconstruction samples. For each CU in intra prediction mode, a flag is signalled to indicate whether the proposed technique is employed or not. The simulation results reportedly show that it achieves 0.9% and 0.6% BD rate savings on average for AI-2x and AI-1.5x, respectively, compared with SMuC-0.1.1 anchors. Encoding times are 163.4% and 154.7%, and decoding times are 109.7% and 108.9%.

# Introduction

In [1], video coding based on difference picture has been introduced. In this contribution, intra coding in D-domain at EL is presented.

# Proposed technique

Figure 1. Reconstruction process of proposed technique

Figure 1 shows the reconstruction process of a block to which the proposed technique is applied. In order to perform intra prediction in D-domain, we need reference samples in D-domain. They can be obtained by subtracting EL reconstruction samples from up-sampled BL reconstruction samples. After intra prediction with reference samples in D-domain, we get reconstruction block in D-domain by adding residuals. And then, we obtain the final reconstruction block by adding collocated block from BL.

If the block which the reference samples belong to is also coded with the proposed technique, reference samples are not calculated. Instead, reconstruction samples in D-domain are directly employed.

In the proposed method, reference, prediction and reconstruction values in D-domain are clipped from [-255,255] into [-128, 127]. By doing it, we can keep the range size the same with the original domain.

# Results

The proposed method has been implemented on SMuC 0.1.1 and simulated under the common test conditions. Table 1 summarizes the experimental results. It reportedly shows 0.9%, 0.6% BD-rate savings on average for AI-2x and AI-1.5x, respectively. The encoding times are 163.4% and 154.7%, and decoding times are 109.7% and 108.9%.

Table 1. Experimental results on AI configuration

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.6% | -0.4% | -0.6% |  |  |  |
| Class B | -1.1% | -0.4% | -0.6% | -0.6% | 0.1% | 0.0% |
| Overall (EL+BL) | -0.9% | -0.4% | -0.6% | -0.6% | 0.1% | 0.0% |
| Overall (EL) | -1.6% | -0.7% | -1.0% | -1.8% | 0.5% | 0.0% |
| Enc Time[%] | 163.4% |  |  | 154.0% |  |  |
| Dec Time[%] | 109.7% |  |  | 108.9% |  |  |

# References

[1] C. Kim, J. Park, J. Kim, Hendry, S. Park, Y. Jeon, J. Lim, N. Park, B. Jeon, S. Liu, X. Zhang, M. Guo, Z. Chen, T. Chuang, S. Hsiang, C. Chen, C. Tsai, C. Fu, C. Hsu, Y. Huang, S. Lei, “Description of scalable video coding technology proposal by LG Electronics and MediaTek (differential coding mode on)”, JCTVC-K0033, Oct. 2012

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).